| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0154 |
| --- | --- |

| Title: | AHG4: Syntax to disable tile markers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Chia-Yang Tsai , Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chiayang.tsai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HEVC CD, two methods are provided to locate tile start points in the bitstream. One is tile entry point offset in the slice header. The other is tile entry point maker within the slice data. Tile entry point offset in slice header can be easily disabled by setting num_entry_point_offsets to zero, while tile entry point markers are always sent as long as the number of tiles is greater than 1. In this contribution, we propose a syntax design that can disable tile markers if not necessary.

# Introduction

In HM-6.0, independent tiles are supported, which means encoding and decoding LCUs (e.g. MV prediction, intra prediction, entropy coding) for one tile do not need data from any other tile. Independent tiles can be used for parallel processing. However, for decoders to parse multiple tiles in parallel, mechanisms to locate each tile in the bitstream are required. In current CD syntax, two methods are provided. One is to put the number of bytes for each tile called as tile entry point offset in the slice header. The other is to use specific syntax patterns within slice data called as tile entry point makers to distinguish one tile from the other. Tile entry point offsets in slice header can be easily disabled by setting num_entry_point_offsets to zero, while tile entry point markers are always sent as long as the number of tiles is greater than 1. In this contribution, we propose a syntax design that can disable tiles marker if not necessary.

# Proposed syntax design

We propose to add one flag in both SPS and PPS to disable the tile entry point makers. In that way, the marker bits can be saved if disabled. The proposed syntax is shown in Table 1, Table 2 and Table 3.

Table 1.  Proposed flag in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| disable_tile_marker_flag | u(1) |
| } |  |
| … |  |
| } |  |

disable_tile_marker_flag equal to 1 specifies that there is no tile marker sent within the slice data; equal to 0 specifies that there could be tile marker sent within the slice data depending on the number of tiles. The default value of disable_tile_marker_flag is set to 0 if not present.

Table 2.  Proposed flag in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) { |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| disable_tile_marker_flag | u(1) |
| } |  |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |
| … |  |
| } |  |

disable_tile_marker_flag equal to 1 specifies that there is no tile marker sent within the slice data; equal to 0 specifies that there could be tile marker sent within the slice data depending on the number of tiles. The default value of disable_tiles_marker_flag is set to 0 if not present.

Table 3.  Proposed syntax change in slice data

| slice_data( ) { | Descriptor |
| --- | --- |
| … |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| … |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( disable_tile_marker_flag != 1 ) { |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

# Conclusions

In this contribution, one disable flag in both SPS and PPS is proposed to disable the tile entry point makers. In this way, the marker bits can be saved if disabled.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).