| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0155 |
| --- | --- |

| Title: | AHG5: Unified context selection for significance map coding of 4x4 and 8x8 TUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-6.0, for significance map coding of 4x4 and 8x8 TUs, three context selection look-up tables are used and in total 37 contexts are required. In this contribution, two methods are proposed for unified context assignment and context reductions. One method is to unify the context selection look-up tables between 4x4 and 8x8 TUs, which results in only one look-up table and reduces of one context. The other method is based on the first method and further shares the contexts being used between 4x4 and 8x8 TUs, which results in only one lookup table and reduces 19 contexts. Simulation results reportedly show 0% and 0.1% BD-rate increases for the first method and the second method, respectively, with unchanged run times in comparison with HM-6.0.

# Introduction

In current HM-6.0, different context selection methods of significance map coding are used for different TU sizes. For the TU size equal to 4x4 or 8x8, the context selection is based on coefficient positions with different look-up tables. Figure 1 shows the position-based context selection look-up table for 4x4 TU. Figure 2 shows the position-based context selection look-up table for 8x8 TU. A number denotes a context used for a position of a TU. There are 37 contexts in total: nine for 4x4 luma, six for 4x4 chroma, 11 for 8x8 luma, and 11 for 8x8 chroma. In this contribution, we propose two methods to unify the context selection for the 4x4 and 8x8 TUs. One is to unify the position-based context selection look-up table between 4x4 and 8x8 TUs, which results in saving of one context. The other one is to not only unify the look-up table but also share contexts for 4x4 and 8x8 TUs, which reduces 19 context. The proposed methods are studied on top of HM-6.0. The rest of this document is organized as follows. Details of the proposed methods will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

- 4x4 luma context assignment   (b) 4x4 chroma context assignment

Figure 1.  Position-based context selection look-up table for 4x4 TU

Figure 2.  Position-based context selection look-up table for 8x8 luma and chroma TUs

# Proposed methods

When the position-based context selection method is used for 4x4 and 8x8 TUs, look-up tables will be needed to store the context assignment for each coefficient position. In HM-6.0, there are three look-up tables, i.e. one for 4x4 luma component, one for 4x4 chroma components, and one for both 8x8 luma and chroma components, as shown in Figure 1 and Figure 2. From hardware implementation point of view, the more the look-up tables, the higher the hardware cost. It is preferable to keep the number of look-up tables as small as possible. Consequently, we proposed two methods as follows.

## Method 1: Unification of significance map context selection for 4x4 and 8x8 TUs

In method 1, we use the same context selection look-up table for both 4x4 luma and chroma TUs. Moreover, we also reuse it for both 8x8 luma and chroma TUs. For one 8x8 TU, each position performs 2:1 down-scaling in both horizontal and vertical directions to get the down-scaled position, and then the down-scaled position is used as the input of the 4x4 luma context selection look-up table to get the context. In this method, only one 4x4 look-up table is needed. The unified context selection look-up table for 4x4 and 8x8 TUs is shown in Figure 3. Therefore, this scheme results in nine contexts for 4x4 luma TU, nine contexts for 4x4 chroma TU, nine contexts for 8x8 luma TU, and nine contexts for 8x8 luma TU. There are 36 contexts in total. In comparison with HM-6.0, one context is reduced.

(a) 4x4 luma and chroma context assignment

(b) 8x8 luma and chroma context assignment

Figure 3.  Method 1: unified context selection look-up table

## Method 2: Sharing of significance map contexts for 4x4 and 8x8 TUs

In method 2, as in method 1, we use only one look-up table for both 4x4 and 8x8 TUs and for both luma and chroma components. Moreover, in order to reduce the required context number, we use the same set of contexts for 4x4 and 8x8 TUs. Therefore, this scheme results in nine contexts for 4x4 and 8x8 luma TUs and another nine contexts for 4x4 and 8x8 chroma TUs. There are 18 contexts in total. In comparison with HM-6.0, 19 contexts are reduced.

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-H1100 [1], and the software is based on HM-6.0. Simulation results of method 1 and method 2 are shown in Table 1 and Table 2, respectively. Method 1 does not cause any noticeable change in terms of BD-rates and run times. Method 2 shows 0.1% bit rate increase with unchanged run times.

Table 1.  Results of the proposed method 1

Table 2.  Results of the proposed method 2

# Conclusions

In this contribution, two methods were proposed for unified context assignment of significance map coding for 4x4 and 8x8 TUs. Both methods can reduce the context look-up tables from 3 to 1 with one and 19 context reductions, respectively. Simulation results reportedly show 0% and 0.1% bit rate increases for these two methods, respectively, with unchanged run times in comparison with HM-6.0.

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).