| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0156 |
| --- | --- |

| Title: | Simplification of Multiple Sign Bit Hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Yu Jing Wang Dake He Gaelle Martin-Cocher | Email: | xiayu@rim.com wangj@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the Multiple Sign Bit Hiding (SBH) by removing the transmission of the SBH controlling threshold and using a fixed value for it. This simplification is proposed to facilitate decoder conformance testing. Studies on varying the threshold also showed that the rate distortion coding benefit of using different thresholds is very marginal and the defaulted value is the best choice on average. HM6.0 test confirmed no coding performance impact by this change.

# Proposed method

It is proposed to remove the threshold transmission for Multiple Sign Bit Hiding [1] in HM6.0. A fixed value of 4, which has been tested against other choices and has shown the best average coding performance, is proposed to be used for this threshold.

# Experimental Results

HM6.0 common test conditions.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  | 104% |  |  |
| Dec Time[%] | 94% |  |  | 95% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 90% |  |  | 90% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 111% |  |  |
| Dec Time[%] | 86% |  |  | 86% |  |  |

# Note that the enc/dec time is not reliable as the anchor data is from BBC while the tested data is generated on a cluster.

Other tests on varying the threshold TSIG were also carried out. Note that anchor uses TSIG=4.

TSIG=2

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% |
| Class B | 0.0% | 0.6% | 0.6% | 0.1% | 0.4% | 0.4% |
| Class C | 0.0% | 0.6% | 0.7% | 0.1% | 0.4% | 0.5% |
| Class D | 0.0% | 0.8% | 0.8% | 0.1% | 0.5% | 0.6% |
| Class E | 0.2% | 0.5% | 0.2% | 0.2% | 0.1% | 0.1% |
| Overall | 0.1% | 0.6% | 0.6% | 0.1% | 0.4% | 0.4% |
|  | 0.1% | 0.6% | 0.6% | 0.1% | 0.4% | 0.4% |
| Class F | 0.1% | 0.6% | 0.5% | 0.1% | 0.3% | 0.2% |
| Enc Time[%] | 136% |  |  | 140% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

TSIG=3

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.5% | 0.0% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 129% |  |  | 133% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

TSIG=5

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 126% |  |  | 127% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

TSIG=6

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.1% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.2% | 0.3% | 0.0% | 0.2% | 0.1% |
| Overall | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class F | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 127% |  |  | 127% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Syntax for working draft

- This section describes the change request of the current proposal with regard to the Committee Draft of HEVC.

#### 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| enable_temporal_mvp_flag | u(1) |

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| …… |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >=4) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| …… |  |

#### 7.4.2.2Picture parameter set RBSP semantics

pic_parameter_set_id identifies the picture parameter set that is referred to in the slice header. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

seq_parameter_set_id refers to the active sequence parameter set. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

sign_data_hiding_flag equal to 0 specifies that sign bit hiding is disabled. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled

cabac_init_present_flag equal to 1 specifies that cabac_init_flag is present in the slice header.

# Conclusion

It is proposed to adopt this proposal in HM7/DIS.

# References

[1] B. Bross, W-J Han, J-R Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 8th Meeting, [2] X. Yu J. Wang, D. He, G. Martin-Cocher, and S. Campbell, “Multiple Sign Bits Hiding”, JCTVC-H0481.

[3] Yong, etc,   Cross-check

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).