| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0158 m24397 |
| --- | --- |

| Title: | AHG 4: Picture Raster Scan Decoding in the presence of multiple tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs) Félix Henry (Orange Labs) Stéphane Pateux (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com stephane.pateux@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

Picture raster scan single core decoding of frames encoded with multiple tiles is desirable in order to avoid the buffering of most of the picture before a single line of LCUs can be output. In the current design of HEVC, picture raster scan decoding requires bitstream jumping and CABAC state memorization/restore. The current contribution proposes to flush CABAC at the end of each LCU line inside a tile, so that CABAC state operations can be avoided and buffers can be eliminated. The impact on rate-distortion performance is +0.1% (Intra), +0.6% (Random Access), +1.5% (Low Delay) compared to current design when a large number of tiles is used (JCTVC-F335 tile configuration).

# Problem Statement

When decoding frames with multiple tiles with a single core, normative decoding imposes that tile-raster scan order is followed. However, such a scan order imposes that most of the decoded frame is buffered before the first line of LCU can be output (Figure 1). This problem has been indentified in [H0463].

Figure 1: When decoding LCUs in tile scan order, the first line of LCUs is available only when tiles 1,2,3 are decoded.

In order to avoid buffering most of the frame, it is possible to perform picture raster scan order decoding of the bitstream. In this case, LCUs are decoded in the usual picture raster scan order and several operations must be performed when transitioning from one tile to the other (Figure 2).

- A bitstream pointer must be stored in order to be able to continue parsing the tile substream when the next line of LCUs will be decoded

- CABAC state must be stored in order to be able to continue parsing the tile substream when the next line of LCUs will be decoded

- CABAC state must be restored from a previously stored one

- A bitstream jump must be performed to access a location that corresponds either to a tile entry points described by an offset or a marker in the bitstream, or by a previously stored bitstream pointer

In some cases not all operations are necessary (last row in a tile, for instance):

In terms of memory, this process requires one (bit level) bitstream pointer per tile column and one CABAC state buffer per tile column.

Figure 2: Decoding a frame dividing in multiple tile in picture raster scan order (top) using bitstream jumping (bottom).

# Proposal

The current contribution proposes that a CABAC flush is performed at the end of each line of LCUs in a tile (except for the last LCU in a tile, where flushing and byte alignment is already performed in the current specification).

This modification allows the removal of steps 2 and 3 from the previous section. Also, additional CABAC state buffers for the worst case (largest number of tile columns) are no longer necessary.

# Performance

The current proposal has been tested using common test conditions [2] with tiles activated, where tile sizes are set according to JCTVC-F335 [3] (tile size are as close as possible to class D images). The anchor uses HM6.1 with tiles activated, and the proposal uses the same configuration, with the additional CABAC flush at the end of each line of LCU in a tile, except for the last LCU in a tile. The impact of the proposed modifications is shown on Table 1.

Table 1: Rate distortion performance of the proposed modification using tile division as per JCTVC-F335

# Proposed text modifications

9.2 Cabac parsing process for slice data

This process is invoked when parsing syntax elements with descriptor ae(v) in subclauses Error: Reference source not found and Error: Reference source not found.

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

When starting the parsing of the slice data of a slice in subclause Error: Reference source not found, the initialization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found. When tiles_or_entropy_coding_sync_idc is equal to 2 and num_substreams_minus1 is greater than 0, a mapping table BitStreamTable with num_substreams_minus1 + 1 entries specifying a bitstream pointer table to use for later current bitstream pointer derivation is derived as follows.

–	BitStreamTable[ 0 ] is initialized to contain the bitstream pointer.

–	For all indices i greater than 0 and less than num_substreams_minus1+1, BitStreamTable[ i ] contains a bitstream pointer to entry_point_offset[ i ] bytes after BitStreamTable[ i − 1 ].

The current bitstream pointer is set to BitStreamTable[ 0 ].

Figure 9-1 – Spatial neighbour T that is used to invoke the treeblock availability derivation process relative to the current largest coding unit (informative)

The mininum coding block address of the coding treeblock containing the spatial neighbor block T (Figure 9-1), ctbMinCbAddrT, is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding treeblock as follows.

x = x0 + 2 << Log2CtbSize − 1
y = y0 − 1
ctbMinCbAddrT = MinCbAddrZS[ x >> Log2MinCbSize ][ y >> Log2MinCbSize ]	(9-1)

The variable availableFlagT is obtained by invoking the coding block availability derivation process as specified in subclause Error: Reference source not found with ctbMinCbAddrT as input.

When starting the parsing of a coding tree as specified in subclause Error: Reference source not found and tiles_or_entropy_coding_sync_idc is equal to 2 and num_substreams_minus1 is greater than 0 , the following applies.

–	If CtbAddrRS % PicWidthInCtbs is equal to 0, the following applies.

–	When availableFlagT is equal to 1, the synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

–	The decoding process for binary decisions before termination specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine specified in subclause Error: Reference source not found.

–	The current bitstream pointer is set to indicate BitStreamTable[ i ] with the index i derived as follows.

i = ( CtbAddrRS / PicWidthInCtbs ) % ( num_substreams_minus1 + 1 )	(9-2)

–	Otherwise, if CtbAddrRS % PicWidthInCtbs is equal to 2, the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

When starting the parsing of a coding tree as specified in subclause 7.3.4.1 and tiles_or_entropy_coding_sync_idc is equal to 1, if TileId[ CtbAddrTS[ ctbAddrRS ] ] is not equal to  TileId[ CtbAddrTS[ ctbAddrRS-1 ] ] and TileId[ CtbAddrTS[ ctbAddrRS ] ] is equal to TileId[ CtbAddrTS[ ctbAddrRS ] - 1 ] and ctbAddrRS is not equal to 0 then the decoding process for binary decisions before termination specified in subclause 9.2.3.2.4 is invoked, followed by the initialisation process for the arithmetic decoding engine specified in subclause 9.2.1.4.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as described in subclause Error: Reference source not found.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause Error: Reference source not found.

For each bin of the binarization of the syntax element, which is indexed by the variable binIdx, a context index ctxIdx is derived as specified in subclause Error: Reference source not found.

For each ctxIdx the arithmetic decoding process is invoked as specified in subclause Error: Reference source not found.

The resulting sequence ( b0..bbinIdx ) of parsed bins is compared to the set of bin strings given by the binarization process after decoding of each bin. When the sequence matches a bin string in the given set, the corresponding value is assigned to the syntax element.

In case the request for a value of a syntax element is processed for the syntax element pcm-flag and the decoded value of pcm_flag is equal to 1, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit, num_subsequent_pcm, and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause Error: Reference source not found.

# Bibliography

[1] W. Wan, T. Hellman, “Comments on specifying tile support in HEVC Profiles and Levels”, JCTVC-H0463, San Jose, February 2012.

[2] F. Bossen, “Common test conditions”, JCTVC-H1100, San Jose, February 2012.

[3] A. Fuldseth, M. Horowitz, S. Xu, A. Segall, M. Zhou, JCTVC-F335, Torino, July 2011

# Patent rights declaration(s)

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).