| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0159 |
| --- | --- |

| Title: | AHG 4: Proposals on entry points signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs) Félix Henry (Orange Labs) Stéphane Pateux (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com stephane.pateux@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

Currently, signalling of entry points for tiles and wavefront substreams are done with offsets or markers. Offsets can be used for tiles or wavefront entry points, and are written in the slice header. This contribution proposes that offsets are written at the start of each substream or tile instead. It is asserted that the proposed modifications reduce encoder delay for parallel and single core scenarios. This contribution also proposes that offsets are byte aligned. It is asserted that this byte alignment facilitates offset and substream concatenation. This contribution also proposes that TileID is written after a marker only when tiles_or_entropy_coding_sync_idc is equal to 1 since this syntax element is not used otherwise. Finally, this contribution proposes that one offset per tile is mandated. It is asserted that this modification is necessary to allow picture raster scan decoding of LCUs when multiple tiles are used. The proposed modification of offset entry points produce BD-rate modifications of 0.0%, 0.0%, +0.1% (using WPP) and 0.0%, 0.0%, 0.0% (using tiles) compared to anchor in Intra, Random Access and Low Delay configurations.

# Current design

In the current design of HEVC, both WPP [1] and multiple tiles [2] require that entry points are located in the bitstream in order to allow decoder parallelization.

Entry points are used to signal the location of wavefront/tile substreams, so that a decoder can start decoding each signalled substream in parallel without parsing the rest of the bitstream beforehand.

In both cases, entry points are written using offsets in the slice header. They can also be signalled using additional markers at the start of each substream.

# Encoder delay

## Problem statement

Currently, the offsets are written in the slice header. This causes delay in the encoder since the length of all substreams must be known before the slice header can be built. When the slice encompasses the whole frame, this introduces a delay of one frame.

## Proposed solution

### Writing offsets at the beginning of each substream

The proposed solution consists in writing the offsets at the beginning of each substream in the slice data. Thus, offsets do not prevent the construction of the slice header before all substreams of the current slice are fully encoded. The encoder delay is thus reduced from one frame to one substream.

For decoding, entry points can be recovered by retrieving the offsets from their respective locations. Substreams are byte aligned so retrieving the offsets is a byte level operation.

### Byte aligning the offsets

It is proposed that offsets are byte aligned.

Since the substream is generated before its offset, but the offset is written before the substream, this implies that a bit-aligned offset would require bit shifting of the whole substream. Byte aligning the offset allows a simple byte level concatenation with the substream.

It is proposed that 2 bits are used to signal whether the substream length (in bytes) is encoded using 6, 14, 22 or 30 bits. Thus, the complete offsets are always using 8,16,24 or 32 bits. This approach is proposed because it yields the lowest rate-distortion performance. Other approaches are possible, such as using a ue(v) element followed with byte alignment.

# Picture raster scan decoding of LCUs in multiple tiles

## Problem statement

Picture raster scan decoding of LCUs in tile-divided frames has been identified as a needed feature in [3]. If LCUs are decoded in tile order by a single core, one may need to buffer most of the decoded frame before the first line of LCUs can be output. In order to avoid this, it is possible to decode the LCUs in picture raster scan order by using bitstream jumping. However, bitstream jumping requires that each tile can be individually accessed in the bitstream. This is not possible if some tiles do not have entry points.

## Proposed solution

It is proposed that one entry point offset per tile is made mandatory.

# TileID syntax element

## Problem statement

Currently, when a marker is used to signal the start of a substream, a TileID is also written. However this syntax element has no use when tiles_or_entropy_coding_sync_idc is equal to 2.

## Proposed solution

It is proposed to write this syntax element only when tiles_or_entropy_coding_sync_idc is equal to 1.

# Performance

## Test conditions

The current proposal has been tested using common test conditions [4] with either tiles or WPP activated.

## Proposed offsets with multiple tiles

Tile sizes are set according to JCTVC-F335 [3] (tile size are as close as possible to class D images). The anchor uses HM6.1 with tiles activated, and the proposal uses the same configuration with the new offset coding approach described in the previous section. Table 1 shows the rate-distortion performance change.

Table 1: Rate-distortion performance of the proposed modifications when using tiles

## Proposed offsets with WPP

WPP is set using one WPP substream per LCU line (thus achieving about the same degree of parallel processing as tiles in the prevous experiment). The anchor uses HM6.1 with WPP activated, and the proposal uses the same configuration with the new offset coding approach described in the previous section. Table 2 shows the rate-distortion performance change.

Table 2: Rate-distortion performance of the proposed modifications when using WPP

# Proposed text modifications

## Syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_interleaving_flag | u(1) |
| slice_sample_adaptive_offset_flag | u(1) |
| if( slice_sao_interleaving_flag && slice_sample_adaptive_offset_flag ) { |  |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| } |  |
| } |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_aps_enabled_flag ¦ ¦ ( sample_adaptive_offset_enabled_flag && !slice_sao_interleaving_flag ) ¦ ¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) { |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| } |  |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc > 0 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| If ((tiles_or_entropy_coding_sync_idc == 1 && (CtbAddrTS == CtbAddrRStoTS[SliceCtbAddrRS] ¦¦ TileId[CtbAddrTS] != TileId[CtbAddrTS-1]) && TileId[CtbAddrTS] != TileId[CtbAddrRStoTS[PicWidthInCtbs * PicHeightInCtbs -1]]) ¦¦ (tiles_or_entropy_coding_sync_idc == 2 && ( num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs < num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 && ) { |  |
| entry_point_prefix | u(2) |
| entry_point_offset | u(v) |
| } |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| AddrUp= CtbAddrRS – PicWidthInCtbs |  |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| if(sao_cb_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| If (tiles_or_entropy_coding_sync_idc = = 1) |  |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

## Semantics

entry_point_prefix specifies the length, in bits, of the entry_point_offset syntax element according to the following table.

entry_point_offset specifies an entry point offset, in bytes and shall be represented by the number of bits specified by entry_point_prefix according to the following table. The coded slice NAL unit consists of  subsets and each subset consists of the bytes between one entry_point_offset element and the next entry_point_prefix, exclusive.

When tiles_or_entropy_coding_sync_idc is equal to 1, each subset shall contain all coded bits of one tile, and the number of subsets shall be equal to the number of tiles in the slice.

When tiles_or_entropy_coding_sync_idc is equal to 2, subset k, for each of all the possible k values, shall contain all coded to be decoded by bitstream pointer BitStreamTable[k].

Table X – Name association to slice_type

| - entry_point_prefix | - Number of bits used to decoded entry_point_offset |
| --- | --- |
| - 0 | - 6 |
| - 1 | - 14 |
| - 2 | - 22 |
| - 3 | - 30 |

# Bibliography

[1] C. Gordon, F. Henry, S. Pateux, “Wavefront Parallel Processing for HEVC Encoding and Decoding”, JCTVC-F274, Torino, July 2011.

[2] A. Fuldseth, M. Horowitz, S. Xu, A. Segall, M. Zhou, JCTVC-F335, Torino, July 2011

[3] W. Wan, T. Hellman, “Comments on specifying tile support in HEVC Profiles and Levels”, JCTVC-H0463, San Jose, February 2012.

[4] F. Bossen, “Common test conditions”, JCTVC-H1100, San Jose, February 2012.

# Patent rights declaration(s)

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).