| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27-07 APRIL , 2012 | Document: JCTVC-I0160 |
| --- | --- |

| Title: | CE1, Test2.2: Reduced number of edge offsets per LCU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Samsung Electronics Co. Ltd., Qualcomm Inc., MediaTek Inc.. |  |  |

_____________________________

# Abstract

In this test number of edge offsets per LCU was reduced from 4 (in HM6.0) to 2 or 1 in SAO Interleaving mode. Reduced number of offsets allows memory reduction for offset values storage and increases throughput. Test results for 1 edge offset per LCU show 0.1%, 0.3%, 0.3% average Luma BR-rate improvement for LCU sizes 64x64, 32x32 and 16x16 correspondently. If 2 edge offsets are used per LCU then performance improvement is 0.0%, 0.1%, 0.2% for LCU sizes 64x64, 32x32 and 16x16 correspondently.

# Introduction

In HM 6.0 SAO design all SAO parameters are assigned per LCU. So for each LCU first SAO type is encoded. It can be SAO off or on of 4 Edge Offset (EO) types or Band Offset (BO). If SAO is used in current LCU then 4 offset values are encoded. Additionally if SAO type is band correction then position of first corrected band is signaled.

It is clear that SAO parameters information assigned per LCU is heavy. This is the reason why SAO Interleaved mode gain degenerates quickly if LCU size becomes smaller.

The goal of this contribution is to reduce SAO parameters information using smaller than 4 edge offsets per LCU. We utilized statistical information about EOs distribution described in [1], [2]

# Algorithm description

One Edge Offset per LCU case.

There are 4 categories of EO in HM6.0:

- Category 0 is local minimum;

- Category 1 is concave edge;

- Category 2 convex edge;

- Category 3 means local maxim.

During collection statistics for [2] the strong correlation between EO in different categories was noticed. For example EO(0) = -EO(3) in many cases. Weak extreme correction offsets EO(1) and EO(2) have the same signs as EO(0) and EO(3) correspondently but smaller magnitude.

If we allow only one EO per LCU then we signal EO(0) as positive value to keep consistency with HM6.0. Remaining offsets are derived from this value:

EO(0) >0;

EO(1) =  EO(0) >>1;                     (1)

EO(2) = -(EO(0) >>1);

EO(3) = - EO(0).

Ratio 2 between EO(1) and EO(0) (E(3) and EO(2)) was chosen in order to simplify allow fast calculation.

Signaling for EO(1) for example and deriving remaining offsets from reduces SAO side information further but introduce bigger error and so design (1) was tested in CE1.

As it is mentioned in (1) signaled value EO(0) should not be zero because this is equivalent to SAO disabled in this LCU.

Two Edge Offset per LCU case.

If we allow only 2EOsper LCU then we signal EO(0) and EO(1) as non-negative values (again for consistency with HM6.0). Remaining offsets are derived from these values as follows:

EO(0) 0; EO(1)0;

EO(2) = -EO(1) ;                     (2)

EO(3) = - EO(0).

Internally we tried the valiant when EO(0) and EO(3) are signaled and remaining offsets are derived from them as in (1). But there was almost no performance difference with (2). So variant (2) was chosen because EO(1) statistically has lower magnitude and requires less bits for encoding.

# Simulation Results

Both 1 EO and 2 EOs per LCU concepts were implemented on top of CE1 s/w and tested according to [2]. Table 1 reports summary of simulation results vs CE1 anchor if interleaving SAO mode is used. We report average gain across all 8 test cases: I-main/ he10, RA--main/ he10, LD--main/ he10, LDP--main/ he10. Class F is included into averaging when Table 1 numbers were calculated.

Only number of EOs was reduced in these tests. Band offset coding was not modified compare to CE1 anchor.

For all LCU sizes 1 EO per LCU shows better performance compare to anchor (4 EOs per LCU) and variant when 2 EOs are used per LCU.

Table 1. Test results summary compare to CE1 anchor (SAO Interleaving mode).

| LCU size | 1 EO per LCU |  |  | 2 EOs per LCU |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y, BD-rate | U, BD-rate | V, BD-rate | Y, BD-rate | U, BD-rate | V, BD-rate |
| 64x64 | -0.1% | 0.0% | 0.1% | 0.0% | 0.7% | 0.8% |
| 32x32 | -0.3% | -0.3% | -0.3% | -0.1% | 0.1% | 0.1% |
| 16x16 | -0.3% | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% |

Since SAO gain has biggest gain for class F additionally to Table 1 we report average gain if class F is excluded from averaging in Table 1* (this was requested by CE members). There is almost no difference between Tables 1 and 1* which confirms that proposed reduction for EOs number works without high dependency on content.

Table 1*. Test results summary compare to CE1 anchor (class F is excluded from averaging).

| LCU size | 1 EO per LCU |  |  | 2 EOs per LCU |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y, BD-rate | U, BD-rate | V, BD-rate | Y, BD-rate | U, BD-rate | V, BD-rate |
| 64x64 | -0.1% | -0.1% | 0.0% | 0.0% | 0.7% | 0.8% |
| 32x32 | -0.3% | -0.3% | -0.4% | -0.2% | 0.0% | 0.0% |
| 16x16 | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |

More details about test results can be found in appendix (Tables 3.x, 4.x) and excel spreadsheet attached to this contribution. An accurate run-time measurement was not mandatory according to CE1 description. Only for LCU size 64 both anchor and modification were simulate using the same machine. For other cases run-time is removed from Tables 3.x and 4.x in order to avoid confusion. Proposed changes should not introduce noticeable run-time change. .

Test data for additional reduction for number of BOs per LCU will be show in separate contribution.

S/w and test data were release to all CE1 participants. We would like to thank Cisco for very careful cross-check.

# Subjective quality test

We have found no visual quality difference between CE1 anchor and proposed modification. Decoded yuv files will be available for visual quality check during meeting. Several figures illustrating visual quality if proposed method is applied together with number of Band offsets reduction are included into [4].

# CD changes

This contribution requires following CD text changes.

#### Sample adaptive offset CABAC syntax

| sao_offset_cabac( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 0 ] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 0 ]++; |  |
| sao_offset[ cIdx ][ rx][ ry ][ 1 ] = sao_offset[ cIdx ][ rx][ ry ][ 0 ]>>1; |  |
| sao_offset[ cIdx ][ rx][ ry ][ 2 ] =  (sao_offset[ cIdx ][ rx][ ry ][ 0 ]>>1); |  |
| sao_offset[ cIdx ][ rx][ ry ][ 3 ] =  sao_offset[ cIdx ][ rx][ ry ][ 0 ]; |  |
| } |  |

Instead parsing 4 unsigned values of EOs only EO(0) parsing is needed. Edge offsets for remaining categories are derived.

# Bin statistics

As was mentioned above only one edge offset parsing is needed per LCU compare to 4 edge offsets per LCU in HM6.0. Offsets values in SAO Interleaving mode are encoded using CBAC with probability up-date. So reduction of these bins helps throughput.]

Table 2 reports number of bins for edge off-set encoding reduction compare to CE1 anchor. We collected this statistics using CE1 anchor for LCU size 64x64.

Table 2. Number of bins for EO encoding for proposed modification compare to CE1 anchor for LCU size 64x64.

|  | Main | HE10 |
| --- | --- | --- |
| Intra | 23.3% | 39.5% |
| RA | 24.9% | 42.4% |
| LDB | 27.9% | 27.5% |
| LDP | 25.9% | 41.2% |

In average compare to anchor proposed modification reduces Edge Offset bins 4 times in main test condition and 3 times for HE-10.

# Conclusions

Based on reported test results

0.1% BD rate reduction if LCU size is 64x64;

0.3% BD rate reduction if LCU size is 32x32;

0.3% BD rate reduction if LCU size is 16x16;

Samsung, Qualcomm, and MediaTek propose to reduce the number of edge offsets to 1 per LCU. This helps to reduce average memory size for offset storage and helps throughput since less number of bins need to be parsed.

# Patent rights declarations

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), “Non-CE8: Offset coding in SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-G222, November, 2011

- E. Alshina, A. Alshin, J.H. Park (Samsung), Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei (MediaTek), I. S. Chong, M. Karczewicz(Qualcomm), “Non-CE8: Sample adaptive offset with constrained ranges of edge offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0114, February 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012

- E. Alshina, A. Alshin, J.H. Park (Samsung), Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei (MediaTek), I. S. Chong, M. Karczewicz(Qualcomm), “Non-CE1, Test2.1: Reduced number of band offsets per LCU,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0162, April 2012.

# Appendix: Test results

Table 3.1. Test results for 1 EO per LCU. Anchor is CE1 s/w. LCU size 16x16.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% | 0.0% | 0.0% | -0.1% |
| Class B | 0.1% | 0.3% | 0.3% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.3% | 0.3% | -0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.3% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class E | 0.1% | 0.4% | 0.4% | 0.0% | 0.1% | 0.1% |
| Class F | -0.3% | -0.1% | -0.2% | -0.7% | 0.0% | 0.0% |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | -0.4% | 0.0% | 0.3% | -0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.2% | 0.1% | -0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.2% | -0.1% | 0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | -0.3% | -0.3% | -0.1% | -0.3% | -0.1% | 0.0% |
| Overall | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.0% |
|  | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.3% | -0.2% | 0.1% | -0.3% |
| Class C | 0.0% | 0.4% | 0.5% | -0.1% | -0.1% | 0.2% |
| Class D | 0.0% | 0.4% | 0.0% | -0.1% | 0.5% | -0.1% |
| Class E | -0.2% | 0.3% | 0.4% | -0.1% | 0.2% | -0.1% |
| Class F | -0.1% | -2.7% | -1.0% | -0.4% | -1.0% | 1.6% |
| Overall | -0.1% | -0.3% | 0.0% | -0.2% | -0.1% | 0.3% |
|  | -0.1% | -0.5% | 0.1% | -0.2% | -0.1% | 0.5% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.5% | -0.6% | 0.8% | -0.7% | -0.3% | -0.5% |
| Class C | -1.4% | -0.3% | 0.1% | -0.7% | -0.2% | -0.3% |
| Class D | -1.1% | -0.2% | -0.3% | -0.8% | 0.6% | -1.2% |
| Class E | -1.8% | 0.4% | 0.2% | -0.7% | -0.5% | -0.5% |
| Class F | -0.5% | -0.4% | -0.7% | -0.7% | -0.4% | 0.0% |
| Overall | -1.5% | -0.3% | 0.0% | -0.7% | -0.2% | -0.5% |
|  | -1.5% | -0.3% | 0.1% | -0.7% | -0.2% | -0.5% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table 3.2. Test results for 1 EO per LCU. Anchor is CE1 s/w. LCU size 32x32.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | 0.0% | -0.1% | -0.3% | -0.6% |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class C | -0.2% | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% |
| Class D | -0.2% | -0.4% | -0.6% | -0.3% | -0.2% | -0.3% |
| Class E | -0.2% | 0.0% | 0.0% | -0.2% | 0.2% | 0.1% |
| Class F | -0.2% | -0.3% | -0.1% | -0.2% | -0.2% | 0.1% |
| Overall | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% |
|  | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 78% |  |  | 86% |  |  |
| Dec Time[%] | 87% |  |  | 86% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | 0.4% | 0.3% | -0.1% | -0.1% | -0.4% |
| Class B | -0.1% | -0.1% | 0.2% | -0.2% | -0.1% | 0.0% |
| Class C | -0.1% | 0.2% | 0.1% | -0.3% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | -0.2% | -0.4% | -0.2% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -0.1% | 0.2% | -0.3% | -0.2% | 0.1% |
| Overall | -0.2% | 0.1% | 0.1% | -0.2% | -0.1% | -0.1% |
|  | -0.2% | 0.1% | 0.2% | -0.2% | -0.2% | -0.1% |
| Enc Time[%] | 80% |  |  | 83% |  |  |
| Dec Time[%] | 90% |  |  | 87% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | -0.3% | -0.2% | -0.2% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | -0.4% | 0.0% | 0.2% |
| Class D | -0.1% | -0.4% | -0.3% | -0.2% | -0.2% | -0.4% |
| Class E | 0.3% | -0.7% | -0.5% | -0.3% | -0.2% | -0.5% |
| Class F | 0.0% | 0.3% | 0.8% | -0.4% | -0.5% | 0.3% |
| Overall | 0.1% | -0.2% | -0.1% | -0.3% | -0.2% | -0.1% |
|  | 0.1% | -0.2% | 0.0% | -0.3% | -0.2% | -0.2% |
| Enc Time[%] | 79% |  |  | 82% |  |  |
| Dec Time[%] | 95% |  |  | 93% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.4% | -1.0% | -0.7% | -0.5% | -1.0% | 0.0% |
| Class C | -0.8% | -0.8% | -0.8% | -0.7% | -0.5% | -0.2% |
| Class D | -0.6% | -0.8% | -1.6% | -0.6% | -0.9% | -1.3% |
| Class E | -0.6% | -1.1% | -1.0% | -0.9% | -0.8% | -0.2% |
| Class F | -0.2% | -0.1% | -0.2% | -0.5% | 0.0% | -0.1% |
| Overall | -0.8% | -0.8% | -0.9% | -0.6% | -0.7% | -0.4% |
|  | -0.8% | -0.8% | -0.9% | -0.6% | -0.5% | -0.4% |
| Enc Time[%] | 80% |  |  | 83% |  |  |
| Dec Time[%] | 94% |  |  | 92% |  |  |

Table 3.3. Test results for 1 EO per LCU. Anchor is CE1 s/w. LCU size 64x64.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.2% | 0.3% | -0.1% | -0.1% | -0.1% |
| Class B | -0.1% | 0.1% | 0.2% | -0.1% | 0.1% | 0.1% |
| Class C | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% |
| Class D | -0.1% | -0.2% | -0.3% | -0.2% | -0.3% | -0.4% |
| Class E | -0.2% | 0.0% | 0.1% | -0.2% | 0.4% | 0.1% |
| Class F | 0.0% | 0.2% | 0.4% | 0.0% | 0.2% | 0.2% |
| Overall | -0.1% | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% |
|  | -0.1% | 0.1% | 0.1% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 1.2% | 0.9% | -0.2% | 0.0% | 0.0% |
| Class B | 0.1% | 0.5% | 0.8% | -0.1% | 0.1% | 0.2% |
| Class C | 0.0% | 0.4% | 0.5% | -0.2% | -0.1% | 0.0% |
| Class D | -0.1% | 0.5% | 0.5% | -0.1% | -0.1% | -0.4% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.4% | 0.6% | -0.2% | 0.2% | 0.4% |
| Overall | 0.0% | 0.6% | 0.7% | -0.2% | 0.0% | 0.0% |
|  | 0.0% | 0.6% | 0.7% | -0.2% | -0.1% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.8% | 1.0% | 0.0% | -0.5% | -0.5% |
| Class C | 0.1% | 0.4% | 0.7% | -0.3% | -0.4% | -0.4% |
| Class D | -0.1% | 0.2% | 0.3% | -0.2% | -0.5% | -0.5% |
| Class E | 0.7% | 1.1% | 0.6% | 0.0% | -0.3% | -0.5% |
| Class F | 0.2% | 0.6% | 0.8% | 0.0% | 0.8% | 0.5% |
| Overall | 0.2% | 0.6% | 0.7% | -0.1% | -0.2% | -0.3% |
|  | 0.2% | 0.6% | 0.7% | -0.1% | -0.3% | -0.3% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.2% | 0.8% | -0.2% | -1.3% | -1.0% |
| Class C | -0.1% | -0.1% | 0.0% | -0.4% | -1.1% | -0.8% |
| Class D | -0.2% | -0.3% | -0.5% | -0.4% | -1.3% | -1.0% |
| Class E | 0.8% | 0.6% | 0.2% | -0.4% | -1.7% | -1.8% |
| Class F | 0.2% | 0.7% | 1.1% | -0.3% | 0.3% | 0.3% |
| Overall | 0.0% | 0.2% | 0.4% | -0.3% | -1.0% | -0.8% |
|  | 0.0% | 0.2% | 0.4% | -0.3% | -0.9% | -0.8% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table 4.1. Test results for 2 EOs per LCU. Anchor is CE1 s/w. LCU size 16x16

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class D | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.2% | -0.2% | -0.4% | -0.1% | -0.1% |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class B | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | -0.2% | -0.1% | -0.2% | -0.1% | 0.0% | 0.0% |
| Class D | -0.1% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% |
| Overall | -0.2% | -0.2% | -0.2% | -0.1% | 0.0% | -0.1% |
|  | -0.2% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% | 0.2% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | 0.1% |
| Class D | -0.1% | -0.2% | -0.2% | 0.0% | 0.6% | 0.5% |
| Class E | -0.2% | 0.1% | 0.0% | -0.1% | 0.0% | -0.2% |
| Class F | 0.1% | -0.9% | -1.0% | -0.2% | -1.5% | -1.3% |
| Overall | -0.1% | -0.3% | -0.3% | -0.1% | -0.3% | -0.1% |
|  | -0.1% | -0.3% | -0.2% | -0.1% | -0.3% | -0.2% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.1% | -0.3% | -0.3% | -0.4% | -0.2% | -1.0% |
| Class C | -0.6% | -0.4% | -0.3% | -0.4% | -0.2% | -0.1% |
| Class D | -0.5% | -0.5% | 0.2% | -0.4% | 0.3% | -1.5% |
| Class E | -0.9% | -0.2% | -0.6% | -0.3% | -0.6% | -0.4% |
| Class F | 0.0% | 0.0% | -0.9% | -0.4% | 0.4% | -0.2% |
| Overall | -0.6% | -0.3% | -0.4% | -0.4% | 0.0% | -0.7% |
|  | -0.7% | -0.2% | -0.3% | -0.4% | -0.1% | -0.7% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table 4.2. Test results for 2 EOs per LCU. Anchor is CE1 s/w. LCU size 32x32

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.3% |
| Class B | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% |
| Class C | -0.2% | 0.1% | 0.2% | -0.2% | 0.0% | 0.1% |
| Class D | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% |
| Class E | -0.2% | 0.1% | 0.0% | -0.2% | 0.1% | 0.1% |
| Class F | -0.1% | 0.2% | 0.3% | -0.2% | 0.0% | 0.2% |
| Overall | -0.1% | 0.1% | 0.1% | -0.2% | 0.0% | -0.1% |
|  | -0.1% | 0.1% | 0.1% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | 0.4% | 0.4% | -0.1% | 0.0% | -0.1% |
| Class B | 0.0% | 0.1% | 0.3% | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | 0.4% | 0.5% | -0.2% | 0.0% | 0.0% |
| Class D | -0.1% | 0.2% | 0.2% | -0.1% | -0.3% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.1% | 0.4% | 0.7% | -0.2% | 0.0% | 0.2% |
| Overall | -0.2% | 0.3% | 0.3% | -0.1% | -0.1% | 0.0% |
|  | -0.1% | 0.3% | 0.4% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | -0.2% | -0.1% | 0.1% | 0.2% |
| Class C | 0.0% | 0.3% | 0.7% | -0.2% | 0.1% | 0.3% |
| Class D | -0.1% | -0.1% | 0.1% | -0.1% | -0.2% | -0.8% |
| Class E | 0.1% | 0.3% | -0.3% | -0.2% | 0.0% | -0.1% |
| Class F | 0.2% | 0.8% | 1.3% | -0.2% | 0.2% | -1.4% |
| Overall | 0.1% | 0.3% | 0.3% | -0.1% | 0.0% | -0.4% |
|  | 0.1% | 0.2% | 0.3% | -0.1% | 0.0% | -0.5% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.0% | 0.1% | -0.3% | -0.5% | 0.0% |
| Class C | -0.2% | 0.3% | 0.4% | -0.4% | 0.0% | 0.3% |
| Class D | -0.2% | -0.2% | -0.2% | -0.3% | 0.3% | -0.6% |
| Class E | -0.1% | 0.2% | 0.0% | -0.3% | 0.2% | 0.0% |
| Class F | 0.1% | 0.2% | 0.8% | -0.2% | 1.0% | -0.4% |
| Overall | -0.2% | 0.1% | 0.2% | -0.3% | 0.2% | -0.1% |
|  | -0.2% | 0.1% | 0.1% | -0.3% | 0.2% | -0.1% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table 4.3. Test results for 2 EOs per LCU. Anchor is CE1 s/w. LCU size 64x64.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.5% | 0.7% | -0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.5% | 0.7% | -0.1% | 0.2% | 0.3% |
| Class C | 0.0% | 0.4% | 0.5% | -0.2% | 0.1% | 0.2% |
| Class D | -0.1% | 0.3% | 0.3% | -0.1% | 0.0% | -0.1% |
| Class E | -0.1% | 0.5% | 0.7% | -0.2% | 0.4% | 0.4% |
| Class F | 0.1% | 0.5% | 0.6% | -0.1% | 0.3% | 0.4% |
| Overall | 0.0% | 0.4% | 0.6% | -0.1% | 0.2% | 0.2% |
|  | 0.0% | 0.5% | 0.6% | -0.1% | 0.1% | 0.2% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 1.4% | 1.1% | -0.1% | 0.1% | -0.1% |
| Class B | 0.3% | 1.1% | 1.1% | -0.1% | 0.3% | 0.4% |
| Class C | 0.0% | 0.8% | 1.0% | -0.1% | 0.2% | 0.4% |
| Class D | 0.0% | 0.8% | 0.8% | -0.1% | 0.1% | 0.3% |
| Class E |  |  |  |  |  |  |
| Class F | 0.2% | 1.0% | 1.1% | -0.2% | 0.2% | 0.5% |
| Overall | 0.1% | 1.0% | 1.0% | -0.1% | 0.2% | 0.3% |
|  | 0.1% | 1.0% | 1.0% | -0.1% | 0.2% | 0.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4% | 1.6% | 1.8% | 0.0% | 0.1% | 0.6% |
| Class C | 0.2% | 1.2% | 1.6% | -0.2% | 0.3% | 0.4% |
| Class D | 0.0% | 1.1% | 1.3% | -0.1% | 0.3% | -0.1% |
| Class E | 0.4% | 1.9% | 2.6% | 0.0% | 0.3% | 0.4% |
| Class F | 0.5% | 1.5% | 1.5% | -0.1% | 0.8% | 2.1% |
| Overall | 0.3% | 1.5% | 1.7% | -0.1% | 0.4% | 0.7% |
|  | 0.3% | 1.4% | 1.7% | -0.1% | 0.3% | 0.6% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5% | 2.1% | 2.6% | -0.1% | -0.3% | 0.2% |
| Class C | 0.3% | 1.2% | 1.8% | -0.3% | 0.0% | 0.1% |
| Class D | 0.1% | 1.5% | 0.9% | -0.2% | 0.0% | -0.3% |
| Class E | 1.1% | 3.6% | 2.6% | -0.2% | -0.7% | 0.1% |
| Class F | 0.4% | 1.8% | 1.8% | -0.2% | 0.7% | 0.7% |
| Overall | 0.4% | 2.0% | 1.9% | -0.2% | 0.0% | 0.2% |
|  | 0.4% | 2.0% | 2.0% | -0.2% | 0.0% | 0.3% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |