| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0161 |
| --- | --- |

| Title: | CE1 Test1.5: Results of forcing chroma off when luma is off in interleaving mode, CE1 Test4.1: Removing run prediction in APS mode, and CE1 Test4.2: Region-based syntax in APS mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Chih-Wei Hsu, Ching-Yeh Chen, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, cw.hsu, chingyeh.chen, chiayang.tsai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the interleaving mode of HM-6.0, SAO parameters of three components in one LCU are coded independently. CE1 Test1.5 forces chroma of one LCU to be off without sending anything for chroma when luma of the LCU is off. It is reported that in comparison with CE1 anchors, luma BD-rates of CE1 Test1.5 are -0.4%, -0.7%, and -1.4% for LCU size equal to 64x64, 32x32, and 16x16, respectively, in both Main-LDB and HE10-LDB conditions without class F sequences.

In the APS mode of HM-6.0, run coding is used to share SAO parameters for multiple LCUs on the same LCU row, and the run value can be predicted. CE1 Test4.1 removes the run prediction. It is reported that in comparison with CE1 anchors, luma BD-rates of CE1 Test4.1 are 0.1% and 0.0% for Main-RA and HE10-RA, respectively, without class F sequences.

In the APS mode of HM-6.0, each LCU is allowed to send its own SAO parameters, which makes on-chip storage of all decoded offsets from APS very expensive because the LCU size can be 16x16. CE1 Test4.2 replaces the LCU-based syntax by a region-based syntax for the APS mode, where the coded region size is power of two and must be equal to or larger than 64x64, 128x128, and 256x256 for levels 1-4.3, levels 5-5.2, and levels 6-6.2, respectively. It is reported that in comparison with CE1 anchors, luma BD-rates of CE1 Test4.2 are 0% for both Main-RA and HE10-RA without class F sequences.

# Algorithm description

In HM-6.0, SAO has two signaling modes: APS mode (sending SAO parameters at picture level) and interleaving mode (sending SAO parameters at LCU level). APS mode is more suitable for high coding efficiency applications, and the encoding latency is one picture. Interleaving mode is more suitable for low complexity applications, and the encoding latency is one LCU. In this contribution, three coding efficiency improvement method for interleaving mode and two complexity reduction methods for APS mode are proposed as CE1 Test1.5, CE1 Test4.1, and CE1 Test4.2 as in CE1 description [1].

## CE1 Test1.5: Force chroma off when luma is off in interleaving mode

In the interleaving mode of HM-6.0, SAO parameters of three components in one LCU are coded independently. It is found that when luma of one LCU is off, two chroma components of the LCU are very likely to be off. Therefore, CE1 Test1.5 forces two chroma components of one LCU to be off without sending anything for chroma when the luma component of the LCU is off. The syntax change of CE1 Test1.5 is highlighted in Table 1.

Table 1.  Syntax change in CE1 Test1.5

| sao_unit_cabac( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if(cIdx==0 ¦¦ sao_type_idx[ 0 ][ rx ][ ry ]!=0) { |  |
| if( rx > 0 && CtbAddrInSlice != 0 ) |  |
| sao_merge_left_flag | ae(v) |
| if( !sao_merge_left_flag ) |  |
| if( ry > 0 && ( AddrUp > 0 ¦ ¦ slice_loop_filter_across_slices_enabled_flag ) ) |  |
| sao_merge_up_flag | ae(v) |
| if( !sao_merge_up_flag ) |  |
| sao_offset_cabac( rx, ry, cIdx ) |  |
| } |  |
| } |  |

Please note that CE1 Test1.5 can also be done in a non-normative way on the encoder side.

## CE1 Test4.1: Remove run prediction in APS mode

In the APS mode of HM-6.0, run coding is used to share SAO parameters for multiple LCUs on the same LCU row, and a run value can be predicted from its upper LCU. In this way, a run line buffer is needed. In order to save the run line buffer, CE1 Test4.1 removes the run prediction. The syntax change of CE1 Test4.1 is highlighted in Table 2. The sao_run replaces the sao_run_diff.

Table 2.  Syntax change in CE1 Test4.1

| sao_unit_vlc( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| if( !sao_repeat_row_flag[ cIdx ] ) { |  |
| if( rx == 0 ¦¦ saoRun[ cIdx ][ rx ][ ry ] < 0 ) { |  |
| sao_run | u(v) |
| saoRun[ cIdx ][ rx ][ ry ] = sao_run |  |
| if( ry > 0 ) |  |
| sao_merge_up_flag | u(1) |
| if( !sao_merge_up_flag ) |  |
| sao_offset_vlc( rx, ry, cIdx) |  |
| } |  |
| saoRun[ cIdx ][ rx + 1 ][ ry ] = saoRun[ cIdx ][ rx ][ ry ] − 1 |  |
| } |  |
| } |  |

## CE1 Test4.2: Region-based SAO signaling in APS mode

In the APS mode of HM-6.0, each LCU is allowed to send its own SAO parameters, which makes on-chip storage of all decoded offsets from APS very expensive because the LCU size can be 16x16. In order to reduce the memory, CE1 Test4.2 replaces the LCU-based syntax by a region-based syntax for the APS mode, where the region size is coded and must be power of two. Moreover, the region size must be equal to or larger than 64x64, 128x128, and 256x256 for levels 1-4.3, levels 5-5.2, and levels 6-6.2, respectively, which means the required memory sizes are only 1/16, 1/64, and 1/256 of the original memory sizes, respectively. The syntax change of CE1 Test4.2 is highlighted in Table 3, where (1 << (log2_sao_unit_width_minus6 + 6)) specifies the region width of each SAO unit, and (1 << (log2_sao_unit_height_minus6 + 6)) specifies the region height of each SAO unit. Other syntax elements such as sao_num_lcu_in_width and sao_num_lcu_in_height are renamed as sao_num_unit_in_width and sao_num_unit_in_height respectively to match the meanings. In the simulation, the minimum region size is set to 128x128 for class A sequences and 64x64 for rest sequences.

Table 3.  Syntax change in CE1 Test4.2

| aps_sao_param( ) { | Descriptor |
| --- | --- |
| log2_sao_unit_width_minus6 | ue(v) |
| log2_sao_unit_height_minus6 | ue(v) |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| sao_num_unit_in_width_minus1 | ue(v) |
| sao_num_unit_in_height_minus1 | ue(v) |
| … |  |
| } |  |

# Simulation results

Table 4 shows the results of CE1 Test1.5 with normative syntax changes. Three different LCU sizes are tested under Main-LDB and HE10-LDB conditions. The anchor is SAO-on with non-normative changes defined in CE1. Only run times of 64x64 are accurate. As can be seen, luma gains without class F sequences are around 0.4%, 0.7%, and 1.4% for 64x64, 32x32, and 16x16, respectively. For 64x64, chroma losses of 0.9-1.1% and 0.3% are observed in Main-LDB and HE10-LDB, respectively. For 32x32, chroma losses of 0.1-0.3% and chroma gains of 0.1-0.3% are observed in Main-LDB and HE10-LDB, respectively. For 16x16, chroma gains of 1.0-1.2% are observed, but it is suggested to use SAO-off for 16x16 because in this case SAO-off has better coding efficiency than SAO-on. For 64x64 and 32x32, the luma gains seem to justify the chroma losses, so normatively forcing chroma off when luma is off can improve the coding efficiency of the current SAO.

Table 5 shows the results of CE1 Test1.5 without normative syntax changes. Run times are not accurate. In comparison with CE1 Test1.5 with normative syntax changes, all the luma gains disappear, and the chroma coding efficiency is also degraded. This means if we want to force chroma off when luma is off, the normative method is needed, and the non-normative method does not worth.

Table 6 shows the results of CE1 Test4.1. It is shown that removing run prediction in APS mode does not causes significant coding efficiency loss. In Main-RA, the loss is only 0.1%. In HE10-RA, there is no impact on coding efficiency.

Table 7 shows the results of CE1 Test4.2. It is reported that region-based SAO syntax in APS mode does not change luma BD-rates and has 0.2% chroma coding efficiency loss. In fact, only class A sequences have 0.7-1.0% chroma losses, while the rest sequences have no loss at all.

# Conclusions

In the interleaving mode of SAO, CE1 Test1.5 normatively forces chroma of one LCU to be off when luma of the LCU is off. It is reported that luma BD-rates of CE1 Test1.5 are -0.4%, -0.7%, and -1.4% for LCU size equal to 64x64, 32x32, and 16x16, respectively, in both Main-LDB and HE10-LDB conditions.

In the APS mode of SAO, CE1 Test4.1 removes the run prediction. It is reported that luma BD-rates of CE1 Test4.1 are 0.1% and 0.0% for Main-RA and HE10-RA, respectively.

In the APS mode of SAO, CE1 Test4.2 replaces the LCU-based syntax by a region-based syntax. It is reported that luma BD-rates of CE1 Test4.2 are 0% for both Main-RA and HE10-RA.

# References

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1101, February, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 4.  Results of CE1 Test1.5 with normative syntax changes

Table 5.  Results of CE1 Test1.5 without normative syntax changes

Table 6.  Results of CE1 Test4.1

Table 7.  Results of CE1 Test4.2