| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27-07 APRIL , 2012 | Document: JCTVC-I0162 |
| --- | --- |

| Title: | Non-CE1, Test2.1: Reduced number of band offsets per LCU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Samsung Electronics Co. Ltd., Qualcomm Inc., MediaTek Inc.. |  |  |

_____________________________

# Abstract

Our CE1 contribution [1] reports performance improvement when number of edge offsets was reduced per LCU from 4 in HM6.0 to 1 or 2 in SAO Interleaving mode. In this test additionally the number of band offsets per LCU was reduced to be equal with number of EOs per LCU. Together proposed modifications allows memory reduction for offsets values storage both in average and the worst case and increase throughput of SAO parsing process. Two combinations 1EO & 1BO per LCU and 2 EOs & 2 BOs per LCU are presented.

# Introduction

In HM 6.0 SAO design all SAO parameters are assigned per LCU. So for each LCU first SAO type is encoded. It can be SAO off or on of 4 Edge Offset (EO) types or Band Offset (BO). If SAO is used in current LCU then 4 offset values are encoded. Additionally if SAO type is band correction then position of first corrected band is signaled.

It is clear that SAO parameters information assigned per LCU is heavy. This is the reason why SAO Interleaved mode gain degrades fast if LCU size becomes smaller.

Additionally decoder should store offsets in buffer which in the worst case (4K video, 16x16 LCU) is ~1500 bytes, which is not small.

The goal of this contribution is to reduce SAO parameters information using smaller than 4 edge offsets per LCU. We utilized statistical information about EOs distribution described in [1], [2].

# Algorithm description

One Offset per LCU.

There are 4 categories of EO in HM6.0:

- Category 0 is local minimum;

- Category 1 is concave edge;

- Category 2 convex edge;

- Category 3 means local maxim.

During collection statistics for [2] the strong correlation between EO in different categories was noticed. For example EO(0) = -EO(3) in many cases. Weak extreme correction offsets EO(1) and EO(2) have the same signs as EO(0) and EO(3) correspondently but smaller magnitude.

If we allow only one EO per LCU then we signal EO(0) as positive value to keep consistency with HM6.0. Remaining offsets are derived from this value:

EO(0) >0;

EO(1) =  EO(0) >>1;                     (1)

EO(2) = -(EO(0) >>1);

EO(3) = - EO(0).

Ratio 2 between EO(1) and EO(0) (E(3) and EO(2)) was chosen in order to simplify allow fast calculation.

Additionally number of band offsets per LCU was set 1 (SAO_BO_LEN  = 1 in HM6.0 s/w ).

If only one offset per LCU is applied then it cannot be zero. All zero offsets in LCU are equivalent to SAO disabled in this LCU. This was taken into account during implementation: abs(offset) -1 was encoded.

Two Offsets per LCU.

If we allow only 2Eos per LCU then we signal EO(0) and EO(1) as non-negative values (again for consistency with HM6.0). Remaining offsets are derived from these values as follows:

EO(0) 0; EO(1)0;

EO(2) = -EO(1) ;                     (2)

EO(3) = - EO(0).

Internally we tried the valiant when EO(0) and EO(3) are signaled and remaining offsets are derived from Additionally number of band offsets per LCU was set 2 in this test (SAO_BO_LEN  = 2 in HM6.0 s/w ).

# BO&EO coding unification

In current SAO design EO is encoded and unsigned value but BO is encoded as signed value. In order to remove inconsistency we suggest encoding BO sign separately using CABAC by pass mode and encode EO and BO magnitude utilizing the same function and contexts. Which allows

- removing 3 contexts

- 2 function from HM code

- shorten specification text.

According to our observation suggested EO&BO coding unification doesn’t change performance (0.00% in terms of BD-rate).

# Memory reduction

In current design in the worst case (4K, 16x16 LCU, 10 bits per sample) we have:

For offset storage 6(bits)  × 4 × 3 × 256 = 2304 bytes.

For left band storage 5(bits)  × 3 × 256 = 480 bytes

the SAO Type can be represented in 3(bits)  × 3 × 256 = 288 bytes

In total 3072 bytes.

If one offset per LCU is signaled then 75% less memory reduction for offset storage can be achieved (which is 1.1 ~ 1.7 Kbytes) and roughly 4 times less bins need to be parsed for decoding offsets.

If 2 offsets per LCU are signaled then 50% less memory reduction for offset storage can be achieved (which is 0.8 ~ 1.5 Kbytes) and roughly 2 times less bins need to be parsed for decoding offsets.

With 2 offsets per LCU reduces 1152 bytes memory. Total memory size will be 1152  + 480  + 288  = 1920 bytes.

So it will reduce twice memory for offset storage and 37.5 % of memory for all SAO parameters storage.

# Simulation Results

Both one offset and two offsets per LCU concepts were implemented on top of CE1 s/w (including JCTVC-I0184 and I0199) and tested according to [4]. Table 1 reports summary of simulation results if 1 or 2 offsets per LCU are applied on top of I0184&I0199, and also report performance improvement which will be achieved by all three tools combination vs CE1 anchor (SAO interleaving mode is used). We report average BD-rate across all 8 test cases: I-main/ he10, RA--main/ he10, LD--main/ he10, LDP--main/ he10. Class F is included into averaging when Table 1 numbers were calculated (without class F performance is roughly 0.1% better).

Table 1. Test results summary compare to CE1 anchor (SAO Interleaving mode, class F is included).

| LCU size | I0184+I0199+ 1 offset per LCU vs I0184+I0199 |  |  | I0184+I0199+ 2 offsets per LCU vs I0184+I0199 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y, % | U, % | V, % | Y, % | U, % | V, % |
| 64x64 | 0.1 | -0.1 | -0.1 | 0.0 | -0.2 | -0.1 |
| 32x32 | -0.2 | -0.6 | -0.4 | -0.1 | -0.5 | -0.3 |
| 16x16 | -0.3 | -0.1 | 0.0 | -0.2 | -0.1 | 0.0 |

| LCU size | I0184+I0199+ 2 offsets per LCU |  |  |
| --- | --- | --- | --- |
|  | Y, % | U, % | V, % |
| 64x64 | -0.3 | -0.7 | -0.6 |
| 32x32 | -0.7 | -1.1 | -1.0 |
| 16x16 | -1.4 | -1.1 | -1.1 |

More details about test results can be found in appendix (Tables 2.x) and excel spreadsheet attached to this contribution. Run-time measurement is not accurate here. But suggested changes should not have an effect on run-time.

# Specification text changes

Current proposal requires specification text changes summarized in Table 3 for 1 offset per LCU and in Table 4 if 2 offsets per LCU are signalled.

Table 3. Specification text changes (1 offset per LCU)

| #### 7.3.4.2. Sample adaptive offset CABAC syntax |  |
| --- | --- |
| sao_offset_cabac( rx, ry, cIdx ) { | Descriptor |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ]++ |  |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =4 ) { |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | u(5) |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | se(v) |
| sao_sign_flag_offset | u(1) |
| if (sao_sign_flag_offset) sao_offset[ cIdx ][ rx][ ry ] =-sao_offset[ cIdx ][ rx][ ry ] ; |  |
| } else if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ){ |  |
| sao_offset[ cIdx ][ rx][ ry ][ 0] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 1 ] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 2] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 3 ] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 2] =  sao_offset[ cIdx ][ rx][ ry ][ 2] |  |
| sao_offset[ cIdx ][ rx][ ry ][ 3] =  sao_offset[ cIdx ][ rx][ ry ][ 3] |  |
| } |  |
| } |  |

Table 4. Specification text changes (2 offsets per LCU)

| #### 7.3.4.2. Sample adaptive offset CABAC syntax |  |
| --- | --- |
| sao_offset_cabac( rx, ry, cIdx ) { | Descriptor |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 0] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 1] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =4 ) { |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | u(5) |
| for( i = 0; i < 2; i++ ) { |  |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | se(v) |
| sao_sign_flag_offset | u(1) |
| if (sao_sign_flag_offset) sao_offset[ cIdx ][ rx][ ry ] =-sao_offset[ cIdx ][ rx][ ry ] ; } |  |
| } else if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ){ |  |
| sao_offset[ cIdx ][ rx][ ry ][ 0] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 1] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 2] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 3 ] | ae(v) |
| sao_offset[ cIdx ][ rx][ ry ][ 2] =  sao_offset[ cIdx ][ rx][ ry ][ 1] |  |
| sao_offset[ cIdx ][ rx][ ry ][ 3] =  sao_offset[ cIdx ][ rx][ ry ][ 0] |  |
| } |  |
| } |  |

# Subjective quality test

We have checked visual quality difference between CE1 anchor and proposed modification, using list of cases suggested by Panasonic for CE1 Test 3.x. visual quality illustration:

- BasketBallDrill, LD(main), LCU size 64, QP=37, frame 17;

- SlideEditing, LD(main), LCU size 64, QP=37, frame 31;

- Kimono, LD(main), LCU size 64, QP=37, frame 209.

Results of our study are shown on Figures 1-3.

Fig. 1. BasketBallDrill, LD(main), LCU size 64, QP=37, frame 17: CE1 anchor (left), Proposed (right).

Fig. 2. SlideEditing, LD(main), LCU size 64, QP=37, frame 31: SAO off (left), CE1 anchor (middle), Proposed (right).

Fig. 3. Kimono, LD(main), LCU size 64, QP=37, frame 159: CE1 anchor (left), Proposed (right).

There are some non-critical block artifacts on LCU boundary for CE1 anchor. In all cases this block artifact is not visible if proposed modification is applied.

For Slide Editing (Fig.2) we additionally show SAO off case. This is well known example of ringing artifacts removal by SAO. Even only 1 EO or BO is used per LCU ringing artifacts removing functionality of SAO is preserved.

Visual quality of 1 offset per LCU solution was investigated during this meeting and no critical issues were found.

# Conclusions

Based on reported test results

no drop if LCU size is 64x64

0.2% BD rate reduction if LCU size is 32x32;

0.3% BD rate reduction if LCU size is 16x16;

Samsung, Qualcomm, and MediaTek propose to reduce both number of edge and band offsets to 2 per LCU. This reduces ~3000 bytes memory for offsets storage and helps throughput since less number of bins needed to be parsed.

# Patent rights declarations

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- E. Alshina, A. Alshin, J.H. Park (Samsung), Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei (MediaTek), I. S. Chong, M. Karczewicz(Qualcomm), “CE1, Test2.2: Reduced number of edge offsets per LCU,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0160, April 2012.

- C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), “Non-CE8: Offset coding in SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-G222, November, 2011

- E. Alshina, A. Alshin, J.H. Park (Samsung), Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei (MediaTek), I. S. Chong, M. Karczewicz(Qualcomm), “Non-CE8: Sample adaptive offset with constrained ranges of edge offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0114, February 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012

# Appendix: Test results

Table 3.1. Test results for 2 EOs&BOs per LCU. Anchor is CE1 s/w. LCU size 16x16.

Table 2.1. Test results for 2 EOs&BOs per LCU. Anchor is CE1 s/w. LCU size 16x16.

Table 2.1. Test results for 2 EOs&BOs per LCU. Anchor is CE1 s/w. LCU size 16x16.