| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0163 |
| --- | --- |

| Title: | A joint JND model based on luminance and frequency masking for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaeil Kim, Munchurl Kim Korea Advanced Institute of Science and Technology (KAIST) #291 Daehak-ro, Yuseong-gu, Daejeon, 305-732, Korea Jungsun Kim, Sangoh Jeong, Byeongmoon Jeon LG Electronics #19 Yangjae-daero 11gil, Seocho-gu Seoul, 137-130, Korea | Tel: Email: | +82-42-350-7519 jaeil1203@kaist.ac.kr sangoh.jeong@lge.com |
| Source: | KAIST, LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, a joint JND model based on luminance and frequency masking is proposed and integrated into the quantization tool in HM 6.1. The proposed joint JND model combines a luminance JND model and the quantization matrix of HM 6.1. The proposed joint JND model yields the average 6% and maximum 30% reductions in bitrates at similar subjective quality levels of HM 6.1.

# Introduction

In the 8th JCTVC meeting, a Just Noticeable Distortion (JND) based quantization method was proposed [1]. The proposed JND based quantization method relies on the luminance masking JND that is used to modulate the quantization step sizes in the HEVC quantization for each Transform Unit (TU) as follows:

QstepJND=Qstep⋅JNDLμ                           (1)

Here, JNDLμ is a luminance masking threshold for the average pixel value μ of a prediction for the current TU block and given by

JNDLμ=−μJNDA−1/μB+Aμ≤μB                    1μB<μ<μCμ−μCJNDD−1/2bit−depth−1−μC+1μ≥μC         (2)

where [1] has used the following parameter values:

JNDA = 4, µB = 62, µC = 115, JNDD = 4

However, this contribution considers both the luminance masking JND model [1] and the frequency masking JND model with the default frequency weighting matrices [2] of HM 6.1 into a combined JND model to reflect the human perception characteristics of both luminance and frequency.

# Proposed Methods

In this proposal, a proposed joint JND model considers both luminance and frequency masking effects into quantization process in HM 6.1. The quantization step size (Qstep) of HM 6.1 is modified by the proposed joint JND model as follows:

QstepJNDi,j=Qstepi,j⋅JND(i,j)                           (3)

where JND(i,j) indicates (i,j)-th quantization weighting factor for a current TU block by the proposed JND model, which is given by JNDi,j=JNDLμQMi,j. Here, JNDLμ is a luminance masking threshold for the average pixel valueμof a prediction for the current TU block, and QMi,j is (i,j)-th quantization weighting factor of HM 6.1. In the proposal, the parameters JNDA, μB, μC, JNDD of luminance JND in (2) are set to 3, 82, 95 and 3, respectively.

# Results

The proposed joint JND model is implemented on the top of HM6.1 with the default frequency weighting matrices (ScalingList = 1). The experimental results are shown in Table 1. Notice that, since the joint JND models have aims at reducing bitrates without visual perception in visual quality degradation even with the PSNR drops, the true PNSR drops and rate reductions, the PSNR and Bitrate, are measured in Table 1 instead of using the BD-PSNR and BD-Rate metrics.

Table 1 Performance of the proposed joint JND model for quantization

| Configuration | Sequence | QP | PSNR-Y[dB] | Bitrate[%] |
| --- | --- | --- | --- | --- |
| ra_main | SteamLocomotiveTrain | 22 | -0.202 | -9.529 |
| ra_main | SteamLocomotiveTrain | 27 | -0.112 | -7.038 |
| ra_main | SteamLocomotiveTrain | 32 | -0.111 | -4.256 |
| ra_main | SteamLocomotiveTrain | 37 | -0.059 | -1.335 |
| ra_main | Traffic | 22 | -0.428 | -9.773 |
| ra_main | Traffic | 27 | -0.252 | -5.589 |
| ra_main | Traffic | 32 | -0.151 | -3.046 |
| ra_main | Traffic | 37 | -0.094 | -1.840 |
| ra_main | BQTerrace | 22 | -0.954 | -30.612 |
| ra_main | BQTerrace | 27 | -0.301 | -23.468 |
| ra_main | BQTerrace | 32 | -0.270 | -9.468 |
| ra_main | BQTerrace | 37 | -0.182 | -4.241 |
| ra_main | Cactus | 22 | -0.336 | -18.622 |
| ra_main | Cactus | 27 | -0.220 | -6.079 |
| ra_main | Cactus | 32 | -0.154 | -2.689 |
| ra_main | Cactus | 37 | -0.103 | -1.643 |
| ra_main | BasketballDrive | 22 | -0.283 | -16.585 |
| ra_main | BasketballDrive | 27 | -0.136 | -4.801 |
| ra_main | BasketballDrive | 32 | -0.103 | -2.121 |
| ra_main | BasketballDrive | 37 | -0.080 | -1.118 |
| ra_main | BasketballDrill | 22 | -0.663 | -7.039 |
| ra_main | BasketballDrill | 27 | -0.418 | -4.512 |
| ra_main | BasketballDrill | 32 | -0.239 | -2.623 |
| ra_main | BasketballDrill | 37 | -0.132 | -1.476 |
| ra_main | RaceHorsesC | 22 | -0.773 | -10.370 |
| ra_main | RaceHorsesC | 27 | -0.400 | -6.037 |
| ra_main | RaceHorsesC | 32 | -0.249 | -4.518 |
| ra_main | RaceHorsesC | 37 | -0.150 | -3.286 |

In order to see the subjective qualities for the reconstructed frames by the proposed joint JND model, eight cropped frames with complex regions are shown below for four different sequences encoded at two different QP values (QP=22, QP=27). It can be noticed that it is hard to distinguish two frame patches in each pair between the original HM 6.1 and an extended HM 6.1 with the proposed joint JND model even under PSNR-Y drops of 0.954dB and 0.301dB for QP22 and QP27, respectively.

Original HM 6.1 RA Main (BQTerrace, QP22, 30th frame)

Extended HM 6.1 RA Main with the proposed joint JND Model (BQTerrace, QP22, 30th frame):

Bitrate = -30.612%, PSNR-Y = -0.954dB

Original HM 6.1 RA Main (BQTerrace QP27, 30th frame)

Extended HM 6.1 RA Main with the proposed joint JND Model (BQTerrace QP27, 30th frame)

Bitrate = -23.468 %, PSNR-Y = -0.301

Original HM 6.1 RA Main (BasketballDrive QP22, 200th frame)

Extended HM 6.1 RA Main with the proposed joint JND Model (BasketballDrive QP22, 200th frame)

Bitrate = -16.585 %, PSNR-Y = -0.283

Original HM 6.1 RA Main (Cactus QP22, 100th frame)

Extended HM 6.1 RA Main with the proposed joint JND Model (Cactus QP22, 100th frame)

Bitrate = -18.622 %, PSNR-Y = -0.336

# Conclusion

The proposed joint JND model is well harmonized with the luminance making JND and the default frequency weighting matrices of HM 6.1 to effectively reduce the bitrates in human perception perspective. Since the joint JND based quantization is very effective for bitrate reduction of redundant information for HVS, it is recommended that the joint JND based quantization be adopted in HM for further development.

# References

- M. Naccari, M. Mrak, “On Just Noticeable Distortion Quantization in the HEVC Codec”, JCTVC-H0477, 8th meeting, San José, CA, USA, 1–10 February, 2012.

- M. Haque, A. Tabatabai and Y. Morigami, “HVS model based default quantization matrices”, JCTVC-G880, 7th meeting, Geneva, Switzerland, November 2011.

- F. Bossen, “Common HM test conditions”, JCTVC-H1100, 8th JCT-VC Meeting, San Jose, USA, Mar. 2012.

# Patent rights declaration(s)

KAIST and LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: changes in inverse quantization of working draft

The scaling array levelScale[·] is specified as levelScale[k] = { 40, 45, 51, 57, 64, 72 } with k=0..5.

The variable shiftScale is set equal to shift + 4 – QP/6.

The elements of array M[ i ][ j ] with i = 0..nW − 1, j = 0..nH − 1 are set equal to ScalingFactor[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ], where SizeID and RefMatrixID are specified in Error: Reference source not found and Equation Error: Reference source not found, respectively, and trafoType is derived by

trafoType = ( ( nW = = nH ) ? 0 : ( ( nW > nH ) ? 1 : 2 ) )	(8-1)

The variable JND() is a luminance masking threshold for the average pixel valueμof a prediction for the current k-th TU block and is derived as follows.

–	If μis less than to 82,

JNDμ=−μ/41+3

–	otherwise, if μis less than to 95,

JNDμ=1

–	otherwise,

JNDμ=μ−95×2/(1<<BitDepthY)−1−95+1

The scaled transform coefficient dij with i = 0..nW − 1, j = 0..nH − 1 is derived as follows.

–	If scaling_list_present_flag is equal to 0,

yij = Clip3( −32768, 32767, cij )		(8-2)

dij = Clip3( −32768, 32767,( ( yij * levelScale[ qP%6 ] << ( qP/6 ) ) + ( 1 << ( shift − 1 ) ) ) >> shift )	(8-3)

–	Otherwise, if shiftScale is greater than 0,

yij = Clip3( −32768, 32767, cij )		(8-4)

dij = Clip3( −32768, 32767, ( yij * M[ i ][ j ]*levelScale[ qP%6 ] * JND() +

( 1 << ( shiftScale − 1 ) ) ) >> shiftScale )	(8-5)					( 1 << ( shiftScale − 1 ) ) ) >> shiftScale )	(8-6)

–	Otherwise,

yij = Clip3( -LevelLimit, LevelLimit − 1, cij )		(8-7)

dij = Clip3( −32768, 32767, ( yij * M[ i ][ j ]*levelScale[ qP%6 ] * JND() ) << ( -shiftScale ) ) 	   (8-8)