| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0165 |
| --- | --- |

| Title: | On Signaling of Several Syntax Elements in Slice Header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *Jicheng An, *Xun Guo, #Chih-Wei Hsu, #Yu-Wen Huang, #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, xun.guo, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, some conditions and constrains are proposed to be added on the signaling of two syntax elements in slice header, aps_id and inherit_dbl_params_from_aps_flag. It is reported that the proposed method can save some redundant bits, as well as avoid the potential contradictory between related syntax elements.

# Introduction

In High-Efficiency Video Coding working draft version 6.0 (WD-6.0) [1], the syntax elements aps_id and inherit_dbl_params_from_aps_flag are signaled in slice header. aps_id indicates the adaptation parameter set (APS) ID, and inherit_dbl_params_from_aps_flag indicates if the deblocking parameters from APS should be used for the current slice. There are some conditions when signaling the two flags as shown in Table 1, however, these conditions are not so sufficient that some redundant bits may be sent and contradictory case may occur.

Table 1 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| …… |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_interleaving_flag | u(1) |
| slice_sample_adaptive_offset_flag | u(1) |
| if( slice_sao_interleaving_flag && slice_sample_adaptive_offset_flag ) { |  |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| } |  |
| } |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_aps_enabled_flag ¦ ¦ ( sample_adaptive_offset_enabled_flag && !slice_sao_interleaving_flag ) ¦ ¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) { |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| } |  |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_in_aps_enabled_flag ) |  |
| inherit_dbl_params_from_aps_flag | u(1) |
| if( !inherit_dbl_params_from_aps_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| …… |  |

# Problem statement and proposed methods

## Signaling of aps_id

The conditions for signaling aps_id should also consider other related syntax elements to save some redundant bits.

Firstly, if deblocking_filter_control_present_flag, which is in picture parameter set (PPS), is equal to 0, the inferred deblocking parameters shall be used according to current HEVC text, and there is no need to refer to aps_id to obtain the deblocking parameters from APS.

Secondly, if slice_sample_adaptive_offset_flag, which is in slice header, is equal to 0, the SAO shall be off for current slice and there is no need to refer to aps_id to obtain the SAO parameters.

Thirdly, if alf_coef_in_slice_flag, which is in sequence parameter set (SPS), is equal to 0, the ALF coefficients in slice header rather than in APS shall be used and there is no need to refer to aps_id to obtain the ALF coefficients.

Therefore, it is proposed to add related conditions when signaling the aps_id flag in slice header as shown in Error: Reference source not found. The added part is highlighted.

Table 2. aps_id signalling condition improvement related to deblocking, SAO and ALF

| if( scaling_list_enable_flag ¦ ¦ (deblocking_filter_in_aps_enabled_flag && deblocking_filter_control_present_flag) ¦ ¦ ( sample_adaptive_offset_enabled_flag &&! slice_sao_interleaving_flag && slice_sample_adaptive_offset_flag) ¦ ¦ (adaptive_loop_filter_enabled_flag && !alf_coef_in_slice_flag ) ) |  |
| --- | --- |
| aps_id | ue(v) |

## Signaling of inherit_dbl_aps_params_flag

For the signalling of inherit_dbl_from_aps_params_flag, there is one contradictory case when aps_deblocking_filter_flag in APS is equal to 0, and inherit_dbl_params_from_aps_flag is equal to 1. In such case, the deblocking parameters should be inherited from APS, but there is no deblocking parameters sent in APS. It is proposed to add a constraint in the semantic of inherit_dbl_params_from_aps_flag to make the two syntax elements consistent as follows.

inherit_dbl_params_from_aps_flag equal to 1 specifies that deblocking parameters from the APS shall be used for deblocking the current slice. inherit_dbl_params_from_aps_flag equal to 0 specifies that deblocking parameters in the slice header shall be used. When inherit_dbl_params_from_aps_flag is not present, it is inferred to be equal to 0. It is a requirement of the bitstream that if there is an active APS and the value of aps_deblocking_filter_flag is 0, then the value of inherit_dbl_from_aps_params_flag shall be 0.

# Conclusion

In this contribution, changes are made for two syntax elements in slice header. Some conditions are added when signaling aps_id flag in slice header so that some redundant bits are saved, and one constraint is added for inherit_dbl_params_from_aps_flag so that one contradictory case is avoided.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand (Editors), “High Efficiency Video Coding (HEVC) text specification draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1003, February, 2012.