| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, Apr.27-May 7, 2012 | Document: JCTVC-I0166 |
| --- | --- |

| Title: | Simplified Parameter Calculation for Chroma LM Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | +Juliana Hsu *Mei Guo, *Xun Guo, #Shawmin Lei +Institute of Communication Systems and Data Processing, RWTH Aachen University *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | Juliana.hsu@rwth-aachen.de mei.guo@mediatek.com xun.guo@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM6.0, when a chroma prediction unit (PU) is coded with intra LM mode, some parameters are calculated with the neighboring pixels of the current chroma PU and the ones of the corresponding luma PU. The derivation of parameters requires a 16-bit multiplication and an online generated look-up table with 63 entries. This contribution proposes two methods to simplify the complexity of this process. Firstly, the depth of the multiplication is reduced from 16 to the internal bit depth. Secondly, the entries of the look-up table are reduced from 63 to 32, and the bits for each entry are reduced from 16 bits to 10 bits. It is reported that there is almost no BD-rate and running time changes.

# Introduction

In HM6.0 [1], an intra prediction mode called LM is applied to predict chroma PU based on a linear model using the reconstruction of the collocated luma PU. The parameters of the linear model consist of slope (a>>k) and y-intercept (b), which are derived from the neighboring luma and chroma pixels using the least mean square solution. The values of the prediction samples predSamples[x,y], with x,y = 0…nS-1, where nS specifies the block size of the current chroma PU, are derived as follows:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> k ) + b ), with x, y = 0..nS−1,				(1)

where PY’[ x,y ] is the reconstructed pixels from the corresponding luma component. When the coordinates x and y are equal to or larger than 0, PY’ is the reconstructed pixel from the co-located luma PU. When x or y is less than 0, PY’ is the reconstructed neighboring pixel of the co-located luma PU.

Some intermediate variables in the derivation process, L, C, LL, LC, k2 and k3, are derived as:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3				(2)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3				(3)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3				(4)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3			(5)

k2 = Log2( (2*nS) >> k3 )				(6)

k3 = Max( 0, BitDepthC + Log2( nS ) − 14 ) .			(7)

Therefore, variables a, b and k can be derived as:

a1 	= ( LC << k2 ) – L*C				(8)
a2 	= ( LL << k2 ) – L*L				(9)
k1 	= Max( 0, Log2( abs( a2 ) ) − 5 ) – Max( 0, Log2( abs( a1 ) ) − 14 ) + 2			(10)
a1s	= a1 >> Max(0, Log2( abs( a1 ) ) − 14 )				(11)
a2s	= abs( a2 >> Max(0, Log2( abs( a2 ) ) − 5 ) )			(12)
a3	= a2s < 1 ? 0 : Clip3( −215, 215−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 )			(13)
a	= a3 >> Max( 0, Log2( abs( a3 ) ) − 6 )				(14)
k	= 13 – Max( 0, Log2( abs( a ) ) − 6 )				(15)
b	= ( L – ( ( a*C ) >> k1 ) + ( 1 << ( k2 − 1 ) ) ) >> k2,			(16)

where lmDiv is specified in a 63-entry look-up table, i.e. Table 1, which is online generated by

lmDiv(a2s)=( (1 << 15) + a2s/2 ) / a2s .			(17)

Table 1 Specification of lmDiv

| a2s | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lmDiv | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 | 2521 |
| a2s | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| lmDiv | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 | 1260 |
| a2s | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| lmDiv | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 | 840 |
| a2s | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 |
| lmDiv | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 | 630 |
| a2s | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |  |
| lmDiv | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 | 512 |  |

In Equation (13), a1s is a 16-bit signed integer and lmDiv is a 16-bit unsigned integer. Therefore, 16-bit multiplier and 16-bit storage are needed. In this contribution, we propose to reduce the bit depth of multipliers to the internal bit depth, as well as the size of the look-up table.

# Algorithm Description

## Reduced bit depth of multipliers

The bit depth of a1s is reduced to the internal bit depth by changing Equation (11) as

a1s	= a1 >> Max(0, Log2( abs( a1 ) ) – (BitDepthC – 2))	.						(18)

The values of lmDiv with the internal bit depth are achieved with

lmDiv(a2s)=( (1 << (BitDepthC-1)) + a2s/2 ) / a2s								(19)

and stored in the look-up table. Table 2 shows the example of internal bit depth 10.

Table 2 Specification of lmDiv with the internal bit depth equal to 10

| a2s | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lmDiv | 512 | 256 | 171 | 128 | 102 | 85 | 73 | 64 | 57 | 51 | 47 | 43 | 39 | 37 | 34 | 32 |
| a2s | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| lmDiv | 30 | 28 | 27 | 26 | 24 | 23 | 22 | 21 | 20 | 20 | 19 | 18 | 18 | 17 | 17 | 16 |
| a2s | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 |
| lmDiv | 16 | 15 | 15 | 14 | 14 | 13 | 13 | 13 | 12 | 12 | 12 | 12 | 11 | 11 | 11 | 11 |
| a2s | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |  |
| lmDiv | 10 | 10 | 10 | 10 | 10 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 8 | 8 | 8 |  |

Modifications are also made to Equation (10) and (15) as below

k1 	= Max( 0, Log2( abs( a2 ) ) − 5 ) – Max( 0, Log2( abs( a1 ) ) – (BitDepthC – 2) ), and		(20)
k	= BitDepthC – 1 – Max( 0, Log2( abs( a ) ) − 6 ).							(21)

## Reduced entries of the look-up table

The proposed method reduces the entries from 63 to 32, and the bits for each entry from 16 to 10, as shown in Table 3. By doing this, almost 70% memory saving can be achieved. The corresponding changes for Equation (13), Equation (17) and Equation (15) are as follows.

a3	= a2s < 32 ? 0 : Clip3( −215, 215−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 )                  (22)

lmDiv(a2s)=( (1 << (BitDepthC+4)) + a2s/2 ) / a2s								(23)

k= BitDepthC + 4 – Max( 0, Log2( abs( a ) ) − 6 ).								(24)

Table 3 Specification of lmDiv with the internal bit depth equal to 10

| a2s | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lmDiv | 512 | 496 | 482 | 468 | 455 | 443 | 431 | 420 | 410 | 400 | 390 | 381 | 372 | 364 | 356 | 349 |
| a2s | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| lmDiv | 341 | 334 | 328 | 321 | 315 | 309 | 303 | 298 | 293 | 287 | 282 | 278 | 273 | 269 | 264 | 260 |

# Simulation results

The simulations are based on HM6.0 and the recommended test conditions in JCTVC-H1100 [2]. The proposed methods are tested in All Intra HE10 configuration, because chroma LM mode is not included in main profile. The results of look-up tables in Table 2 and Table 3 are shown in Table 4 and Table 5, respectively. Almost no loss in coding performance is observed.

Table 4 Simulation results of Table 2

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.42% | -0.45% |
| Class B | 0.00% | -0.01% | -0.02% |
| Class C | 0.00% | -0.01% | 0.05% |
| Class D | 0.00% | -0.02% | 0.04% |
| Class E | 0.00% | -0.05% | 0.05% |
| Overall | 0.00% | -0.10% | -0.07% |
|  | 0.00% | -0.09% | -0.07% |
| Class F | 0.00% | -0.02% | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 5 Simulation results of Table 3

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.06% | 0.00% |
| Class B | 0.00% | -0.05% | -0.02% |
| Class C | 0.00% | 0.02% | 0.00% |
| Class D | 0.00% | -0.02% | 0.04% |
| Class E | 0.00% | 0.02% | 0.02% |
| Overall | 0.00% | -0.02% | 0.01% |
|  | 0.00% | -0.02% | 0.00% |
| Class F | 0.00% | -0.06% | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

This contribution proposed two methods to reduce the complexity of the parameter derivation for chroma LM mode. Firstly, bit depth of the multiplication is reduced from 16 to the internal bit depth. Secondly, the number of entries in look-up table is reduced from 63 to 32, and the bits for each entry are reduced from 16 to 10. Consequently, one 16-bit multiplication is removed from chroma LM mode, and almost 70% of the storage memory for lmDiv look-up table is saved. There is almost no change for coding performance and run time.

# References

- B. Bross, W. Han, J. Ohm, G. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1003, 8th Meeting: San José, CA, USA, 1–10 Feb., 2012.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1100, 8th Meeting: San José, CA, USA, 1–10 Feb., 2012.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Aachen University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).