| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0167 |
| --- | --- |

| Title: | CE2: Line memory reduction fix at tile boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Ann-Kathrin Seifert, Matthias Narroschke, Thomas Wedi Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 – 1306 semih.esenlik@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

During the 7th JCT-VC Meeting at Geneva JCTVC-G212 “Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9” has been adopted. According to the JCTVC-G212 a “Virtual Boundary (VB)” padding operation is applied at the bottom of each LCU in order to eliminate the line memory requirement for ALF. The current contribution is intended to improve the line memory reduction method of G212 by switching VB padding off at tile boundaries when the loop filtering across tiles is switched off. At tile boundaries there is no line memory requirement since ALF filtering operation is restricted to use samples only within a same tile. Moreover since the tile boundaries are padded for filtering already, the secondary VB padding operation is expected to introduce visual artifacts at the boundaries. Therefore switching VB padding off at tile boundaries when loop filtering across tiles is switched off improves the visual quality as well as the coding gain. The performed tests show coding gains of 0.1% for luminance and 0.2% for each chrominance component (averaged over all test configurations) in the case of multiple tiles per picture.

# Introduction

The application of loop filters is controlled at the tile boundaries with loop_filter_across_tile_flag that is signalled in the Sequence Parameter Set (SPS). According to the state of the flag, loop filters are either allowed to use samples from neighbouring tiles or not. In the case when the loop filters are not allowed to use samples from the neighbouring tile the following operations are applied at the boundaries for the three in-loop filters:

- Deblocking filter: At vertical tile boundaries horizontal deblocking of vertical edges is skipped. At horizontal tile boundaries vertical deblocking of horizontal edges is skipped.

- SAO: SAO operation is skipped at LCU border if the border coincides with a tile boundary.

- ALF: The border of the LCU is extended by repetitive padding if the border coincides with a tile boundary.

Moreover during the 7th JCT-VC Meeting at Geneva JCTVC-G212 “Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9”[1] has been adopted which eliminated the line memory requirement of ALF completely. However the line memory reduction tool is applied to all LCUs in a frame, ignoring the partitioning of the frame into tiles.

# Problem

Figure 1 describes the line memory requirement at tile boundaries. In this example the loop_filter_across_tile_flag in the SPS is set to 0 such that 3 in-loop filters do not use samples from the neighbouring tiles.

Figure 1: Line memory at independent tile boundaries

During the processing of LCU 8, which is depicted in the figure, the 3 loop filters do not require any pixels from LCU 14, which is immediately below LCU 8, due to the fact that LCU 8 and 14 belong to different tiles. In the same way, during the application of loop filters on LCU 14, pixels from LCU 8 are not required. As a result along the horizontal boundary that separates tiles 1 and 2 no line memory is required (the implemented line memory does not need to be used for storing samples).

However according to JCTVC-G212, virtual boundary padding is applied at every LCU (except for the bottom frame border), irrespective of the partitioning of the frame into tiles. Therefore although there is no line memory requirement at the tile boundary above, the VB padding operation is still applied at LCU8.

Figure 2: VB padding and tile boundary padding

Figure 2 depicts the problem in more detail, where the bottom boundary of the depicted LCU is assumed to coincide with a tile boundary. Repetitive padding needs to be applied at the bottom boundary of the LCU since the current LCU lies on a tile boundary. Moreover VB padding is applied 4 lines above the bottom LCU boundary. In other words padding operation is applied two times at the LCU boundary if the LCU lies at a tile boundary.

The application of VB padding in addition to tile boundary padding in the same LCU (where VB padding is actually not required) is expected to introduce artifacts and reduce the coding gain.

# Proposal

The solution can be described with the following rule:

“If the loop_filter_across_tile_flag is set to 0, VB padding is not applied at the horizontal tile boundaries in ALF operation.”

According to the current text specification draft [2], the VB padding is not applied at the bottom frame boundary. Therefore with the proposal the operation at bottom tile boundary is aligned with the operation at the bottom frame boundary.

The proposal eliminates the double padding at tile boundaries for the ALF process. Please note that there is no line memory requirement at horizontal tile boundaries. Therefore switching VB padding off at horizontal tile boundaries does not increase the hardware complexity.

# Results

In the simulations HM6.0 [4] has been used. The following changes have been made in the encoder configuration files in order to allow multi-tile per frame setting:

UniformSpacingIdc                     : 1

NumTileColumnsMinus1            : 1

NumTileRowsMinus1	        : 8

Additionally only picture-based ALF encoding procedure was used due to a reported bug in the LCU-based ALF encoding algorithm (It was reported by the CE2 coordinator that LCU-based ALF encoding algorithm has problems in multiple-tile operation mode).

Two different tests are performed in order to assess the proposal:

- All three loop filters are switched off across the tile boundaries by setting the loop_filter_across_tiles_enabled_flag equal to 0.

- Only Adaptive Loop Filter is switched off across the tile boundaries. Please note that the loop filters cannot be switched on or off across the tile boundaries individually. Therefore changes in the source code had to made to enable this configuration.

## Test 1: All loop filters are switched off across tile boundaries

The following table shows the simulation results for the case for the case when loop_filter_across_tiles_enabled_flag is set to 0.

Table 1: Simulation results

Fluctuations in the runtimes stem from the heterogeneous simulation environment that has been used. No change in the runtimes should be expected. For the complete simulations please refer to the attached document “JCTVC-I0167_3FiltersNonCross.xls”.

## Test 2: Only ALF is switched off across tile boundaries

The following table shows the simulation results for the case for the case when only ALF is switched off across the tile boundaries.

Table 2: Simulation results

Fluctuations in the runtimes stem from the heterogeneous simulation environment that has been used. No change in the runtimes should be expected. For the complete simulations please refer to the attached document “JCTVC-I0167_ALFNonCross.xls”.

# Conclusion

In HM6.0 the line memory reduction method (Virtual Boundary padding) of JCTVC-G212 is applied at tile boundaries, even though no line memory is required at the boundaries between tiles. The current contribution is intended to improve the line memory reduction method of G212 by switching VB padding off at tile boundaries, when the loop filters are not allowed to cross the tile boundaries. It has been shown that by switching VB padding off at the tile boundaries 0.1% coding gain for luminance component and 0.2% coding gain for chrominance components is achieved (averaged over all test configurations). Moreover since the purpose of the tool is to avoid double padding at tile boundaries, visual improvement is expected to be observed.

# References

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, S. Esenlik, M. Narroschke, T. Wedi, “Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9,” Document of Joint Collaborative Team on Video Coding, JCTVC-G212, November 2011.

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6”, Document of Joint Collaborative Team on Video Coding, JCTVC-H1003, February 2012.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

- JCTVC-HM Repository: http://hevc.kw.bbc.co.uk/trac/browser

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).