| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0168 |
| --- | --- |

| Title: | Non-CE1: Bug-fix of offset coding in SAO interleaving mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-6.0, the interleaving mode of SAO sends SAO parameters of three components in one LCU independently. The band offset (BO) is signed, while the edge offset (EO) is unsigned. It is clear that CABAC initialization, binarization, and context formation processes for BO are different from those for EO. However, in HEVC CD, both BO and EO are both described by sao_offset syntax in the syntax table of Section 7.3.4.2 and share the same CABAC processes. To fix this mismatch between HM-6.0 and CD, a straightforward solution is to change the CD according to HM-6.0, but it might be necessary to change hundreds of lines in CD. Moreover, nobody has provided the fix. In this contribution, a simpler method is proposed to fix the mismatch by changing only 8 lines in HM-6.0 and 23 lines in CD. First, the sign of BO is decoupled from sao_offset, and the signs of the four offsets in BO are grouped and bypass coded. Then, the magnitude of BO and the unsigned EO can be represented by the same syntax element, sao_offset, and use the same CABAC processes. It is reported that the average BD-rates are 0.0% in Main-LDB and HE10-LDB conditions.

# Proposed method

The proposed syntax change is shown in Table 1. First, signs of the four offsets in BO are separated from sao_offset. Then, they are grouped together and bypass coded.

Table 1.  Syntax change in offset coding

| sao_offset_cabac( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) { |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_sign [ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| } |  |

In this way, the CABAC initialization, binarization, and context formation processes can be shared for both the magnitude of BO and the unsigned EO.

# Simulation results

The proposed method is implemented on top of the CE1 anchor [1] for SAO interleaving mode. Table 1 shows the results for three different LCU sizes in Main-LDB and HE10-LDB conditions. It can be seen that the overall BD-rates are 0.0%.

Table 1.  Results of the proposed method

# Conclusions

In this contribution, the signs of the four offsets in BO are independent of the sao_offset syntax element. They are grouped together before and bypass coded before the sao_offset. Next, the magnitude of BO and the unsigned EO are both represented by sao_offset and use the same CABAC processes for initialization, binarization, and context formation. The benefit of this proposal is to easily fix the mismatch between HM-6.0 and CD for the offset coding in SAO. The straightforward fix may need to change hundreds of lines in CD, while the proposed method changes only 8 lines in HM-6.0 and 23 lines in CD. It is reported that this proposal does not cause essential coding efficiency difference.

# References

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1101, February, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).