| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0169 |
| --- | --- |

| Title: | Grid displacements for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 1306 Semih.Esenlik@eu.panasonic.com |
| Source: | Panasonic Corp. |  |  |

_____________________________

# Abstract

In the contribution a method to reduce the complexity of the LCU- (Largest Coding Unit) based decoding process for the Adaptive Loop Filter (ALF) operation is proposed via displacements in the filter grids. The contribution was first proposed at 6th JCT-VC Meeting in Geneva and was delegated to be studied in the CE. In the context of the new CE2 for the 9th JCT-VC Meeting it has been discussed again and was suggested to be very beneficial by the CE2 participants.

According to the proposal ALF filtering grid is shifted 4 pixels up in order to eliminate the ALF coefficient line buffer completely (corresponding to about 3.6k Bytes line buffer reduction) in the case of LCU-based ALF parameter syntax. Moreover the proposed method reduces the amount of filter switching that is required during the processing of an LCU.

# Introduction

# For the 9th JCT-VC Meeting 3 options for ALF have been proposed for adoption. The three approaches are summarized below:

- LCU-based ALF: The ALF filter parameters are estimated for each LCU independently and the parameters are interleaved in the LCU data. Moreover generated filters are retained in a history table of size 16 which allows reuse of filters by following LCUs in the decoding order.

- Picture-based ALF: Filter parameters are estimated jointly for all of the LCUs in a frame. The approach requires partitioning of the frame into maximum 16 equal-sized regions. A different ALF filter can be generated for each region and the region boundaries are aligned with the LCU boundaries.

- Picture- and LCU-based ALF: Combination of options 1 and 2.

The picture-based ALF approach which is defined in option 2 utilizes “Region-based filter selection” and achieves local adaptation by generating filter coefficients independently for each filtering region. The filtering regions comprise one or more LCUs and region boundaries are aligned with the LCU boundaries.

The LCU-based ALF on the other hand does not utilize grouping of LCUs into filtering regions. Instead a filter is generated independently for each LCU or previously generated filters are reused in order to reduce signaling overhead. The LCU-based ALF approach is advocated in option 1.

# Problem

In the LCU-based hardware decoding Deblocking Filter (DF), SAO and ALF are applied one after the other on a current LCU and the output is written to the external frame buffer. The processing of the LCU boundaries requires samples from neighboring LCUs since the filter taps of DF and ALF extend beyond the LCU boundaries. Therefore a special type of on-chip memory which is called “line memory” has to be implemented for the processing of LCU boundaries.

In the following Figure 1, application of ALF by the decoder on the current LCU is shown. The ALF parameters are estimated in an LCU aligned fashion in the encoder (One filter for each LCU). However due to the preceding deblocking and SAO operations, “ALF application window” in the decoder is shifted by 4 pixels to the top and 9 pixels to the left during the LCU-based hardware decoding process.

Figure 1: Shifted ALF application window in LCU-based filtering

In Figure 1, the 4 lines at the bottom LCU border are not yet processed by the Deblocking filter and SAO (shaded region), hence inaccessible to the ALF operation. On the right LCU border 9 lines are not accessible to ALF operation since Deblocking and SAO processes are not yet performed and due to the horizontal length of the ALF filter mask. As a result the ALF application window (shown in red broken lines), which describes the region where ALF operation is going to be applied at the current instant, is shifted by 4 lines up and 9 lines to the left accordingly.

The figure below illustrates the LCU-based decoding process. The ALF application window in the decoder intersects 4 different LCUs where a different filter can be designed for each of the LCUs. As a result the ALF application window (red square) is divided into 4 sub-regions and different ALF filters need to be applied in each sub-region.

Figure 2: For the filtering of an LCU, 4 different filters need to be buffered.

## Filter coefficient line buffer

The LCU-based ALF approach that is advocated in option 1 necessitates implementation of an ALF filter coefficient line buffer. According to Figure 2, the ALF filters (filter coefficients) that are used in the upper row of LCUs (LCU1 and LCU2) need to be stored in the line memory, since they are required during the decoding of LCU4. Moreover since 3 different filters can be generated for each of the 3 color components, 3 filters need to be stored in the line memory corresponding to each LCU along the whole picture width.

Therefore the size of the coefficient line buffer is given by the following formula:

Picture WidthSmallest LCU Widthx10Filter Coefficients x 3

For the picture size of 1920x1080 and LCU size of 16x16 the coefficient line memory corresponds to 3.6k Bytes.

## Frequent filter switching

As illustrated in Figure 2 the ALF application window is divided into 4 sub regions and a different filter is applied to each sub region. As a result

- 4 different filters are needed to be buffered for the filtering of an LCU in the LCU-based hardware decoding process.

- Frequent filter switching occurs at sub-region boundaries.

# Proposed Method

In HM6.0 [2] the ALF process in the decoder is applied in a delayed fashion on a window which is not aligned with the LCU (ALF application window shown in Figure 1and Figure 2). Since the deblocking and SAO processes cannot be applied at the 4 lines of the bottom LCU boundary, the ALF operation is also delayed and cannot be applied at the 4 lines above the bottom LCU boundary. Due to this delay in deblocking operation, a coefficient line buffer needs to be implemented for ALF which has an approximate size of 3.6kBytes.

According to the proposal ALF filter grid is shifted 4 pixels up and therefore no longer aligned with the top and bottom LCU boundaries. The following figures (Figure 3 and Figure 4) show the proposed modification and resulting ALF filter grid respectively.

Figure 3: Proposed modification for the ALF filter grid.

Figure 4: Picture frame that is composed of 20 LCUs. The proposed ALF grid is depicted by red dashed lines and the LCU boundaries are depicted by solid black lines.

The proposed filter grid displacement for ALF results in the following benefits:

- Filter coefficient line buffer is completely eliminated for LCU-based ALF approach that is advocated in option 1. In other words the filter coefficients that are used in the upper row of LCU are not required to be stored in the line memory. The saving in line memory corresponds to 3.6k Bytes for 1920x1080 picture resolution.

- Uniform filtering within a filter application window in the LCU-based hardware decoder is achieved. Filter switching within an LCU is reduced for LCU-based hardware decoding. Moreover only 2 filters need to be buffered for the decoding of an LCU instead of 4.

## Processing at slice/tile boundaries

At tile and slice boundaries the filter application window in the decoder shrinks and expands according to the position of the LCU that is currently processed if the in-loop filtering is switched off across the slice or tile boundaries. According to the proposed method ALF grid is also shrinks or expands at slice or tile boundaries. This is illustrated in Figure 4 where the filter grid expands at bottom frame boundaries and shrinks at top boundaries. If the loop filters are not allowed to cross the slice or tile boundaries, the filter grid expands or shrinks at slice and tile boundaries as in the case of frame boundaries.

# Experimental Results

As the reference, the official tagged 6.0 version of the HM software [1] is used. The simulations are performed according to [3]. Table 1 shows the experimental results obtained for the proposal.

Table 1: Experimental results

The obtained results show that the change in the coding gain is negligible. Moreover there is only a very slight variation in the runtimes which is the result of the heterogeneous simulation environment that has been used.

# Conclusion

A method to reduce the complexity of the LCU-based filtering operation and to eliminate the coefficient line buffer proposed. According to the proposal ALF filtering grid is shifted 4 pixels up in order to eliminate the ALF coefficient line buffer completely in the case of LCU-based ALF parameter syntax. Moreover the proposed method reduces the amount of filter switching that is required during the processing of an LCU. The contribution is proposed to be adopted into the Standard Text.

# References

- HM 6.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.0

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, High Efficiency Video Coding (HEVC) text specification draft 6, JCTVC-H1003, San Jose, USA, January 2012.

- F. Bossen, “Common test conditions”, JCTVC-H1100, San Jose, January 2012.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).