| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0170 |
| --- | --- |

| Title: | Parsing robustness issue for deblocking filter syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastrasse 4c,63225, Langen, Germany | Tel: Email: | 0049 6103 766-1306 semih.esenlik@eu.panasonic.com |
| Source: | Panasonic Corp. |  |  |

_____________________________

# Abstract

According to the current HM6.0 the deblocking control parameters can be signalled either in the Adaptation Parameter Set (APS) syntax structure or in the slice header. It is noted in the proposal that when the deblocking control parameters are sent in the APS, the parsing of the syntax element slice_loop_filter_across_slices_enabled_flag in the slice header depends on the DB control parameters that are sent in the APS. This results in a parsing issue for the slice data, since the APS is expected to be subject to packet losses in the communication network. The contribution provides a solution for the parsing problem with a small change in the slice header syntax. With the help of the proposal the slice data can be parsed independently from the APS.

# Introduction

During the 6th JCT-VC meeting APS syntax structure has been adopted into the HEVC. The APS is designed to decouple the crucial parsing parameters that are carried in Sequence and Picture Parameter Sets from the adaptive coding parameters that have less impact on the fidelity of the reconstructed picture.

According to [1] the control parameters for Deblocking (DB) filtering can be signalled either in the Adaptation Parameter Set (APS) or in the slice header depending on the value of inherit_dbl_params_from_aps_flag. The possibility of signaling DB parameters in the APS provides a cost effective (in terms of bitrate) way of controlling the DB filtering operation for each individual picture frame.

# Problem Statement

Table 1 and Table 2 below present the slice header and APS syntax structures according to [1]. For simplicity only the parts that are related to the deblocking filter control parameters are presented.

| 1 | slice_header( ) { | Descriptor |
| --- | --- | --- |
| 2 | … |  |
| 3 | if( !entropy_slice_flag ) { |  |
| 4 | slice_qp_delta | se(v) |
| 5 | if( deblocking_filter_control_present_flag ) { |  |
| 6 | if( deblocking_filter_in_aps_enabled_flag ) |  |
| 7 | inherit_dbl_params_from_aps_flag | u(1) |
| 8 | if( !inherit_dbl_params_from_aps_flag ) { |  |
| 9 | disable_deblocking_filter_flag | u(1) |
| 10 | if( !disable_deblocking_filter_flag ) { |  |
| 11 | beta_offset_div2 | se(v) |
| 12 | tc_offset_div2 | se(v) |
| 13 | } |  |
| 14 | } |  |
| 15 | } |  |
| 16 | } |  |
| 17 | … |  |
| 18 | if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| 19 | slice_loop_filter_across_slices_enabled_flag | u(1) |
| 20 | … |  |
| 21 | } |  |

Table 1: Slice header syntax structure

| 1 | aps_rbsp( ) { | Descriptor |
| --- | --- | --- |
| 2 | aps_id | ue(v) |
| 3 | … |  |
| 4 | aps_deblocking_filter_flag | u(1) |
| 5 | if(aps_deblocking_filter_flag) { |  |
| 6 | disable_deblocking_filter_flag | u(1) |
| 7 | if( !disable_deblocking_filter_flag ) { |  |
| 8 | beta_offset_div2 | se(v) |
| 9 | tc_offset_div2 | se(v) |
| 10 | } |  |
| 11 | } |  |
| 12 | … |  |
| 13 | } |  |

Table 2: APS syntax structure

The syntax element disable_deblocking_filter_flag is signaled at either one of the two different places, i.e. at the 9th line in the slice header or at the 6th line in the APS, depending on the value of inherit_dbl_params_from_aps_flag syntax element. The signaling scheme is summarized as follows:

- If the value of inherit_dbl_params_from_aps_flag is equal to 0, then disable_deblocking_filter_flag is coded at the 9th line in the slice header.

- If the value of inherit_dbl_params_from_aps_flag is equal to 1, then disable_deblocking_filter_flag is coded at the 6th line in the APS.

Further down in the slice header, slice_loop_filter_across_slices_enabled_flag (line 19 in slice header) is coded dependent on the value of disable_deblocking_filter_flag. In other words the parsing operation that is described at lines 18 and 19 of the slice header depends on information that is signaled in the APS. As a result in the case of the loss of an APS, the whole slice data beginning at line 19 of the slice header is rendered “not parsable”.

It has been established in the previous JCT-VC meetings that the APS (unlike PPS or SPS) is prone to packet losses in the network. Therefore it is highly desirable that no parsing dependency exists (in either direction, i.e. from slice header to APS or from APS to slice header) between the APS and slice header.

# Proposed syntax changes

Table 3 below presents the proposed syntax changes. The parsing condition at line 18 is modified in such a way that the presence of slice_loop_filter_across_slices_enabled_flag syntax element does not depend on any information that is signaled in the APS. As a result parsing dependency between APS and slice header is achieved.

| 1 | slice_header( ) { | Descriptor |
| --- | --- | --- |
| 2 | … |  |
| 3 | if( !entropy_slice_flag ) { |  |
| 4 | slice_qp_delta | se(v) |
| 5 | if( deblocking_filter_control_present_flag ) { |  |
| 6 | if( deblocking_filter_in_aps_enabled_flag ) |  |
| 7 | inherit_dbl_params_from_aps_flag | u(1) |
| 8 | if( !inherit_dbl_params_from_aps_flag ) { |  |
| 9 | disable_deblocking_filter_flag | u(1) |
| 10 | if( !disable_deblocking_filter_flag ) { |  |
| 11 | beta_offset_div2 | se(v) |
| 12 | tc_offset_div2 | se(v) |
| 13 | } |  |
| 14 | } |  |
| 15 | } |  |
| 16 | } |  |
| 17 | … |  |
| 18 | if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ¦ ¦ inherit_dbl_params_from_aps_flag) ) |  |
| 19 | slice_loop_filter_across_slices_enabled_flag | u(1) |
| 20 | … |  |
| 21 | } |  |

Table 3: Proposed changes in the slice header syntax structure

With the help of the proposed change no parsing dependency exists between the APS and the slice header. This condition guarantees that losses in APS packets do not cause catastrophic parsing errors.

# Proposed semantic changes

As an additional refinement, the following change (Table 4) in the semantic of disable_deblocking_filter_flag is proposed. The proposed change guarantees that default value of disable_deblocking_filter_flag is specified in text specification draft.  The absence of the default value is considered a bug by the authors.

7.4.3 Slice header semantics disable_deblocking_filter_flag equal to 1 specifies that the operation of the deblocking filter shall be disabled and disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be enabled. When disable_deblocking_filter_flag is not present, it is inferred to be equal to 0.

Table 4: Proposed semantic changes

# Conclusion

A change in the slice header syntax is proposed in order to remove the parsing dependency between the slice header and the APS. With the help of the proposed change no parsing dependency exists between the APS and the slice header. This condition guarantees that losses in APS packets do not cause catastrophic parsing errors.

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6”, Document of Joint Collaborative Team on Video Coding, JCTVC-H1003, February 2012.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).