| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0172 |
| --- | --- |

| Title: | Non-CE1: Prohibition against merging across slice/tile boundaries in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, chiayang.tsai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-6.0, the interleaving mode of SAO uses merge syntax, which allows each LCU to reuse SAO parameters of the neighboring LCU to the left or to the top. In this contribution, it is proposed to disallow merging across a slice or tile boundary.

# Proposed method

In order to disallow copying SAO parameters from a different slice or tile, the proposed syntax change is shown in Table 1. First, two conditions are added to check if the current coding tree block and its left and above coding tree blocks are in the same tile. If the neighbor is a different tile, the corresponding merge syntax is not present and set to zero. Second, we remove slice_loop_filter_across_slices_enabled_flag from the condition in sao_cabac_synatx, which will make sao_merge_up_flag not present and set to zero if the current coding tree block and its above coding tree block is not in the same slice.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Syntax change to avoid merging from a different slice or tile

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| AddrUp= CtbAddrRS – PicWidthInCtbs |  |
| If (TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ]) |  |
| xCtb = -1 |  |
| If (TileId[ CtbAddrTS ] != TileId[ CtbAddrRStoTS [CtbAddrRS – PicWidthInCtbs] ]) |  |
| yCtb = -1 |  |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| if(sao_cb_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| … |  |
| } while( moreDataFlag ) |  |
| } |  |

| sao_unit_cabac( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 & & CtbAddrInSlice != 0 ) |  |
| sao_merge_left_flag | ae(v) |
| if( !sao_merge_left_flag ) |  |
| if( ry > 0 & & ( AddrUp > 0 ¦ ¦ slice_loop_filter_across_slices_enabled_flag ) ) |  |
| sao_merge_up_flag | ae(v) |
| if( !sao_merge_up_flag ) |  |
| sao_offset_cabac( rx, ry, cIdx ) |  |
| } |  |