| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0173 |
| --- | --- |

| Title: | CE2: Results of CE2.b.1, CE2.g.1, and CE2.h.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCTVC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In CE2.b.1, run prediction is removed for APS mode (picture-based optimization). It is reported that CE2.b.1 has no coding efficiency loss in the eight common test conditions (Main-AI/RA/LDB/LDP and HE10-AI/RA/LDB/LDP).

In CE2.g.1, the bottom four luma pixel rows, the bottom two chroma pixel rows, the rightmost five luma pixel columns, and the rightmost three chroma pixel columns in each LCU are not used during ALF coefficient derivation for slice header mode (LCU-based optimization). It is reported that CE2.g.1 causes 0.1-0.3% luma losses and 0.4-1.3% chroma losses in the eight common test conditions in comparison with only avoiding the bottom pixel rows.

In CE2.h.1, reusing luma filter coefficients for chroma filtering in one LCU is allowed for slice header mode (LCU-based optimization). It is reported that CE2.h.1 suffers 0.1-0.2% luma losses and achieves 0.6-3.3% chroma gains in the eight common test conditions.

# Introduction

In the 8th JCT-VC meeting, ALF with LCU-based syntax [1] was adopted in HEVC CD [2] and HM-6.0 [3]. In addition, CE2 [4] was established not only to evaluate ALF coding tools but also to study reasonable constraints of LCU-based encoding algorithms in a practical LCU-based encoding system. Totally, there are 9 subtests including 15 experiments in CE2, and three experiments will be reported in this contribution. The rest of this contribution is organized as follows. The experiments will be introduced in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

## CE2.b.1: Remove run prediction for APS mode

In HM-6.0, ALF uses run coding instead of merge-left flag. A run value of a current LCU represents the number of following LCUs that will use the ALF parameters of the current LCU. When encoding runs, run prediction from the above LCU is applied. If the above LCU is not available, then of the run is directly coded without prediction. The related syntax is shown in Table 1. The purpose of this experiment is to evaluate the coding efficiency of removing run prediction. The modified syntax of removing run prediction is shown in Table 2.

Table 1.  ALF unit syntax in HM-6.0

| alf_unit(rx, ry, cIdx, lcuIdx, endrX, oneUnitFlag ) { | Descriptor |
| --- | --- |
| if( oneUnitFlag ) { |  |
| … |  |
| } else { |  |
| if( !alf_repeat_row _flag[ cIdx ] ) { |  |
| if( rx = = 0 ¦¦ alfRun[ cIdx ][ ry ][ rx ] < 0 ) |  |
| if( lcuIdx –numCtbInWidth < 0 ) |  |
| alfRun[ cIdx ][ ry ][ rx ] = 0 + alf_run_diff | u(v) |
| else |  |
| alfRun[ cIdx ][ ry ][ rx ] = alfRun[ cIdx ][ ry − 1 ][ rx ]+ alf_run_diff | s(v) |
| alfRun[ cIdx ][ ry ][ rx+ 1 ] = alfRun[ cIdx ][ ry ][ rx ] − 1 |  |
| … |  |
| } else |  |
| alfRun[ cIdx ][ ry ][ rx ] = alfRun[ cIdx ][ ry − 1 ][ rx ] |  |
| } |  |
| } |  |

Table 2.  Modified ALF unit syntax for removing run prediction

| alf_unit(rx, ry, cIdx, lcuIdx, endrX, oneUnitFlag ) { | Descriptor |
| --- | --- |
| if( oneUnitFlag ) { |  |
| … |  |
| } else { |  |
| if( !alf_repeat_row _flag[ cIdx ] ) { |  |
| if( rx = = 0 ¦¦ alfRun[ cIdx ][ ry ][ rx ] < 0 ) |  |
| alfRun[ cIdx ][ ry ][ rx ] = alf_run | u(v) |
| alfRun[ cIdx ][ ry ][ rx+ 1 ] = alfRun[ cIdx ][ ry ][ rx ] − 1 |  |
| … |  |
| } else |  |
| alfRun[ cIdx ][ ry ][ rx ] = alfRun[ cIdx ][ ry − 1 ][ rx ] |  |
| } |  |
| } |  |

## CE2.g.1: Apply pixel access constraint in slice header mode

In HM-6.0, two kinds of ALF encoding algorithms are supported. One uses picture-based optimization for high efficiency applications and its corresponding signaling mode is called APS mode, and the other uses LCU-based optimization for low latency applications and its corresponding signaling mode is called slice header mode. When the LCU-based optimization is used, the bottom four luma pixel rows and the bottom two chroma pixel rows in one LCU are disallowed to be used due to the deblocking process. However, this constraint is not enough in practical LCU-based video encoding systems. In the deblocking process, not only the lower LCU is required to process the bottom pixel rows in the current LCU, but also the right LCU is needed to process the rightmost pixel columns in current LCU. Moreover, ALF process is performed after SAO process. Therefore, a reasonable pixel access constraint is to disallow using the bottom four luma pixel rows, the bottom two chroma pixel rows, the rightmost five luma pixel columns, and the rightmost three chroma pixel coumns in each LCU during ALF coefficient derivation.

## CE2.h.1: Reuse luma filter in chroma filtering for slice header mode

In JCTVC-H0066 [5], luma filter coefficients are always used in chroma filtering to save chroma side information. In this experiment, as the syntax shown in Table 3, alf_chroma_coeff_from_luma_flag is added to indicate whether luma filter coefficients are reused in chroma filtering or not. When alf_chroma_coeff_from_luma_flag is equal to zero, chroma filter coefficients should be decoded. If alf_chroma_coeff_from_luma_flag is equal to one, luma filter coefficients are used in chroma filtering and no other information should be decoded for chroma ALF. Note that when luma ALF is disabled, alf_chroma_coeff_from_luma_flag is inferred as zero.

Table 3.  Modified ALF unit syntax for reusing luma filter coefficients in chroma filtering

| alf_info( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| if(NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| alf_new_filter_set_flag | u(1) |
| if( alf_new_filter_set_flag = = 0 && NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| alf_stored_filter_set_idx[ cIdx ] | u(v) |
| else { |  |
| if( cIdx = = 0 ) { |  |
| … |  |
| } else |  |
| alf_chroma_coeff_from_luma_flag[ cIdx ][ ry ][ rx ] | u(1) |
| if( alf_chroma_coeff_from_luma_flag[ cIdx ][ ry ][ rx ] = = 0 ) { |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ][ ry ][ rx ][ 0 ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

# Simulation results

Table 4 shows the results of CE2.b.1. Common test conditions in JCTVC-H1100 [6] are used except that ALF with picture-based optimization and SAO with picture-based optimization are always enabled in both the anchor and the test. It can be seen that removing run prediction has no coding efficiency loss. In fact, run prediction is also used in the slice header mode of HM-6.0, and removing run prediction for slice header mode also shows no coding efficiency loss according to our extra data.

Table 5 shows the results of CE2.g.1. Common test conditions in JCTVC-H1100 [6] are used except that ALF with LCU-based optimization and SAO with LCU-based optimization are always enabled in both the anchor and the test. It is shown that additionally avoiding rightmost pixel columns in each LCU during ALF coefficient derivation causes 0.1-0.3% luma losses and 0.4-1.3% chroma losses in comparison with only avoiding the bottom pixel rows.

Table 6 shows the results of CE2.h.1. Common test conditions in JCTVC-H1100 [6] are used except that ALF with LCU-based optimization and the pixel access constraint of CE2.g.1 and SAO with LCU-based optimization are always enabled in both the anchor and the test. It is observed that reusing luma filter coefficients for chroma filtering in one LCU suffers 0.1-0.2% luma losses and achieves 0.6-3.3% chroma gains.

Table 4.  Results of CE2.b.1 (removing run prediction) with inaccurate run times

Table 5.  Results of CE2.g.1 (additionally avoiding rightmost pixels) with inaccurate run times

Table 6.  Results of CE2.h.1 (reusing luma filters for chroma) with inacuurate run times

# Conclusion

This contribution reports three ALF experiments in CE2. The first one is CE2.b.1: Remove run prediction for APS mode (picture-based optimization). Simulation results show no coding efficiency loss for CE2.b.1. The second one is CE2.g.1: Additionally disallow using rightmost pixel columns in each LCU during filter coefficient derivation for slice header mode (LCU-based optimization). Simulation results show 0.1-0.3% luma coding efficiency losses and 0.4-1.3% chroma coding efficiency losses for CE2.g.1. The last one is CE2.h.1: Reuse luma filter coefficients in chroma filtering for slice header mode (LCU-based optimization). Simulation results show 0.1-0.2% luma coding efficiency losses and 0.6-3.3% chroma coding efficiency gains for CE2.h.1.

# References

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, T. Yamakage, T. Itoh, T. Chujoh, T. Watanabe, “CE8.a.4: One-stage/Two-stage SAO and ALF with LCU-based syntax,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0274, February, 2012.

- “High Efficiency Video Coding (HEVC) text specification draft 6”, JCTVC-H1003, February, 2012.

- “High Efficiency Video Coding (HEVC) Test Model 6 (HM 6) Encoder Description”, JCTVC-H1002, February, 2012.

- T. Yamakage, I. S. Chong, Y.-W. Huang, M. Narroschke, M. Budagavi, “Description of Core Experiment 2 (CE2): Adaptive loop filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1102, February, 2012.

- A. Fuldseth, G. Bjøntegaard, “CE8.a.2: ALF with LCU-based syntax,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0066, February, 2012.

- “Common HM test conditions and software reference configurations”, JCTVC-H1100, February, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).