| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0174 |
| --- | --- |

| Title: | CE3.1.2: Parallel context assignment for significance map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

____________________________

# Abstract

This contribution reports results of CE3.1.2, which is based on the parallel context assignment for significance map coding in JCTVC-H0286. Two context formation patterns are proposed for 3-bin and 4-bin parallel context assignment respectively. Moreover, all positions inside one TU use the same pattern, so the pattern switch in HM-6.0 can be removed for complexity reduction. Simulation results reportedly show 0.1% bit rate increase for the proposed 3-bin parallel context assignment and 0.2% bit rate increase for the proposed 4-bin parallel context assignment. In low QP conditions (QP = 12, 17, 22, 27), the bit rate is increased by 0.0-0.1% for 3-bin and 4-bin parallel context assignments.

# Introduction

For the significant_coeff_flag coding of large TUs (e.g. 16x16, 32x32) and non-square TUs (e.g. 16x4, 4x16, 32x8, 8x32) in HM-6.0, the context selection depends on neighboring values. The context formation pattern of this neighbor-based context selection is shown in Figure 1 and Figure 2(a). For coding coefficient X, five coefficients including I, H, F, E, and B are used for the context formation. In order to achieve 2-bin parallel context assignment, the context formation pattern is modified as the pattern of Figure 2(b) for the blue coefficients in Figure 2(c). For other white coefficients in Figure 2(c), the pattern of Figure 2(a) is used.

Figure 1.  Context formation pattern of neighbor-based context selection

Figure 2.  Context formation patterns for 2-bin parallel context assignment

It can be seen that the parallelism of context assignment for significance map is limited to two bins in HM-6.0. In this contribution, we propose a new context formation pattern to support 3-bin and 4-bin parallel context assignments. The rest of this contribution is organized as follows. Details of the proposed methods will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed methods

## Context formation pattern for 3-bin parallel context assignment

In order to support 3-bin parallel context assignment for significance map coding, the data dependency for two previously decoded bins should be removed. The coefficients I and F in Figure 2(a) should not be used in the context formation pattern. Therefore, we propose a new context formation pattern, as shown in Figure 3. This pattern is to move the coefficients I and F to bottom-right by one pixel.

Figure 3.  Context formation pattern for 3-bin parallel context assignment

## Context formation pattern for 4-bin parallel context assignment

To support 4-bin parallel context assignment, the data dependency for three previously decoded bins is removed. The coefficients I, F and B in Figure 2(a) should not be used in the context formation pattern. A new context formation pattern is proposed as shown in Figure 4. This pattern is to move the coefficients B in Figure 3 to right by two pixels.

Figure 3.  Context formation pattern for 4-bin parallel context assignment

# Simulation results

The proposed methods are implemented on HM-6.0. All the results were simulated according to the common test conditions defined in JCTVC-H1100 [1]. Table 1 shows the results of the proposed context formation pattern for 3-bin parallel context assignment. The bit rate is increased by 0.1/0.1/0.2% in AI/RA/LB main profile conditions, and 0.1% in HE10 conditions. Table 2 shows the results of the proposed context formation pattern for 4-bin parallel context assignment. The bit rate is increased by 0.2% in all conditions. All run times are roughly unchanged. Table 3 and Table 4 show the results of the proposed methods for 3-bin and 4-bin parallel context assignment in low QP (QP = 12, 17, 22, 17), respectively. The bit rate is increased by 0.0% in all intra conditions and 0.1% in random access and low delay-B conditions for 3-bin and 4-bin parallel context assignment.

Since the proposed methods use only one unified pattern for all positions in the TU, the condition checks for blue coefficients in Figure 2(c) are unnecessary. In HM-6.0, the code of “if( ( ( posX != 0 ) || ( posY != 0 ) ) && ( ( posX != 3 ) || ( posY != 2 ) ) )” is used for the condition checks. For every coefficient, a total of 5 comparison and 3 and/or operations can be removed by the proposed unified context formation pattern.

# Conclusions

In this contribution, two new context formation patterns for significance map coding are proposed to support 3-bin and 4-bin parallel context assignments. For each bin of significance flag, the data dependency of two or three previously decoded bins is removed. Simulation results reportedly show 0.1-0.2% bit rate increase and no run time change.

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the proposed context formation pattern for 3-bin parallel context assignment in normal QP conditions (22, 27, 32, 37)

Table 2.  Results of the proposed context formation pattern for 4-bin parallel context assignment in normal QP conditions (22, 27, 32, 37)

Table 3.  Results of the proposed context formation pattern for 3-bin parallel context assignment in low QP conditions (12, 17, 22, 27)

Table 4.  Results of the proposed context formation pattern for 4-bin parallel context assignment in low QP conditions (12, 17, 22, 27)