| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0175 |
| --- | --- |

| Title: | Improved LM mode with template shift |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xingyu Zhang Oscar C. Au Feng Zou Jingjing Dai Chao Pang | Email: | eexyzhang@ust.hk eeau@ust.hk fengzou@ust.hk jjdai@ust.hk pcece@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution improves the chroma intra prediction mode LM mode. By changing the shape of the template in the LM mode, the outer border of the neighbouring pixels can also be considered. We propose four schemes of changing the shape of the template. All of the four schemes do not increase the computational complexity but provide a BD-rate up to -0.02%, -0.27% and -0.26% for Y, Cb and Cr components respectively under intra main configuration with LM on and intra HE 10bit configuration without Class F. No computational complexity is added.

# Introduction

In LM mode, a linear model is used for luma-to-chroma prediction. The parameters of the linear model are calculated using the neighbouring pixels. Specifically, for an NxN chroma block, the top N neighbouring pixels and the left N neighbouring pixels are used to derive the parameters. However, sometimes the neighbouring pixels in the outer border can provide useful hints for the content inside current block. Such behaviour has been studied and utilized in JCTVC-G358 and JCTVC-H0176. Both JCTVC-G358 and JCTVC-H0176 take advantages of the outer border by adding new chroma intra prediction modes, and provide coding gains. Since new modes are introduced, the complexity of the encoder or decoder will be increased to some extent, and mode coding is also an issue to be considered.

In this contribution, instead of introducing new modes, we modify the existing LM mode to include part of the outer border in 4 different schemes. All of the four schemes use 2N pixels. Performance improvement is achieved without computational complexity increasing.

# Proposed Modification

The neighbouring pixels used in current LM mode are shown in Fig. 1(a).  The proposed schemes utilize the outer border pixels by shifting the top N pixels right with a distance Dx, and shifting the left N pixels down with a distance Dy, as shown in Fig. 1(b).

Fig. 1 Neighbouring pixels used for parameters derivation in LM mode:

(a) existing template in LM mode, (b) Modified template in LM mode

Here we propose 4 schemes for different setting of  Dx and Dy.

Scheme 1: Dx=1, Dy=1;

Scheme 2: Dx=1, Dy=0;

Scheme 3: Dx=2, Dy=0;

Scheme 4: Dx=N/4, Dy=0;

# Results

Simulations are based on HM6.0. According to [2], LM mode is off by default under Intra Main configuration, and is on by default under Intra HE 10bit. For the sake of completeness, we test the proposed schemes under Intra Main and Intra HE 10bit conditions. But when Intra Main condition is applied, LM mode is turned on for both the anchor and the proposed solutions.  Table 1 to Table 4 show simulation results correspond to Scheme 1 to Scheme 4. Table 5 gives a summary of those schemes.

Table 1: Scheme 1 ( Dx=1, Dy=1);

|  | All Intra Main (with LM on) |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | Y | U | V |
| Class A | 0.01% | -0.03% | -0.04% | 0.02% | -0.11% | 0.16% |
| Class B | -0.03% | -0.33% | -0.31% | -0.05% | -0.35% | -0.23% |
| Class C | -0.03% | -0.20% | -0.14% | -0.02% | -0.08% | -0.11% |
| Class D | -0.06% | -0.46% | -0.42% | -0.05% | -0.43% | -0.38% |
| Class E | -0.01% | -0.32% | -0.45% | -0.02% | -0.17% | -0.29% |
| Overall | -0.02% | -0.27% | -0.26% | -0.03% | -0.24% | -0.17% |
|  | -0.02% | -0.27% | -0.26% | -0.03% | -0.25% | -0.16% |
| Class F | 0.03% | 0.01% | 0.29% | 0.03% | 0.19% | 0.43% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2: Scheme 2 ( Dx=1, Dy=0)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.18% | 0.00% | 0.00% | -0.15% | 0.13% |
| Class B | -0.02% | -0.24% | -0.20% | -0.04% | -0.26% | -0.18% |
| Class C | -0.03% | -0.20% | -0.19% | -0.04% | -0.15% | -0.17% |
| Class D | -0.06% | -0.37% | -0.39% | -0.05% | -0.34% | -0.31% |
| Class E | 0.00% | -0.16% | -0.24% | -0.01% | -0.19% | -0.19% |
| Overall | -0.02% | -0.23% | -0.20% | -0.03% | -0.22% | -0.14% |
|  | -0.02% | -0.25% | -0.20% | -0.03% | -0.22% | -0.15% |
| Class F | 0.01% | -0.02% | 0.07% | 0.00% | 0.01% | 0.15% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3: Scheme 3 ( Dx=2, Dy=0)

|  | All Intra Main (with LM on) |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.05% | 0.05% | 0.01% | -0.09% | 0.20% |
| Class B | -0.03% | -0.28% | -0.21% | -0.04% | -0.27% | -0.19% |
| Class C | -0.01% | -0.16% | 0.04% | -0.01% | -0.02% | 0.04% |
| Class D | -0.04% | -0.36% | -0.31% | -0.03% | -0.36% | -0.26% |
| Class E | -0.01% | -0.25% | -0.35% | -0.01% | -0.27% | -0.33% |
| Overall | -0.02% | -0.22% | -0.15% | -0.02% | -0.20% | -0.10% |
|  | -0.02% | -0.22% | -0.15% | -0.02% | -0.21% | -0.10% |
| Class F | 0.05% | -0.10% | 0.10% | 0.06% | 0.13% | 0.19% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 4: Scheme 4 ( Dx=N/4, Dy=0)

|  | All Intra Main (with LM on) |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.02% | -0.10% | 0.09% | 0.02% | -0.11% | 0.18% |
| Class B | -0.03% | -0.31% | -0.24% | -0.05% | -0.33% | -0.18% |
| Class C | -0.03% | -0.22% | -0.18% | -0.04% | -0.20% | -0.19% |
| Class D | -0.05% | -0.40% | -0.36% | -0.04% | -0.37% | -0.29% |
| Class E | -0.01% | -0.22% | -0.33% | -0.01% | -0.30% | -0.30% |
| Overall | -0.02% | -0.25% | -0.20% | -0.03% | -0.26% | -0.15% |
|  | -0.02% | -0.26% | -0.19% | -0.03% | -0.26% | -0.15% |
| Class F | 0.01% | -0.02% | 0.10% | 0.01% | -0.10% | -0.02% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 5: Summary of different schemes

|  | All Intra Main (with LM on) |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Scheme 1: Dx=1, Dy=1 | -0.02% | -0.27% | -0.26% | -0.03% | -0.24% | -0.17% |
| Scheme 2: Dx=1, Dy=0 | -0.02% | -0.23% | -0.20% | -0.03% | -0.22% | -0.14% |
| Scheme 3: Dx=2, Dy=0 | -0.02% | -0.22% | -0.15% | -0.02% | -0.20% | -0.10% |
| Scheme 4: Dx=N/4, Dy=0 | -0.02% | -0.25% | -0.20% | -0.03% | -0.26% | -0.15% |

# Conclusion

This contribution modifies the template used in LM mode for parameters calculation. Four schemes are provided. All the modified templates consider not only the inner border pixels but also the outer border pixels. Performance improvement is achieved without additional complexity. In particular, we recommend scheme 4 to be accepted into HM7.0.

# References

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003_dK,  San Jose, Feb. 2012.

- F. Bossen, “Common test conditions”, JCTVC-H1100, San Jose, Feb. 2012.

- J. Chen, et al, “New intra chroma prediction using inter-channel correlation”, JCTVC-E266, Geneva, Mar. 2011.

- X. Zhang and O.C. Au, et al, “New Modes for Chroma Intra Prediction”, JCTVC-G358, Geneva, Nov. 2011

- C. Gisquet and E. Francois, “Non-CE6a: Border-adaptive subsampling for LM mode”, JCTVC-H0176, San Jose, Feb. 2012

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD Texts

The following changes correspond to Scheme 4 with Dx=N/4 and Dy=0, while the changes for other schemes are quite similar.

##### 8.4.3.1.8 Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	 luma neighbouring samples PLM [ x, y ], with x=-1, y = 2*nS/4... 2*(nS+ nS/4)−1; and x=0…2*nS-1, y=-1.

–	chroma neighbouring samples p [ x, y ], with x=-1, y = nS/4... nS+ nS/4−1; and x=0…nS-1, y=-1.

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS−1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + Log2( nS ) − 14 )		(8-1)

pY’[ x, −1 ] = ( PLM[ 2x−1, −1 ] + 2*PLM[ 2x, −1 ] + PLM[ 2x+1, −1 ] + 2 ) >> 2, with x = nS/4..nS+nS/4−1	(8-2)

pY’[ −1, y ] = ( PLM[ −1, 2y ] + PLM[ −1, 2y+1 ] ) >> 1, with y = 0..nS−1	(8-3)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x, y = 0..nS−1	(8-4)

- Variables L, C, LL, LC and k1 are derived as follows:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		(8-5)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		(8-6)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(8-7)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(8-8)

L = 		(8-9)

C = 		(8-10)

LL = 	(8-11)

LC = 	(8-12)