| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0176 |
| --- | --- |

| Title: | Non-CE3: Modified method for coding transform coefficient level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta,hsiang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified method for entropy coding the level values of quantized transform coefficients. In order to further improve the bit-stream parsing throughput rate, the two-stage method for coding significance map is further extended for coding coeff_abs_level_greater1_flag. A new syntax flag is encoded at the beginning of the coding pass for coeff_abs_level_greater1_flag to indicate if there is any significant coefficient with coeff_abs_level_greater1_flag equal to 1 within the current sub-block. The simulation results reportedly show Y BD-rate increases 0.1%, 0.2%, and 0.3% for AI-Main, RA-Main and LDB-Main, respectively, and 0.1%, 0.1%, and 0.1% for AI-HE10, RA-HE10 and LDB-HE10, respectively, under the HM common test conditions.

# Introduction

The current method for entropy coding of quantized transform coefficients, as specified in HM-6.0, divides each transform block into sub-blocks. The transform coefficients within each sub-block are encoded through multiple coding passes for coding syntax elements significant_coeff_flag, coeff_abs_level_greater1_flag, coeff_abs_level_greater2_flag, and coeff_sign_flag, coeff_abs_level_remaining, respectively.

In the first coding pass, a two-stage coding method proposed in [1] is employed for coding significance flags. At the beginning of the pass, syntax significant_coeff_group_flag is encoded to indicate the significance information at the sub-block level. If significant_coeff_group_flag is equal to 1, then the significance flags within the sub-block are explicitly coded one by one. Otherwise, syntax significant_coeff_flag is not encoded and is inferred to be 0 for all the significance flags within the sub-block. In the pass for coding coeff_abs_level_greater1_flag, it was recently adopted in [2] to apply context-adaptive binary arithmetic coding only up to 8 significant coefficients in order to improve throughput rate performance. The information about the level values for the remaining coefficients are encoded in the pass for coding coeff_abs_level_remaining in the bypass entropy coding mode.

# Proposed Method

This contribution proposes a modified method for entropy coding the level values of quantized transform coefficients. The proposed method can be considered as an extension of the current method for the significance flag pass to the pass for coding coeff_abs_level_greater1_flag. A new syntax flag coeff_abs_level_greater1_group_flag is encoded at the beginning of the coding pass for coeff_abs_level_greater1_flag. When the flag is equal to 1, it represents there is at least one coefficient with coeff_abs_level_greater1_flag equal to 1 among up to the last 8 significant coefficients in the backward scanning order within the sub-block. Syntax coeff_abs_level_greater1_flag is then explicitly encoded one by one for those coefficients, except for possible the final inferred flag value equal to 1 given all previous coeff_abs_level_greater1_flag values equal to 0. When coeff_abs_level_greater1_group_flag is equal to 0, the coding pass is terminated and coeff_abs_level_greater1_flag is not encoded. The value of coeff_abs_level_greater1_flag is then inferred to be 0 for up to the last 8 coefficients. In this way, coeff_abs_level_greater1_flag for a large number of coefficients from the same sub-block can be jointly represented by one syntax flag, leading to a reduced number of CABAC bin counts and improved bit-stream parsing throughput rate.

# Experimental Results

The proposed algorithm has been implemented using HM-6.0 as software base and experimentally evaluated according to the common test conditions defined in JCTVC- H1100 [3]. The experimental results are provided in the included spread sheets and summarized in Tables 1.

Table: Summary of the BD-rate results according to the common test conditions

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.6% | 0.6% | 0.1% | 0.4% | 0.6% |
| Class B | 0.2% | 0.6% | 0.6% | 0.2% | 0.5% | 0.5% |
| Class C | 0.1% | 0.4% | 0.5% | 0.1% | 0.3% | 0.3% |
| Class D | 0.1% | 0.4% | 0.3% | 0.1% | 0.3% | 0.3% |
| Class E | 0.2% | 0.4% | 0.5% | 0.2% | 0.2% | 0.4% |
| Overall | 0.1% | 0.5% | 0.5% | 0.1% | 0.3% | 0.4% |
|  | 0.1% | 0.4% | 0.5% | 0.1% | 0.3% | 0.4% |
| Class F | 0.1% | 0.5% | 0.4% | 0.1% | 0.4% | 0.2% |
| Enc Time[%] | 101.5% |  |  | 101.3% |  |  |
| Dec Time[%] | 101.0% |  |  | 101.3% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% |
| Class B | 0.1% | 0.1% | 0.3% | 0.1% | 0.3% | 0.3% |
| Class C | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class D | 0.1% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% |
| Class F | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100.8% |  |  | 100.4% |  |  |
| Dec Time[%] | 101.6% |  |  | 100.9% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.6% | 0.2% | 0.1% | 0.7% | 0.7% |
| Class C | 0.3% | 0.9% | 0.7% | 0.1% | 0.6% | 0.4% |
| Class D | 0.4% | 0.8% | 0.6% | 0.2% | 0.5% | 0.6% |
| Class E | 0.1% | 0.0% | -0.1% | 0.2% | -0.2% | 0.5% |
| Overall | 0.3% | 0.6% | 0.4% | 0.1% | 0.5% | 0.6% |
|  | 0.3% | 0.7% | 0.3% | 0.1% | 0.5% | 0.5% |
| Class F | 0.0% | 0.1% | 0.5% | 0.1% | 1.1% | 0.2% |
| Enc Time[%] | 100.8% |  |  | 100.5% |  |  |
| Dec Time[%] | 101.6% |  |  | 100.4% |  |  |

# References

- N. Nguyen, T. Ji, D. He, G. Martin-Cocher, L. Song “Multi-level significance maps for Large Transform Units,” Document of Joint Collaborative Team on Video Coding, JCTVC-G644, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Jianle Chen, Wei-Jung Chien, Joshi Rajan, Joel Sole, and Marta Karczewicz, “Non-CE1: throughput improvement on CABAC coefficients level coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-H554, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).