The proposed CD modifications are based on HM6 (JCTVC-H1103). The inserted parts and removed parts are marked in yellow and pink, respectively.

### Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 1 ] |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| numNonZeroCoeff = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| numNonZeroCoeff++; |  |
| } |  |
| implicitCoeffAbsLevelGreater1 = 0 |  |
| if( numNonZeroCoeff > 0 ) { |  |
| coeff_abs_level_greater1_group_flag[ xCG ][ yCG ] |  |
| implicitCoeffAbsLevelGreater1 = 1 |  |
| } |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| if( coeff_abs_level_greater1_group_flag[ xCG ][ yCG ] && ( (numSigCoeff < 7 && numSigCoeff < numNonZeroCoeff - 1 ) ¦ ¦ ¦implicitCoeffAbsLevelGreater1 = = 0 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| implicitCoeffAbsLevelGreater1 = 0 |  |
| } |  |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = − transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

## Semantics

### Residual coding semantics

coeff_abs_level_greater1_group_flag[ xCG ][ yCG ] specifies for the coefficient group position ( xCG, yCG ) within the current transform block whether there are transform coefficient levels greater than 1 among the last 8 significant coefficients in scan order within the corresponding coefficient group at location ( xCG, yCG )

When coeff_abs_level_greater1_group_flag[ xCG ][ yCG ] is not present, it is inferred to be equal to 0.

coeff_abs_level_greater1_flag[ n ] specifies for the scanning position n whether there are transform coefficient levels greater than 1.

When coeff_abs_level_greater1_flag[ n ] is not present, it is inferred as follows.

- If both of the following conditions are true, coeff_abs_level_greater1_flag[ n ] is inferred to be equal to 1

- numSigCoeff = = min ( 7, numNonZeroCoeff – 1 )

- implicitCoeffAbsLevelGreater1 is equal to 1

- Otherwise, coeff_abs_level_greater1_flag[ n ] is inferred to be equal to 0.

When coeff_abs_level_greater1_flag[ n ] is not present, it is inferred to be equal to 0.