| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0177 |
| --- | --- |

| Title: | Table size reduction and CABAC context reduction for last significant position coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou, Oscar C. Au, Jingjing Dai, Xingyu Zhang Chao Pang The Hong Kong University of Science and Technology, Clear Water Bay, Hong Kong | Tel: Email: | +852-51328827 fengzou@ust.hk eeau@ust.hk jjdai@ust.hk eexyzhang@ust.hk pcece@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

- In this proposal, a method to reduce the table size and CABAC context models for last position coding is proposed. First, a context mapping table of size 28 is simplified to a table of size 24 by removing 4 unused entries. It is reported that the proposed approach provides no change over the HM6.0 in all common test coding conditions. Second, more than 2 bins (with CABAC context) for TU32x32 share the same CABAC context, thus reducing the CABAC contexts. Simulation results show that there is 0.01%, 0.02%, 0.03%, 0.05%, 0.01% and 0.05% Y-BD rate change for Intra Main, Intra HE10bit, RA Main, RA HE10bit, LD-B Main and LD-B HE10bit respectively under the common test condition.

# Introduction

In the JCT-VC San Jose meeting in Feb, 2012, there is one technique JCTVC-H0537 [1] adopted, which is last significant (none zero) coefficient position coding related, comprising horizontal position x and vertical position y. The last significant coefficient position is represented by a binary string which consists of two parts. The first part is truncated unary code where each bin is encoded with CABAC contexts, while the second part is fixed binary code with by-pass coding (no context).

Table 1, 2, 3 and 4 show the binarization of each position and its corresponding CABAC context assignment for the binarization for different TU sizes.

Table 1. Last position coding for TU 4x4 Luma

Table 2. Last position coding for TU 8x8 Luma

Table 3. Last position coding for TU 16x16 Luma

Table 4. Last position coding for TU 32x32 Luma

For each bin in the truncated unary codeword, the CABAC context assignment is shown in Table 5.

Table 5. The context assignment for each bin in the truncated unary codeword for Luma.

In order to get the CABAC context for each bin for different TU sizes, a 1D table is used to store the CABAC context index in HM6.0 shown in Table 6. The relationship between the entries in the 1D table and the CABAC context index for each bin (with context) of different TU sizes is shown as follows (for horizontal position x/ vertical position y):

For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth – 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]

For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight – 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]

Table 6. 1D table to store the CABAC context index for each bin of different TU sizes in HM6.0

- Note that four elements (labeled with red curve) in the 1D table are never used, and are just place holders. For example, for TU 4x4, there is up to 3 bins that need the context modeling; however, the table specifies four bins for TU 4x4 that need the context modeling. Therefore, the element marked in the red circle is redundant. The same is true for TU 8x8, 16x16 and 32x32. Thus, in total there are four redundant elements in the 1D table.

# Proposed Method

This contribution proposes two modifications for the last significant position coding.

Technique 1. Remove the unused elements in the table.

Accordingly, the derivation of the context index becomes

For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( log2TrafoWidth – 2 )* log2TrafoWidth + binIdx ]

For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[  ( log2TrafoHeight – 2 )* log2TrafoHeight  + binIdx ].

Note that one addition is reduced compared with HM6.0, besides the table size reduction.

Technique 2. Context reduction by confining more than 2 bins to share the same context

We observed that in last position coding some bins have very similar statistics and propose to share contexts for these bins. Specifically, context sharing among several bins for TU32x32 is used. For clarity, the contexts assignment is show in Table 7.

Table 7. The CABAC context index for each bin of different TU sizes (Total contexts 12)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 4 | 5 | 5 | 2 |  |  |  |  |
| TU 16x16 | 6 | 7 | 8 | 8 | 9 | 9 | 2 |  |  |
| TU 32x32 | 10 | 10 | 10 | 10 | 11 | 11 | 11 | 11 | 2 |

# Results

We implemented the technique into the HM6.0 [2] software and tested with the common test condition [3]. For Technique 1: Table size reduction, there is no performance change.

The following table shows the result of Technique 2: Context reduction (12 contexts)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.07% | 0.07% | 0.02% | 0.07% | 0.00% | 0.06% |
| Class B | 0.01% | 0.01% | 0.00% | 0.01% | -0.01% | -0.03% |
| Class C | 0.00% | -0.02% | 0.02% | 0.00% | 0.00% | 0.07% |
| Class D | 0.00% | 0.01% | -0.01% | 0.02% | -0.03% | -0.02% |
| Class E | 0.00% | -0.02% | 0.00% | 0.03% | -0.05% | 0.03% |
| Class F | -0.01% | 0.09% | 0.00% | 0.02% | 0.03% | 0.00% |
| Overall | 0.01% | 0.03% | 0.00% | 0.02% | -0.01% | 0.02% |
|  | 0.01% | 0.02% | 0.00% | 0.02% | 0.00% | 0.02% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.20% | 0.13% | 0.14% | 0.11% | 0.08% | 0.18% |
| Class B | 0.03% | -0.05% | 0.12% | 0.03% | -0.09% | -0.11% |
| Class C | -0.02% | 0.00% | -0.05% | 0.02% | 0.00% | -0.13% |
| Class D | -0.02% | -0.10% | -0.19% | 0.02% | -0.12% | 0.02% |
| Class E |  |  |  |  |  |  |
| Class F | -0.03% | 0.02% | -0.09% | 0.05% | -0.08% | 0.04% |
| Overall | 0.03% | 0.00% | -0.01% | 0.05% | -0.04% | -0.01% |
|  | 0.03% | -0.02% | -0.01% | 0.04% | -0.01% | -0.01% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.07% | -0.35% | -0.09% | 0.06% | -0.18% | 0.01% |
| Class C | 0.06% | 0.18% | 0.10% | 0.02% | 0.04% | -0.09% |
| Class D | 0.06% | -0.30% | 0.19% | 0.04% | -0.19% | -0.05% |
| Class E | -0.02% | -0.13% | 0.52% | 0.05% | 0.07% | 0.33% |
| Class F | -0.15% | 0.06% | 0.26% | 0.10% | -0.94% | 2.22% |
| Overall | 0.01% | -0.12% | 0.17% | 0.05% | -0.25% | 0.47% |
|  | 0.01% | -0.08% | 0.20% | 0.05% | -0.26% | 0.48% |
| Enc Time[%] | 95% |  |  | 94% |  |  |
| Dec Time[%] | 96% |  |  | 94% |  |  |

# Conclusion

The proposed Technique 1 reduces the table size along with reduced complexity and no performance change. The proposed Technique 2 reduces the CABAC contexts by allowing more than 2 bins to share the same context, with no significant RD performance loss. With little RD performance change and reduced complexity, it is recommended to be adopted in the standard.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] Liwei Guo, Marta Karczewicz, Wei-Jung Chien, Xianglin Wang. “CABAC Contexts Reduction for Last Position Coding” JCT-VC Document, JCTVC-H0537, Feb. 2012

[2] HM 6.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.0

[3] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-H1100, Feb. 2012.

# WD Changes based on JCTVC-H1003 V22

Scheme1:

##### 9.2.3.1.2 Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

Inputs to this process are the binIdx, the colour component index cIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

Table 9-1 – Specifcation of lastCtx[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 2 | 2 | 6 | 7 | 8 | 8 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 9 | 9 | 2 | 2 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 | 2 |

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 3 | 4 | 5 | 5 | 2 | 6 | 7 | 8 | 8 | 9 | 9 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |  |  |  |  |
| lastCtx[ i ] | 2 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 |  |  |  |  |

The vector lastCtx[ i ] is specified in Table 9-1 and ctxIdxInc is derived as follows.

When cIdx is equal to 0,

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth − 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]	(9-1)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight − 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]	(9-2)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoWidth − 2 ) )+ 15	(9-3)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoWidth − 2 ) )+ 15	(9-4)

Scheme 2:

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 2 | 2 | 6 | 7 | 8 | 8 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 9 | 9 | 2 | 2 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 | 2 |

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 2 | 2 | 6 | 7 | 8 | 8 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 9 | 9 | 2 | 2 | 10 | 10 | 10 | 10 | 11 | 11 | 11 | 11 | 2 | 2 |