| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0178 |
| --- | --- |

| Title: | Simplified alpha bit-depth restriction in chroma LM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xingyu Zhang Oscar C. Au Feng Zou Jingjing Dai Chao Pang | Email: | eexyzhang@ust.hk eeau@ust.hk fengzou@ust.hk jjdai@ust.hk pcece@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution simplifies the process of restricting the bit-depth of alpha value to 7 bits in the LM mode. With proposed operations, the if-else conditional judgement can be skipped. Therefore, both HM software and WD can be simplified. Moreover, with the simplification, a modest gain is achieved for chroma components under intra main configuration (with LM on) and intra HE 10bit configuration.

# Introduction

In LM mode, a linear model is used for luma-to-chroma prediction.

(1)

The parameters  and  of the linear model are derived using the neighbouring pixels. In order to apply integer operations, the  value is up-scaled by 213 and clipped to the range [-215, 215-1], named.  In HM6.0, after the clipping operation, the precision of  is reduced to 7 bits with a normalized factor iShift. ( i.e. except the sign bit, only 6 most significant bits will be preserved while other lower bits are discarded by right shift.) An example is illustrated in Fig. 1.

Fig. 1 Example of alpha bit-depth restriction.

The value of  is derived by  as equation (2).										                                        (2)

Thus, the actual prediction process using a lower precision  is as equation (3).

(3)

However, as it can be seen in the bit-depth restriction process, a condition needs to be judged after the clipping process as follows:

iShift=13;

……

= Clip3(-( 1 << 15 ), ( 1 << 15 ) - 1, );

if(<= (26 -1) &&  >= -26 )

{

// do nothing

}

else

{

Short n = CountLeadingZerosOnes();

= >> (9-n);

iShift -= (9-n);

}

# Proposed Modification

The aim of the proposed modifications is to remove the if-else conditional judgement in the bit-depth restriction process. This can be achieved in two different ways.

## Scheme 1

Scheme 1 removes the if-else conditional judgement by setting the last B bits of  to zeros. For example, when B=6, the following steps will be taken:

iShift=13;

……

= Clip3(-( 1 << 15 ), ( 1 << 15 ) - 1, );

=& (~(26-1));

Short n = CountLeadingZerosOnes();

= >> (9-n);

iShift -= (9-n);

As the bit-depth of alpha is restricted to 7, the if-else conditional judgement can be removed when B >=5.

## Scheme 2

Scheme 2 realizes the same function by the following changes:

- value is up-scaled by 213-B and clipped to the range [-215-B, 215-B-1] to get .

- After the clipping operation, is left shifted by B bits.

For example, when B=6, the following steps will be taken:

iShift=7;

……

= Clip3(-( 1 << 9 ), ( 1 << 9 ) - 1, );

=<<6;

Short n = CountLeadingZerosOnes();

= >> (9-n);

iShift = 13-(9-n);

Same as Scheme 1, the if-else conditional judgement can be removed when B >=5.

# Simulation

Simulations are based on HM6.0. According to [2], LM mode is off by default under Intra Main configuration, and is on by default under Intra HE 10bit. For the sake of completeness, we test the proposed simplification under Intra Main and Intra HE 10bit conditions. But when Intra Main condition is applied, LM mode is turned on for both the anchor and the proposed solutions.

For the same B value, Scheme 1 and Scheme 2 have the same results. Summary of results with different values of B are shown in Table 1 and Table 2. More detailed results are shown in Table 3 to 6.

Table 1. Summary of the simulation results (8 bit and 10 bit sequences included)

| Class A-E | Intra Main (with LM ON) |  |  | Intra HE 10bit |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| B = 5 | 0.00% | -0.21% | -0.18% | 0.00% | -0.24% | -0.24% |
| B = 6 | 0.00% | -0.46% | -0.39% | 0.00% | -0.58% | -0.60% |
| B = 7 | 0.00% | -0.88% | -0.76% | 0.00% | -0.98% | -0.96% |
| B = 8 | 0.00% | -1.18% | -1.02% | 0.03% | -1.33% | -1.19% |

Table 2. Summary of the simulation results (10 bit sequences excluded)

| Class A-E (8 bit sequence only) | Intra Main (with LM ON) |  |  | Intra HE 10bit |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| B = 5 | 0.00% | -0.05% | -0.01% | 0.00% | -0.06% | 0.00% |
| B = 6 | 0.00% | -0.07% | 0.00% | 0.00% | -0.12% | 0.00% |
| B = 7 | 0.00% | -0.15% | 0.02% | 0.00% | -0.18% | -0.01% |
| B = 8 | 0.00% | -0.15% | 0.12% | 0.02% | -0.19% | 0.11% |

Table 3. Results for B=5.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.87% | -0.90% | 0.00% | -0.99% | -1.25% |
| Class B | 0.00% | -0.05% | -0.04% | 0.00% | -0.06% | -0.05% |
| Class C | 0.00% | -0.08% | -0.01% | 0.00% | -0.06% | 0.03% |
| Class D | 0.00% | -0.04% | -0.02% | 0.00% | -0.04% | 0.02% |
| Class E | 0.00% | 0.01% | 0.08% | 0.01% | -0.03% | 0.09% |
| Overall | 0.00% | -0.21% | -0.18% | 0.00% | -0.24% | -0.24% |
| Overall (8 bit) | 0.00% | -0.05% | -0.01% | 0.00% | -0.06% | 0.00% |

Table 4. Results for B=6.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | -2.07% | -1.98% | -0.01% | -2.51% | -3.02% |
| Class B | -0.01% | -0.10% | -0.05% | -0.01% | -0.16% | -0.12% |
| Class C | 0.00% | -0.08% | 0.01% | 0.00% | -0.09% | 0.07% |
| Class D | -0.01% | -0.02% | -0.02% | 0.00% | -0.02% | 0.00% |
| Class E | 0.01% | 0.00% | 0.10% | 0.01% | -0.09% | 0.14% |
| Overall | 0.00% | -0.46% | -0.39% | 0.00% | -0.58% | -0.60% |
| Overall (8 bit) | 0.00% | -0.07% | 0.00% | 0.00% | -0.12% | 0.00% |

Table 5. Results for B=7.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | -3.90% | -3.89% | 0.00% | -4.35% | -4.81% |
| Class B | -0.02% | -0.22% | -0.07% | -0.01% | -0.28% | -0.19% |
| Class C | 0.01% | -0.13% | 0.00% | 0.01% | -0.05% | 0.09% |
| Class D | 0.00% | -0.06% | -0.01% | 0.01% | -0.06% | -0.01% |
| Class E | 0.00% | -0.02% | 0.22% | 0.01% | -0.11% | 0.23% |
| Overall | 0.00% | -0.88% | -0.76% | 0.00% | -0.98% | -0.96% |
| Overall (8 bit) | 0.00% | -0.15% | 0.02% | 0.00% | -0.18% | -0.01% |

Table 6. Results for B=8.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.02% | -5.48% | -5.59% | 0.06% | -6.12% | -6.43% |
| Class B | -0.02% | -0.33% | -0.07% | 0.00% | -0.41% | -0.17% |
| Class C | 0.02% | 0.01% | 0.18% | 0.06% | 0.09% | 0.30% |
| Class D | 0.01% | -0.06% | 0.06% | 0.03% | -0.10% | 0.08% |
| Class E | 0.01% | 0.03% | 0.47% | 0.01% | -0.04% | 0.45% |
| Overall | 0.00% | -1.18% | -1.02% | 0.03% | -1.33% | -1.19% |
| Overall (8 bit) | 0.00% | -0.15% | 0.12% | 0.02% | -0.19% | 0.11% |

# Conclusion

This contribution simplifies the process of alpha bit-depth restriction in the LM mode. With the proposed simplification, the conditional judgement can be removed with a modest gain for chroma components. Both the WD text and HM code will be simplified. It’s recommended to accept this simplification to HM7.0 with B=7.

# References

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003_dK,  San Jose, Feb. 2012.

- F. Bossen, “Common test conditions”, JCTVC-H1100, San Jose, Feb. 2012.

- J. Chen, et al, “Chroma intra prediction by reconstructed luma samples”, JCTVC-C206, Guangzhou, Oct. 2010.

- M. Budagavi, et al, “Luma-based chroma intra prediction simplification”, JCTVC-F233, Torino, Jul. 2011.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD Changes

Note that in current WD [1], the description for LM mode has a lot mismatches from the HM software, e.g. the use of   “Log2(x)” function may give a floating point number, and equation (8-74) should use a3 instead of a . Those mismatches have been clearly stated in the contribution JCTVC-I0148 .

In the following, the needed WD changes are given in two versions. One is based on current WD description, while the other is based on the suggested WD description in JCTVC-I0148. Without generality for different value of B when B>=5, the following changes can be applied.

## WD Changes for Scheme 1

### WD changes based on current WD text

##### 8.4.3.1.8 Specification of Intra_FromLuma prediction mode

……….

- Variables a, b and k are derived as:

a1 = ( LC << k2 ) – L*C		(8-1)
a2 = ( LL << k2 ) – L*L		(8-2)
k1 = Max( 0, Log2( abs( a2 ) ) − 5 ) – Max( 0, Log2( abs( a1 ) ) − 14 ) + 2	(8-3)
a1s= a1 >> Max(0, Log2( abs( a1 ) ) − 14 )		(8-4)
a2s= abs( a2 >> Max(0, Log2( abs( a2 ) ) − 5 ) )		(8-5)
a3= a2s < 1 ? 0 : Clip3( −215, 215−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 )	(8-6)

-
			-

a4=a3&(~(2B-1))                                                                                                               (8-73)

a=a4>> (Log2( abs( a4 ) ) − 6 )                                                                                         (8-74)

k=13-(Log2( abs( a4 ) ) − 6 )                                                                                         (8-75)

b= ( L – ( ( a*C ) >> k1 ) + ( 1 << ( k2 − 1 ) ) ) >> k2	(8-10)

where lmDiv is specified in Error: Reference source not found with the input a2s.

……….

### WD changes based on suggested text for LM mode in JCTVC-I0148

##### 8.4.3.1.8 Specification of Intra_FromLuma prediction mode

……….

3) 	Variables a, b and k are derived as:

a1 = ( LC << k1 ) – L*C		(8-11)
a2 = ( LL << k1 ) – L*L		(8-12)

if (a1==0), k2=0

otherwise, k2= Max( 0, Floor(log2(abs(a1)))− 14 )	(8-69)

if (a2==0), k3=0

otherwise, k3= Max( 0, Floor(log2(abs(a2))) − 5 )	(8-70)

k4 = k3 – k2 + 2 		(8-71)

a1s = a1 >> k2 		(8-72)

a2s = a2 >> k3 		(8-73)

if (a2s < 1), a3 = 0

otherwise, if (k4 >= 0), a3 = (a1s*lmDiv) >> k4)

otherwise, (k4 < 0), a3 = (a1s*lmDiv) << (-k4)	(8-74)

a4  = Clip3( −215, 215−1, a3);		(8-75)

a5=a4&(~(2B-1))                                                                                                                     (8-76)

if (a5 == 0), k5 = 0

otherwise, k5 = Floor(log2(abs( a5) + (sign(a5) -1)/2 ))– 5	(8-77)

k = 13 - k5		(8-78)

a=a4>> k5		(8-79)

b= ( C – ( ( a*L ) >> k ) + ( 1 << ( k1 − 1 ) ) ) >> k1	(8-80)

where lmDiv is specified in Error: Reference source not found with the input a2s.

……….

## WD Changes for Scheme 2

### WD changes based on current WD text

##### 8.4.3.1.8 Specification of Intra_FromLuma prediction mode

……….

- Variables a, b and k are derived as:

a1 = ( LC << k2 ) – L*C		(8-13)
a2 = ( LL << k2 ) – L*L		(8-14)
k1 = Max( 0, Log2( abs( a2 ) ) − 5 ) – Max( 0, Log2( abs( a1 ) ) − 14 ) + 15-(13-B)	(8-15)
a1s= a1 >> Max(0, Log2( abs( a1 ) ) − 14 )		(8-16)
a2s= abs( a2 >> Max(0, Log2( abs( a2 ) ) − 5 ) )		(8-17)
a3= a2s < 1 ? 0 : Clip3( −215-B, 215-B−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 )	(8-18)

-
			-

a4=a3<<B                                                                                                                              (8-73)

a=a4>> (Log2( abs( a4 ) ) − 6 )                                                                                      (8-74)

k=13-(Log2( abs( a4 ) ) − 6 )                                                                                         (8-75)

b= ( L – ( ( a*C ) >> k1 ) + ( 1 << ( k2 − 1 ) ) ) >> k2	(8-21)

where lmDiv is specified in Error: Reference source not found with the input a2s.

……….

### WD changes based on suggested text for LM mode in JCTVC-I0148

##### 8.4.3.1.8 Specification of Intra_FromLuma prediction mode

……….

3) 	Variables a, b and k are derived as:

a1 = ( LC << k1 ) – L*C		(8-22)
a2 = ( LL << k1 ) – L*L		(8-23)

if (a1==0), k2=0

otherwise, k2= Max( 0, Floor(log2(abs(a1)))− 14 )	(8-69)

if (a2==0), k3=0

otherwise, k3= Max( 0, Floor(log2(abs(a2))) − 5 )	(8-70)

k4 = k3 – k2 + 15-(13-B) 		(8-71)

a1s = a1 >> k2 		(8-72)

a2s = a2 >> k3 		(8-73)

if (a2s < 1), a3 = 0

otherwise, if (k4 >= 0), a3 = (a1s*lmDiv) >> k4)

otherwise, (k4 < 0), a3 = (a1s*lmDiv) << (-k4)	(8-74)

a4  = Clip3( −215-B, 215-B−1, a3);		(8-75)

a5=a4<<B                                                                                                                               (8-76)

if (a5 == 0), k5 = 0

otherwise, k5 = Floor(log2(abs( a5) + (sign(a5) -1)/2 ))– 5	(8-77)

k = 13 - k5		(8-78)

a=a4>> k5		(8-79)

b= ( C – ( ( a*L ) >> k ) + ( 1 << ( k1 − 1 ) ) ) >> k1	(8-80)

where lmDiv is specified in Error: Reference source not found with the input a2s.

……….