| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0180 |
| --- | --- |

| Title: | Parallel NxN merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-6.0, reference indices for temporal merging candidates of non-first PUs are set to zero, and the spatial merging candidates located in the first PU are excluded from the merging candidate list for 2NxN, Nx2N, and AMP partitions to remove the motion data dependency in merge mode. However, for the NxN merge mode, the spatial merging candidates located in other NxN PUs are still included in the merging candidate list, which prevents the NxN merge mode from being processed in parallel. Although the NxN merge is not allowed in the common test conditions, it is still allowed by CD (and by the Main Profile). In this contribution, the spatial merging candidates located in the other NxN PUs are excluded to remove data dependency between NxN PUs. Simulation results reportedly show no essentially coding efficiency loss, while all partition types in merge mode can be processed in parallel.

# Introduction

In the adopted JCTVC-H0278 [1], it was proposed to set reference indices for temporal merging candidates of non-first PUs to zero to remove the motion data dependency. In HM-6.0, for the second PU of Nx2N/nLx2N/nRx2N and 2NxN/2NxnU/2NxnD, the spatial merging candidate located in the first PU has also been removed to avoid the motion data dependency. However, for the NxN merge, the spatial merging candidates located in other PUs are still included in the merging candidate list. This causes the motion data dependency issue in NxN merge because the merging candidate list construction process of the 2nd, 3rd, or 4th NxN PUs needs to wait until the motion data of the previous PUs is available. Although the NxN merge is not allowed in the common test conditions, it is still allowed by CD (and by the Main Profile).

Figure 1 illustrates the spatial merging candidates of the 2nd, 3rd, and 4th NxN PUs. In this example, the spatial merging candidates A1 for the 2nd NxN PU, B1 and B0 for the 3rd NxN PU, and A1, B2 and B1 for the 4th NxN PU cannot be accessed before the motion data of the previous PUs is available. This causes motion data dependency between NxN PUs and prevents parallel processing of NxN merge.

In this contribution, we propose to remove the spatial merging candidates located in other NxN PUs from the merging candidate list for NxN merge to provide the complete CU-based parallel merge solution. The rest of this contribution is organized as follows. The proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Figure 1.  Illustration of spatial merging candidates of the 2nd, 3rd, and 4th NxN PUs

# Proposed method

In the NxN merge mode, the spatial motion vector neighbours located in other NxN PUs are removed from the merging candidate list to avoid the motion data dependency. In this way, all partition types in merge mode can be processed in parallel.

# Simulation results

The common test conditions specified in JCTVC-H1100 [1] are used. In the common test conditions, there is no Inter NxN partition because the Inter 4x4 partition is disabled and the SCU size is 8x8. In test1, we enable the Inter 4x4 partitions to generate the test data, and disabling Inter 4x4 partitions in HM-6.0 is used as the anchor. In Table 1, it is reported that enabling Inter 4x4 partition reduces 0.1% bit rate.

In test2, in the NxN merge mode, the spatial motion vector neighbours located in other NxN PUs are removed from the merging candidate list to generate the test data. The test data in test1 is used as anchor. In Table 2, it is reported that for non-first NxN PUs, removing spatial motion vector neighbours located in other NxN PUs from the merging candidate list does not cause essential coding efficiency loss.

# Conclusions

MediaTek propose to remove the spatial merging candidates located in other NxN PUs from the merging candidate list to remove the motion data dependency in NxN merge mode and to provide the complete CU-based parallel merge solution. Simulation results reportedly show no essential bit rate increases in comparison with HM-6.0 when the Inter 4x4 partition is enabled.

# References

- Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei, “Common Non-CE9: CU-based parallel merge mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0278, February, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February, 2012.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of test1, where the test is enabling Inter 4x4 partitions for HM-6.0 and the anchor uses HM-6.0 common test condition settings

Table 2.  Results of test2, where the proposed parallel NxN merge and the anchor is enabling Inter 4x4 partitions for HM-6.0