| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0182 |
| --- | --- |

| Title: | Constrained motion data compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Wade Wan 5300 California Ave, Irvine, CA 92617, USA Tim Hellman 200 Brickstone Square, Suite 401, Andover, MA 01810, USA Peisong Chen 16340 W Bernardo Dr, San Diego, CA 92127, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com Wade Wan +1-949-926-6387 wwan@broadcom.com Tim Hellman +1-978-719-1348 timhellman@broadcom.com Peisong Chen +1-858-521-4835 peisongc@broadcom.com |
| Source: | MediaTek Inc., Broadcom Corporation |  |  |

_____________________________

# Abstract

In HM-6.0, the motion data in the upper LCU row are 2:1 down-sampled to reduce the motion data line buffer size. The motion data compression is always performed regardless the minimum PU width. However, the motion data compression for minimum PU width equal to or larger than 8 cannot reduce any line buffer due to the need to support the worst case that happens when the minimum PU width is 4. In this proposal, a new constraint is added: the motion data compression is performed only when the minimum PU width equal to 4. In common test conditions, this constraint will not cause any difference.

# Constraint on Motion Data Compression

Motion data of a block include inter prediction direction (2 bits), reference index (4 bits), and motion vector (MV, 16 bits for MV_x and 16 bits for MV_y). In order to reduce the line buffer storage and access, the motion data compression in JCTVC-G229 [1] was adopted. The motion data compression is to perform 2:1 motion data down-sample in the motion data line buffer. For each PU that needs to access the motion data of the upper LCU row, only the compressed motion data in the line buffer can be accessed. Figure 1 shows an example of the 2:1 motion data compression under the common test condition [2]. Each motion data unit corresponds to a minimum PU width. In this example, the minimum PU width is four. For every four motion data units (e.g. 0-3 or 4-7 or 8-11 or 12-15), the motion data of the first and the last units are stored in the line buffer. The motion data of the first unit represents the motion data of the first two units. The motion data of the last unit represents the motion data of the last two units. In this way, only half memory is required. The memory size of motion data line buffer for 4Kx2K videos is reduced by 4.86K bytes.

Figure 1.  The proposed 2:1 motion data compression of one line

In current HEVC committee draft (CD), the motion data compression is always performed regardless the minimum PU width. In Main Profile, the minimum PU width equal to 4 shall be supported. Therefore, performing motion data compression when the minimum PU width is equal to or larger than 8 cannot help to reduce the line buffer at all and can be regarded as a waste of computation. In this proposal, the motion data compression is allowed only when the minimum PU width is equal to 4. If the minimum PU width is equal to or larger than 8, the motion data compression is skipped. In the common test condition, this proposal will not result in any difference.

The texts in Section 8.5.2.1.2 of the HEVC CD are modified as follows.

xB0 = (xB0>>Log2MinCbSize)<<Log2MinCbSize) + ((xB0>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-1)
xB1 = (xB1>>Log2MinCbSize)<<Log2MinCbSize) + ((xB1>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-2)
xB2 = (xB2>>Log2MinCbSize)<<Log2MinCbSize) + ((xB2>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-3)

where MinPuSize = 1 << ( Log2MinCbSize − 1)

xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-4)
xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-5)
xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-6)

Similar modifications are also made in Section 8.5.2.1.6 and Section 8.7.1.3.

# Conclusions

In this contribution, the motion data compression is allowed only for minimum PU width equal to 4. It can reduce the complexity and might achieve a little coding gain when the minimum PU width is larger than 4. In the common test condition, this constraint will not lead to any difference.

# References

- T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei, “CE12.3.3: Reducing motion data line buffers,” Document of Joint Collaborative Team on Video Coding, JCTVC-G229, November, 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).