# Draft Changes

###### 8.5.2.1.2 Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- a variable singleMCLFlag,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP − 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP − 1 ), ( xP + nPSW − 1,  yP − 1 ) or ( xP − 1,  yP − 1 ), the following applies.

–	If yP−1 is smaller than yCtb, the following applies.

xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-1)
xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-2)
xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-3)

xB0 = (xB0>>Log2MinCbSize)<<Log2MinCbSize) + ((xB0>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-4)
xB1 = (xB1>>Log2MinCbSize)<<Log2MinCbSize) + ((xB1>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-5)
xB2 = (xB2>>Log2MinCbSize)<<Log2MinCbSize) + ((xB2>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-6)

where MinPuSize = 1 << ( Log2MinCbSize − 1)

–	If one of the following conditions is true with X being replaced by 0 and 1, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xN >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2)).

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and N is equal to B1

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and N is equal to A1

- N is equal to B1 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B0 and the prediction units covering luma location ( xP + nPSW − 1, yP − 1 ) ( N = B1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to A0 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xP + nPSW − 1, yP − 1 ) ( N = B1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xP − 1, yP + nPSH − 1 ) ( N = A1 ) and luma location ( xN, yN ) (Cand. N) have the same motion vectors and the same reference indices

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

###### 8.5.2.1.6 Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-1 – Spatial motion vector neighbours

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP − 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line.

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- If one of the following conditions is true, the variable isScaledFlagLX is set equal to 1.

- the prediction unit covering luma location ( xA0, yA0 ) is available and PredMode is not MODE_INTRA.

- the prediction unit covering luma location ( xA1, yA1 ) is available and PredMode is not MODE_INTRA.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 − MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xAk ][ yAk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-7)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-8)

mvLXA = Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 			
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-9)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) )	(8-10)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-11)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0− MinPuSize , xB2 = xP − MinPuSize and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line.

- If yP−1 is less than yCtb, the following applies.

xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-12)
xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-13)
xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-14)

xB0 = (xB0>>Log2MinCbSize)<<Log2MinCbSize) + ((xB0>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-15)
xB1 = (xB1>>Log2MinCbSize)<<Log2MinCbSize) + ((xB1>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-16)
xB2 = (xB2>>Log2MinCbSize)<<Log2MinCbSize) + ((xB2>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-17)

where MinPuSize = 1 << ( Log2MinCbSize − 1)

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPSW, xB1 = xB0 − MinPuSize , and xB2 =  xP − MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xBk ][ yBk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1,  mvLXA is set equal to mvLXB and refIdxA is set equal to refIdxB and availableFlagLXA is set equal to 1.

- When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListListB( refIdxB ) ) is not equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-18)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-19)

mvLXB =Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-20)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListB( refIdxB ) ) )	(8-21)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-22)

#### Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE−1 and j = 0..nE−1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CbSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE−1 and j = 0..nE−1, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1, for ( xEk+j, yEj ) with j=0, 1, the following applies.

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj − 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1, for ( xEk, yEj +j) with j=0, 1, the following applies.

- Set sample p0 = recPicture[ xC + xEk − 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1 and yC + yEj − 1 is less than yCtb, the following applies.

    - One motion vector is used to predict the prediction unit containing sample recPicture[ xL ][ yC + yEj − 1 ], one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples, where xL is set equal to ( ( ( xC + xEk ) >> 3 ) << 3 ) + ( ( ( xC + xEk ) >> 3 ) & 1 ) * 7. ( ( ( xC + xEk ) >> Log2MinCbSize ) << Log2MinCbSize ) + ( ( ( xC + xEk ) >> Log2MinCbSize ) & 1 ) * ( MinPuSize * 2 − 1) and MinPuSize is set equal to 1<<(Log2MinCbSize − 1).

  - Otherwise, the following applies.

    - One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

    - The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.