# CD changes

Proposed committee draft changes are described below using the following rules:

- Removed text from JCTVC-H1003 is shown with strikethrough

- Added test is highlighted in yellow

- 7.3.2.6 Sample adaptive offset parameter syntax

| aps_sao_param( ) { | Descriptor |
| --- | --- |
| sao_cb_cr_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| sao_num_lcu_in_width_minus1 | ue(v) |
| sao_num_lcu_in_height_minus1 | ue(v) |
| sao_one_luma_unit_flag | u(1) |
| if(sao_one_luma_unit_flag) |  |
| sao_offset_vlc( 0, 0, 0 ) |  |
| if( sao_cb_cr _enable_flag ){ |  |
| sao_one_cb_cr _unit_flag | u(1) |
| if( sao_one_cb_cr_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag ){ |  |
| sao_one_cr_unit_flag | u(1) |
| if( sao_one_cr_unit_flag ) |  |
| sao_offset_vlc( 0, 0, 2 ) |  |
| } |  |
| for( ry = 0; ry <= sao_num_lcu_in_height_minus1; ry++ ) { |  |
| for( rx = 0; rx <= sao_num_lcu_in_width_minus1; rx++ ) { |  |
| if( aps_sample_adaptive_offset_flag && ! sao_one_luma_unit_flag ) { |  |
| if( ry > 0 && rx = = 0 ) |  |
| sao_repeat_row_flag[ 0 ] | u(1) |
| sao_unit_vlc(rx, ry, 0 ) |  |
| } |  |
| if( sao_cb_cr_enable_flag && ! sao_one_cb_cr_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 1 ] | u(1) |
| sao_unit_vlc(rx, ry, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag & & !sao_one_cr_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 2 ] | u(1) |
| sao_unit_vlc( rx, ry, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

### 7.3.4 Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| AddrUp= CtbAddrRS – PicWidthInCtbs |  |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| if(sao_cb_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( nextbits(24) = = 0x000002 ) { |  |
| entry_point_marker_two_3bytes | f(24) |
| tile_idx_minus_1 | u(v) |
| } |  |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

      - 7.4.2.6 Sample adaptive offset parameter semantics

sao_cb_cr _enable_flag equal to 1 specifies that the sample adaptive offset process for Cb and Cr is applied to the current picture.

sao_cr_enable_flag equal to 1 specifies that the sample adaptive offset process for Cr is applied to the current picture.

sao_num_lcu_in_width_minus1 plus 1 specifies the number of coding treeblocks in picture width.

sao_num_lcu_in_height_minus1 plus 1 specifies the number of coding treeblocks in picture height.

sao_one_luma_unit_flag equal to 1 specifies that all of coding treeblocks in luma component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the luma component are signalled on a coding treeblock basis.

sao_one_cb_cr _unit_flag equal to 1 specifies that all of coding treeblocks in Cb component and Cr component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the Cb component and the Cr component are signalled on a coding treeblock basis.

sao_one_cr_unit_flag equal to 1 specifies that all of coding treeblocks in Cr component of current slice are processed using the same SAO parameters; equal to 0 specifies that the SAO parameters for the Cr component are signalled on a coding treeblock basis.

sao_repeat_row_flag[ cIdx ] equal to 1 specifies that the SAO parameters of coding treeblocks in the current coding treeblock row are the same as those of above coding treeblocks for the colour component cIdx; equal to 0 specifies that the current coding treeblock row has new SAO parameters. When sao_repeat_row_flag is not present, it is inferred to be equal to 0.

      - 7.4.2.7 Sample adaptive offset unit VLC semantics

The number of times the SAO parameters corresponding to a coding treeblock are repeated for subsequent coding treeblocks in the same row is represented by saoRun[ cIdx ][ rx ][ ry ]. The array index cIdx specifies the colour component; cIdx is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices rx and ry specify the location ( rx, ry ) of the considered coding treeblock relative to the top-left coding treeblock of the picture.

sao_run_diff specifies the saoRun of current coding treeblock if the current row is the first row, otherwise specifies the difference between the run of current coding treeblock and the run of the above coding treeblock. When saoRun is greater than or equal to 0, the syntax elements in sao_offset_vlc() are derived from the corresponding syntax elements of the left coding treeblock. When ry is equal to 0, the length of the sao_run_diff syntax element is Ceil( Log2(sao_num_lcu_in_width_minus1 − rx + 2) ) bits.

sao_merge_up_flag equal to 1 specifies that the syntax elements in sao_offset_vlc() are derived from the corresponding syntax elements of the above coding treeblock; equal to 0 specifies that the syntax elements in sao_offset_vlc() are not derived from the corresponding syntax elements of the above coding treeblock. When sao_merge_up_flag is not present, sao_merge_up_flag is inferred to be equal to 0.

      - 7.4.2.8 Sample adaptive offset VLC semantics

sao_type_idx[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-1 of current coding treeblock at position rx and ry for the colour component cIdx.

When sao_type_idx[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx − 1 ][ ry ].

Table 7-1 – Specification of the edge type for SAO

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - Edge type (informative) |
| --- | --- |
| - 0 | - Not applied |
| - 1 | - 1D 0-degree edge |
| - 2 | - 1D 90-degree edge |
| - 3 | - 1D 135-degree edge |
| - 4 | - 1D 45-degree edge |
| - 5 | - Band |

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the pixel range when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5.

When sao_band_position[ cIdx ][ rx ][ ry ] is not present it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx − 1 ][ ry ].

sao_offset[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current coding treeblock at position rx and ry for the colour component cIdx.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to 1 or 2), bitDepth is set equal to BitDepthC.

It is a requirement of bitstream conformance that when sao_type_idx[ cIdx ][ rx ][ ry ] is not equal to 5, the values of sao_offset[ cIdx ][ rx ][ ry ][ i ] shall be in the range of 0 to ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) − 1, inclusive and otherwise, the values of sao_offset[ cIdx ][ rx ][ ry ][ i ] shall be in the range of −( 1<< ( Min( bitDepth, 10 ) − 5 ) ) to ( 1<< ( Min( bitDepth, 10 ) − 5 ) ) − 1, inclusive.

When sao_offset[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_up_flag is equal to 1, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset[ cIdx ][ rx − 1 ][ ry ][ i ].

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is less than 5 and i is larger than 1,offsetSign is set to −1.

- Otherwise (sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5 or i is less than 2), offsetSign is set to 1.

The array SaoOffsetVal is derived as follows.

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0		 (7-1)

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = 
			offsetSign*sao_offset[ cIdx ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) ) 	(7-2)

    - Sample adaptive offset process

A sample adaptive offset process shall be conditionally performed after the completion of the deblocking filter process for the decoded picture. This process is invoked when both sample_adaptive_offset_enabled_flag and aps_sample_adaptive_offset_flag are equal to 1.

This process is performed on a region basis which is aligned with the coding treeblock boundaries after the completion of the picture construction process prior to sample adaptive offset process for the entire decoded process.

The sample adaptive offset process is invoked with the region equal to entire picture here.

Depending on slice_sample_adaptive_offset_flag, sao_cb_cr_enable_flag and sao_cr_enable_flag, the following applies:

–	When slice_sample_adaptive_offset_flag is equal to 1 the following applies:

  - The sample adaptive offset process for luma region as specified in subclause 4 is invoked with the component index set equal to 0, the coding treeblock location ( 0, 0 ), the largest CU location ( 0, 0 ), the variable saoWidthInCtbs set equal to PicWidthInCtbs, and the variable saoHeightInCtbs set equal to PicHeightInCtbs, and the modified luma picture buffer of current region as output.

  - When sao_cb_cr_enable_flag is equal to 1, the sample adaptive offset process for colour component Cb as specified in subclause 4 is invoked with the component index set equal to 1, the coding treeblock location ( 0, 0 ), the largest CU location ( 0, 0 ), the variable saoWidthInCtbs set equal to PicWidthInCtbs, and the variable saoHeightInCtbs set equal to PicHeightInCtbs, and the modified chroma picture buffer for Cb of current region as output.

  - If sao_cb_cr_enable_flag is equal to 1, the sample adaptive offset process for colour component Cr as specified in subclause 4 is invoked with the component index set equal to 2, the coding treeblock location ( 0, 0 ), the largest CU location ( 0, 0 ), the variable saoWidthInCtbs set equal to PicWidthInCtbs, and the variable saoHeightInCtbs set equal to PicHeightInCtbs, and the modified chroma picture buffer for Cr of current region as output.

      - 8.7.2.1 Modification process for sample adaptive offset

Inputs to this process are:

–	a variable cIdx specifying colour component index,

–	a pair of variables ( rx, ry ) specifying the coding treeblock location,

–	a sample location ( x0, y0 ) specifying the top-left sample of the current largest coding unit,

–	a variable saoWidthInCtbs specifying the number of largest CUs in width of the current region

–	a variable saoHeightInCtbs specifying the number of largest CUs in height of the current region.

Output of this process is a modified picture buffer for the colour component cIdx.

The modified picture buffer for the colour component cIdx is derived as follows.

- Variable nS is set equal to ( 1 << Log2CtbSize ).

- Modification process for samples in a largest coding unit covering ( iC, jC ) with iC = x0 + k*nS for k = 0..saoWidthInCtb − 1 and jC = y0 + j*nS for j = 0..saoHeightInCtb − 1 specified in subclause 2 is invoked with the colour component index cIdx, the top-left sample of current largest coding unit ( iC, jC ), the coding treeblock location ( rx, ry ), the offset array saoValueArray set equal to SaoOffsetVal[ min (cIdx,1) ][ saoDepth ][ rx ][ ry ] and the largest coding unit size nS as inputs and the output is the modified picture buffer of colour component cIdx.

        - Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying colour component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the coding treeblock location,

–	an array saoValueArray specifying offset values,

–	a block size nS.

Output of this process is a modified picture buffer for the colour component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of colour component cIdx and saoTypeIdx is set equal to sao_type_idx[ min (cIdx,1) ][ rx ][ ry ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-2.

- A variable edgeIdx is derived as follows.

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-3)

- When any of the following conditions is true, edgeIdx is set equal to 2.

- The pixel at position  (xC + i + hPos[ k ], yC + j + vPos[ k ]) is ouside picture boundary

- slice_loop_filter_across_slices_enabled_flag is equal to 0 and the pixel at position (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to different slice.

- loop_filter_across_tiles_enabled_flag is equal to 0 and the pixel at position (xC + i + hPos[ k ], yC + j + vPos[ k ]) belongs to different tile.

- The reconstructed picture buffer is modified as follows:

- If pcm_loop_filter_disable_flag is equal to 1 and the recPicture[ xC + i, yC + j ] is a reconstructed sample of an I_PCM block or qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, the recSaoPicture[ xC + I yC + j ] is derived as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ]	(8-4)

with i = 0..nS−1 and j = 0..nS−1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

- Otherwise if cIdx is equal to 0 or 1,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]	(8-5)

with i = 0..nS−1 and j = 0..nS−1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

- Otherwise cIdx is equal to 2,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] - saoValueArray[ edgeTable[ edgeIdx ] ]	(8-6)

with i = 0..nS−1 and j = 0..nS−1 where 	edgeTable[5] = { 4, 3, 0, 2, 1}.

- Otherwise, , if saoTypeIdx is equal to 5, the following ordered steps apply:

- If cIdx is equal to 0, the variable bandShift is set equal to BitDepthY − 5

- Otherwise (cIdx is not equal to 0), bandShift is set equal to BitDepthC − 5.

- If cIdx is equal to 0 or 1, Tthe variable saoLeftClass is set equal to sao_band_position[ cIdx ][ rx ][ ry ]. Otherwise saoLeftClass is set equal to 32- sao_band_position[ 1 ][ rx ][ ry ]-4.

- The vector bandTable is defined with 32 elements and all elements are initially set to 0. Then, 4 of its elements (indicating the position of bands for which an offset is explicitly transmitted) are modified as follows:

for( i = 0; i < 4; i++ )
	bandTable[ (i + saoLeftClass) & 0x1F ]  = i + 1		(8-7)

- The reconstructed picture buffer recSaoPicture is modified as follows.

bandIdx = ( recPicture[ xC + i, yC + j ] >> bandShift )

- If cIdx is equal to 0 or 1
recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ bandTable[ bandIdx ] ]	(8-8)
	with i = 0..nS−1 and j = 0..nS−1

- Otherwise

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] - saoValueArray[ bandTable[ bandIdx ] ]		(8-9)
	with i = 0..nS−1 and j = 0..nS−1

–	Otherwise (sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS−1 and j = 0..nS−1	(8-10)

Table 8-2 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | −1 | 0 | −1 | 1 |
| hPos[1] | 1 | 0 | 1 | −1 |
| vPos[0] | 0 | −1 | −1 | −1 |
| vPos[1] | 0 | 1 | 1 | 1 |

Table 9-3 – Values of variable initValue for sao_merge_left_flag ctxIdx

| Initialization variable | sao_merge_left_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 118 | 118 | 118102 | 102 | 102 | 102 | 102 | 102 | 102 |

| Table 9-4 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element |  | initialisationType |  | Type of binarization |  |  | maxBinIdxCtx | ctxIdxTable |  | ctxIdxOffset |  |  |
| sao_merge_left_flag |  | 0 |  | FL, cMax = 1 |  |  | 0 | Table 9-3 |  | 0 |  |  |
|  |  | 1 |  |  |  |  | 0 | Table 9-3 |  | 32 |  |  |
|  |  | 2 |  |  |  |  | 0 | Table 9-3 |  | 64 |  |  |
| Table 9-5 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |  |  |  |  |  |
| Syntax element | ctxIdxTable, ctxIdxOffset |  |  |  | binIdx |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 1 |  |  | 2 |  | 3 | >=4 |
| sao_merge_left_flag | Table 9-3 |  | 0 |  | cIdx | na |  |  | na |  | na | na |
|  |  |  | 32 |  | cIdx | na |  |  | na |  | na | na |
|  |  |  | 64 |  | cIdx | na |  |  | na |  | na | na |