| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0184 |
| --- | --- |

| Title: | Non-CE1: Encoder modification for SAO interleaving mode. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume LAROCHE Tangi POIRIER Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents a non-normative change which consists in a simple encoder modification for the SAO interleaving LCU mode. The present contribution proposes to turn on/off the SAO tool for the current frame base on a statistical criterion of the previous frame. The aim is to save bitrate allocated to signal the non-use of SAO at LCU level by signaling it at frame level instead. This modification enables to reduce the encoding complexity by skipping the SAO process for some frames. The proposed method provides gains of respectively 0.5%, 0.4%, 0.3% for Y, U and V color components for LCU size 64x64 and 1.6%, 1.5%, 1.4% for LCU size 16x16 compared to CE1 anchors.

# Introduction

The interleaving SAO signalization was introduced in HM6.0. The principle is to transmit SAO parameters at LCU level instead of transmit them at frame level. The SAO parameters for each color component can be copied from the left or from the top LCU by using some flags. The syntax element regarding the SAO parameters and the associated flags are sent for each LCU and for each color component. Consequently, with the current syntax for SAO at least 3 flags are transmitted at LCU level.

The current syntax offers some advantages when using SAO for low delay scenario but it increases significantly the bitrate dedicated to SAO parameters. In particular, the cost is high when SAO filtering is not selected for the current LCU. Several contributions from CE1 or AHG 6 try to modify this syntax in order to reduce the cost when SAO is not selected for a given LCU. These contributions give some gains but should decrease the efficiency of SAO when it needs to be used.

In this contribution, we proposed a non-normative modification at the encoder side to disable SAO at frame level. The proposed modification saves bits when SAO is rarely used in a frame and can reallocate these bits when it SAO is often used in another frame.

In the current HM6.0 common conditions, a hierarchical coding of frames is used for both low delay and random access configurations which corresponds to a particular QP setting for each frame. Figure 1 shows this hierarchy of frames for low delay B configurations and the associated. Frames of depth 0 have the highest quality in the hierarchy. The QP for the frames of depth 0 corresponds to the traditional QP setting associated to the whole sequence. For frames of depth 0, the quantization parameter is set to the value QP for the first Intra frame, otherwise it is set to QP+1 for B frames. For the B frames of depth 1, the QP is set to QP+2 and for the B frames of depth 2, the QP is set to QP+3.

Figure 1: Illustration of the frame depth for low delay configurations.

By analysing the SAO usage (e.g. the rate of LCU where SAO is turned on) at frame level according to the frame depth, we observed that SAO is often/always selected for frame of depth 0 and less selected for depth 1 and even less for frames of depth 2 especially at low bitrates. Moreover, the number of SAO usage for frames of depth 1 and 2 is most of the time proportional to the number of SAO usage for frames of depth 0. The proposed encoding modification exploits these observations in order to improve the coding efficiency for the interleaving mode.

# Proposed Modification

The proposed encoding choice consists in enabling or disabling SAO at frame level. More precisely, the encoder algorithm turns off (or on) SAO for frames of depth 1 and depth 2 according to the number of LCUs filtered with SAO for the previous frame of level 0.

During the encoding of frame with a depth equal to 0 in the hierarchy, the number of times when sao_type_idx is equal to 0 (no SAO) for all components is counted. If this number of no SAO usage represents more than a certain amount (currently 75%) of all LCUs for all components, SAO is disabled for the following frames of depth 1 and depth 2, otherwise SAO is disabled for these depths.

This modification represents only 25 lines of modification in the source code in the HM6.0 encoder and does not affect at all the decoding process of SAO.

# Results

Simulations results were performed on a Linux platform according to the CE1 common test conditions [1]. The results are reported for interleaving mode based on low delay B configurations for 3 LCU sizes: 64x64, 32x32 and 16x16.

Table 1 shows the results of the proposed encoder modification compared to CE1 anchors. For LCU size 64x64 the average BDR gains are respectively 0.5%, 0.4% and 0.3% for Y, U and V color components. These average BDR gains increase when the LCU size is reduced. For LCU size 32x32, the average BDR gains for the three color components are 0.8% 0.9% 1.0% and for LCU size 16x16 the average BDR gains are 1.6% 1.5% 1.4%.

Table 1 Simulation results of the proposed encoder modification compared to the CE1 Anchors.

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -1.0% | -1.1% | -0.9% | -1.0% | -0.9% |
| Class C | -0.7% | -0.5% | -0.5% | -0.6% | -0.8% | -0.6% |
| Class D | -1.0% | -1.2% | -1.0% | -1.0% | -0.7% | -0.7% |
| Class E | -4.7% | -4.1% | -4.2% | -4.7% | -4.0% | -3.9% |
| Overall | -1.6% | -1.5% | -1.5% | -1.6% | -1.4% | -1.4% |
|  | -1.6% | -1.5% | -1.5% | -1.6% | -1.4% | -1.4% |
| Class F | -2.0% | -2.4% | -2.7% | -2.1% | -2.5% | -2.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.5% | -1.0% | -0.3% | -0.3% | -0.2% |
| Class C | -0.3% | -0.6% | -0.5% | -0.4% | -0.3% | -0.4% |
| Class D | -0.8% | -0.8% | -1.2% | -0.8% | -0.6% | -1.4% |
| Class E | -2.6% | -2.6% | -2.2% | -2.5% | -2.2% | -2.2% |
| Overall | -0.8% | -1.0% | -1.2% | -0.9% | -0.8% | -0.9% |
|  | -0.8% | -1.0% | -1.1% | -0.9% | -0.8% | -0.9% |
| Class F | -0.3% | -0.3% | -0.3% | -0.7% | -0.6% | -0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | 0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | -0.3% |
| Class D | -0.3% | -0.2% | 0.3% | -0.4% | -0.7% | -0.2% |
| Class E | -1.5% | -1.5% | -1.9% | -1.7% | -1.4% | -1.1% |
| Overall | -0.4% | -0.4% | -0.3% | -0.5% | -0.5% | -0.3% |
|  | -0.4% | -0.4% | -0.4% | -0.5% | -0.6% | -0.4% |
| Class F | 0.2% | -0.1% | -0.2% | -0.1% | 0.2% | -0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

# Conclusion

In this contribution, we present a simple encoder modification to enable or disable SAO at frame level based on the QP settings hierarchy. The aim is to save bits used for the LCU syntax when SAO is disabled at LCU level. This is particularly true when SAO is rarely used for some frames. The proposed method provides gain compared to CE1 anchors of respectively 0.5%, 0.4%, 0.3% for Y, U and V color component for LCU size 64x64 and 1.6%, 1.5%, 1.4% for LCU size 16x16. Compared to experiments conducted in CE1 where different normative changes have been proposed. This contribution demonstrates that similar results can be reached in a simplest way without introducing any non-normative change.

According to the coding efficiency improvement and complexity reduction at encoder side, it is recommended to adopt the proposed encoder modification for HM7.0.

# References

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb. 2012.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).