# Proposal 1: MPM0 derived from only left CU

2 less lines in CD text.

### Derivation process for luma intra prediction mode

…

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

- …

- The candModeList[ x ] with x=0..2 is derived as follows:

- candModeList[ 0 ] is set equal to candIntraPredModeA

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is less than 2 (either Intra_Planar or Intra_DC), candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = Intra_Planar		(8-1)
candModeList[1] = Intra_DC 1 - 	candModeList[ 0 ]	(8-2)
candModeList[2] = Intra_Angular (26)		(8-3)

  - Otherwise, candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = candIntraPredModeA		(8-4)
candModeList[1] = 2 + ( ( candIntraPredModeA − 2 − 1 ) % 32	(8-5)
candModeList[2] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32	(8-6)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[0] and candModeList[1] are derived as follows:

candModeList[0] = candIntraPredModeA		(8-7)
candModeList[1] = candIntraPredModeB		(8-8)

  - If none of candModeList[0] and candModeList[1] is equal to Intra_Planar, candModeList[2] is set equal to Intra_Planar,

  - Otherwise, if none of candModeList[0] and candModeList[1] is equal to Intra_DC, candModeList[2] is set equal to Intra_DC,

  - Otherwise, candModeList[2] is set equal to Intra_Angular (26).

…

# Proposal 2: MPM2 set to Planar mode

15 less lines in CD text.

### 8.4.1	Derivation process for luma intra prediction mode

…

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

- …

- The variable defaultMode is set equal to Intra_Planar.

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If availableN is equal to FALSE, intraPredModeN is set equal to Intra_DC.

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, if yB−1 is less than yCtb, intraPredModeA is set equal to IntraPredMode[ xBA ][ yBA ] and intraPredModeB is set equal to Intra_DC.

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- If intraPredModeN is equal to Intra_Planar, it is set equal to defaultMode and defaultMode mode is set equal to Intra_Angular (26).

- The candModeList[ x ] with x=0..2 is derived as follows:

- candModeList[ 0 ] is set equal to candIntraPredModeA

- candModeList[ 2 ] is set equal to Intra_Planar

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is less than 2 (either Intra_Planar or Intra_DC), candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = Intra_Planar		(8-9)
candModeList[1] = Intra_DC		(8-10)
candModeList[2] = Intra_Angular (26)		(8-11)

  - Otherwise, candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = candIntraPredModeA		(8-12)
candModeList[1] = 2 + ( ( candIntraPredModeA − 2 − 1 ) % 32	(8-13)
candModeList[21] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32	(8-14)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[0] and candModeList[1] are derived as follows:

candModeList[0] = candIntraPredModeA		(8-15)
candModeList[1] = candIntraPredModeB		(8-16)

  - If none of candModeList[0] and candModeList[1] is equal to Intra_Planar, candModeList[2] is set equal to Intra_Planar,

  - Otherwise, if none of candModeList[0] and candModeList[1] is equal to Intra_DC, candModeList[2] is set equal to Intra_DC,

  - Otherwise, candModeList[2] is set equal to Intra_Angular (26).

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If prev_intra_pred_flag[ xB ][ yB ] is equal to TRUE, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[ mpm_idx ],

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

- The array candModeList[x], x=0..2 is modified as the following ordered steps follows:

If candModeList[0] is greater than candModeList[1], swap two values.

  - If candModeList[0] is greater than candModeList[2], swap two values.

  - If candModeList[1] is greater than candModeList[2], swap two values.

- IntraPredMode[xB][yB] is derived as the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_luma_pred_mode + 1

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 2 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

# Proposal 2: MPM2 set to DC mode

15 less lines in CD text.

### 8.4.1	Derivation process for luma intra prediction mode

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar Intra_DC |
| 1 | Intra_DC Intra_Planar |
| Otherwise (2..34) | Intra_Angular |
| 35 | Intra_FromLuma (used only for chroma) |

For the rest, same as “Proposal 2: MPM2 set to Planar mode”, but switch Intra_DC and Intra_Planar modes