| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0186r1 |
| --- | --- |

| Title: | On Intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Stephane PAUTET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes two independent proposals related to intra prediction mode coding. The first proposal consists in making the first MPM (MPM0) only dependent on left neighboring CU. The second proposal consists in setting the third MPM (MPM2) to planar mode. These modifications are reported to simplify the intra mode coding process, by reducing the number of checks when MPM0 (2 to 3 less checks), MPM2 (6 to 8 less checks) or remaining mode (worst case, 4 less checks) is signaled. It is reported that the proposal 1 has no impact on the coding efficiency. With the proposal 2, it is reported an average BD-Rate gain of 0.1% for AI-Main and AI-HE10 cases and no difference for inter cases.

# Introduction

This proposal aims at simplifying the intra mode decoding process. It consists in two independent proposals.

- Proposal 1 slightly modifies the MPM derivation process by making the first MPM (MPM0) dependent from only the left CU. This saves decoder computation when MPM0 is signaled, which statistically occurs for around 35% of the CUs:  2 checks are needed instead of 5 to 6 checks in the current design. This proposal is described in section 3.

- Proposal 2 consists in always setting the third MPM to mode 0 (Planar). In current HM design, Planar mode is 90% of time among the 3 MPMs. Therefore, setting Planar as one of the MPMs can save computation.  This solution allows simplifying the intra mode decoding process. The worst case implies 4 less checks than the current design. In addition, when MPM2 is signaled (which occurs around for 20% of the CUs) no derivation is required, instead of 6 to 8 less checks in the current design.

Both proposals results in a simplified design and HEVC specification without any coding penalty (a slight gain is even observed with proposal 2).

# MPM derivation and Decoding tree in current HM

Intra mode decoding involves two parts: the MPM derivation and the Decoding tree.

Figure 1 depicts the current MPM derivation process. MPM0 and MPM1 derivation requires 5 to 6 checks. When all MPMs are needed (worst case), 8 checks are required.

Figure 2 depicts the current Decoding tree. In the remaining mode branch, 3 checks are required to sort the 3 MPMs. 3 additional checks are required to possibly increment the mode value. In the worst case, the decoding tree involves 7 checks.

In total, the Intra mode decoding worst case needs 15 checks (8 for the MPM derivation, 7 for the Decoding tree).

Figure 1: MPM derivation in HM6.

Figure 2: Decoding tree in HM6.

# Proposal 1: MPM0 derived from only left CU

Changes in proposal 1 are depicted in Figure 3 (red font). MPM0 only depends on left CU, which results in a slight modification of the MPM1 derivation when top and left CU have same mode, and this mode is Planar or DC.

Thanks to this change, when MPM0 is signaled, 2 to 3 checks are saved, compared to the current HM, which is beneficial in software implementation (MPM0 is selected in average around 35% of the time). The impact on coding efficiency is not significant (see ‘Results’ section and attached xls file).

Figure 3: modified MPM derivation in proposal 1.

# Proposal 2: MPM2 set to Planar mode

Statistically, Planar mode is among the 3 MPMs in 90% of the time (see Table 7 in annex). Therefore this proposal consists in always setting MPM2 as Planar mode.

The MPM derivation process of proposal 2 is depicted in Figure 4. Compared to the current HM (cf Figure 1), 1 less check is required in the worst case. In addition, when MPM2 is signaled, 6 to 8 checks are saved, which is beneficial in software implementation, since Planar is used 20% of the time (see the statistics on usage of the different modes in Table 4).

The Decoding tree of proposal 2 is depicted in Figure 5. Compared to the current HM (cf Figure 2), 3 less checks are required in the worst case.

The encoding process is slightly modified in order to ensure that the third MPM is always among the candidates set of the RDO check, after the first HAD-based selection. In the HM6.0, the third MPM is not guaranteed to be checked. With the proposed modification, if the 3rd MPM is not in the candidates set, the last candidate is replaced by the 3rd MPM.

In total, proposal 2 results in 4 less checks for the worst case of the full intra mode decoding process.

Figure 4: modified MPM derivation in proposal 2.

Figure 5: modified Decoding tree in proposal 2.

# Results

## Proposal 1

The impact on coding efficiency, summarized in the following table, is not significant in any of the test configurations considered in JCTVC-H1100.

Table 1: BDR results of proposal 1.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Proposal 2

### Modified encoding process in HM6

Before reporting the results of proposal 2, we report first the results showing the impact of the encoder modification in HM6. The modification consists in checking if the MPM2 is among the candidates list after the fast HAD selection. If not, the MPM2 replaces the last candidate in this list. This modification brings a slight gain of 0.1% in AIMain and AIHE10 configurations.

Table 2: BDR results of HM6 with modified encoder.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
|  | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

### Using MPM2=Planar

The impact on coding efficiency is summarized in the following tables. A slight gain is observed for AIMain and AIHE10 configurations while for inter cases, almost no difference is observed. This slight gain comes from the encoder modification (and is of same order), which guaranties that Planar is always tested in the RDO check. Compared to the HM6 with modified encoder (cf section 5.2.1 and attached xls files), the average variations for All Intra, Random Access and Low Delay B configurations are of 0.04% Y, -0.06% U and 0.05% V.

Table 3: BDR results of proposal 2 with MPM2=Planar.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class D | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
|  | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | -0.1% | -0.2% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.2% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.3% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.4% |
| Class D | 0.1% | -0.1% | 0.4% | 0.1% | -0.6% | 0.0% |
| Class E | 0.2% | -1.4% | 0.0% | 0.1% | 0.2% | 0.2% |
| Overall | 0.1% | -0.3% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.1% | -0.3% | -0.1% | 0.0% | -0.1% | 0.1% |
| Class F | -0.2% | -0.4% | -0.1% | 0.0% | 0.3% | 0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

The Table 4 depicts different statistics gathered for AIMain and AIHE10 cases, over 25 first frames of each sequence:

- percentage of usage of MPM0, MPM1, MPM2 and Planar mode as the intra coding mode

- percentage of occurrence of Planar mode among the 3 MPMs

It can be observed that the statistics are very close between HM and proposal 2, which means that proposal 2 does not fundamentally change the intra mode distribution. In particular, Planar mode is not more often chosen in proposal 2 than in HM6.0.

Table 4: statistics related to intra mode.

|  | MPM0 | MPM1 | MPM2 | Planar mode | Planar among the 3 MPMs |
| --- | --- | --- | --- | --- | --- |
| HM | 35% | 18% | 13% | 20% | 90% |
| Proposal 2 | 33% | 16% | 19% | 19% | 100% |

### Using MPM2=DC

We have also tested the option of setting MPM2 to DC instead of Planar. At the same time, we set DC to mode 0 and Planar to mode 1. Results, depicted in next table also show no significant difference in coding efficiency.

Table 5: BDR results of proposal 2 with MPM2=DC.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class E | 0.1% | 0.1% | -0.1% | 0.1% | -0.1% | -0.2% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class F | -0.1% | 0.3% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.3% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | -0.1% | 0.2% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | 0.0% | 0.1% | 0.0% | -0.2% | 0.0% |
| Class D | 0.0% | -0.2% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | 0.0% |
| Class F | -0.1% | 0.0% | 0.1% | 0.0% | -0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class D | 0.0% | 0.7% | 0.2% | 0.0% | 0.1% | 0.5% |
| Class E | -0.1% | -0.1% | -0.2% | 0.0% | -0.4% | 0.0% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.2% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | -0.2% | 0.4% | -0.3% | 0.0% | -0.3% | 0.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The Table 6 depicts the same statistics as in previous section, but considering DC instead of Planar as the preferred mode in proposal 2. It can be again observed that the statistics are not fundamentally changed by applying proposal 2 with DC.

Table 6: statistics related to intra mode.

|  | MPM0 | MPM1 | MPM2 | Planar mode | Planar among the 3 MPMs |
| --- | --- | --- | --- | --- | --- |
| HM | 35% | 18% | 13% | 20% | 90% |
| Proposal 2 with DC | 36% | 17% | 15% | 19% | - |

# Conclusion

Two independent proposals are submitted to simplify the intra mode coding design.

Proposal 1 makes MPM0 only depending on left CU. This saves 2 to 3 checks when MPM0 is chosen, which statistically occurs for around 35% of the CUs.

Proposal 2 sets MPM2 to a pre-defined mode (namely Planar), which saves 4 checks in the worst decoding case. In addition, 6 to 8 checks are saved when MPM2 is chosen, which statistically occurs for around 20% of the CUs.

It is recommended to adopt these 2 proposals.

It is also proposed to adopt the encoder modification, which enables to get a slight gain without any added complexity.

Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

The following table depicts the occurrence of Planar mode as one of the 3 MPMs in the current HM. The statistics are established using the 25 first frames per sequence for AIMain and AIHE10 configurations. Planar mode is among the 3 MPMs in 90% of time. The smallest ratio is observed on C_BasketballDrill, with a minimum value of 84%.

Table 7: occurence of Planar mode as one of the 3 MPMs in HM6.0.

|  | AI-Main |  |  |  | AI-HE10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | 22 | 27 | 32 | 37 | 22 | 27 | 32 | 37 |  |
| A_Traffic | 88.6 | 87.8 | 88.0 | 89.0 | 88.4 | 87.8 | 87.9 | 88.8 |  |
| A_People | 92.6 | 91.6 | 90.7 | 90.0 | 92.5 | 91.6 | 90.7 | 90.0 |  |
| B_Kimono1 | 94.5 | 94.8 | 95.5 | 95.9 | 94.5 | 95.1 | 95.9 | 96.3 |  |
| B_ParkScene | 94.4 | 94.1 | 94.1 | 94.2 | 94.4 | 94.3 | 94.2 | 94.3 |  |
| B_Cactus | 92.9 | 90.5 | 89.4 | 88.9 | 92.6 | 90.3 | 89.5 | 89.0 |  |
| B_BasketballDrive | 85.5 | 84.2 | 84.7 | 86.2 | 85.2 | 84.0 | 84.3 | 86.0 |  |
| B_BQTerrace | 89.2 | 86.9 | 85.6 | 84.8 | 89.0 | 86.9 | 85.7 | 84.7 |  |
| C_BasketballDrill | 84.2 | 84.4 | 86.3 | 87.9 | 83.8 | 84.2 | 86.2 | 87.8 |  |
| C_BQMall | 88.7 | 88.1 | 88.0 | 88.1 | 88.4 | 88.2 | 88.0 | 88.0 |  |
| C_PartyScene | 93.3 | 92.5 | 91.9 | 91.1 | 93.3 | 92.5 | 92.0 | 91.1 |  |
| C_RaceHorses | 92.9 | 92.2 | 92.3 | 91.9 | 92.9 | 92.2 | 92.3 | 92.0 |  |
| D_BasketballPass | 86.0 | 87.2 | 88.1 | 88.8 | 85.7 | 87.2 | 88.0 | 88.6 |  |
| D_BQSquare | 94.9 | 94.1 | 93.6 | 92.8 | 94.6 | 94.2 | 93.7 | 92.4 |  |
| D_BlowingBubbles | 92.0 | 91.2 | 90.7 | 91.1 | 91.6 | 91.4 | 90.9 | 91.1 |  |
| D_RaceHorses | 93.1 | 92.6 | 92.5 | 91.9 | 93.0 | 92.6 | 92.5 | 91.8 |  |
| E_FourPeople | 88.7 | 88.5 | 88.4 | 89.3 | 88.3 | 88.3 | 88.4 | 89.4 |  |
| E_Johnny | 88.7 | 88.3 | 88.4 | 88.5 | 88.4 | 88.1 | 88.3 | 88.7 |  |
| E_KristenAndSara | 88.2 | 87.4 | 86.7 | 87.2 | 87.5 | 87.1 | 86.7 | 87.3 |  |
| F_SlideEditing | 88.5 | 88.6 | 88.7 | 88.7 | 88.3 | 88.5 | 88.7 | 88.7 |  |
| F_SlideShow | 89.7 | 88.7 | 87.9 | 87.3 | 89.2 | 89.0 | 87.8 | 86.8 |  |
| Average | 90.3 | 89.7 | 89.6 | 89.7 | 90.1 | 89.7 | 89.6 | 89.6 | 89.8 |
| min value | 84.2 | 84.2 | 84.7 | 84.8 | 83.8 | 84.0 | 84.3 | 84.7 | 83.8 |