| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0187 |
| --- | --- |

| Title: | Border subsampling for LM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe GISQUET Edouard FRANCOIS Stephane PAUTET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33-2 99 87 96 14 christophe.gisquet@crf.canon.fr edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution aims at simplifying the luma-based chroma (LM) prediction. In JCTVC-G129, in order to reduce the number of operations, it was proposed to downsampled by a factor of 2 the input luma and chroma signal for the OLS computation of the LM mode. However, applying this downsampling to 4x4 CUs (worst decoding case) was bringing coding efficiency loss. In the current proposal, in order to create more diversity, LM mode applies either to the even or odd samples (parity is signaled to the decoder). The technique reportedly brings 0.7% (U) and 0.8% (V) BDR gains in AIMain, 0.4% (U) and 0.6% (V) in AIHE10, 1% (U) and 0.9% (V) in RAMain, 0.8% (U) and 0.9% (V) in RAHE10, while enabling to save 22% of the multiplications in the worst decoding case.

# Introduction

## Chroma from luma prediction mode

LM mode predicts chroma samples of the current block based on corresponding reconstructed luma samples in the following linear relationship:

(1)

where  is obtained by downsampling reconstructed luma samples.

Parameters  and are derived from reconstructed samples around the current block. The equation used for calculating  (and ) is the ordinary least square (OLS) solution:

whereand indicate reconstructed chroma samples and filtered luma samples around the target block, indicates total samples number of neighboring data. In HM-6.0, I can be 4, 8 or 16 and larger blocks require more computation for calculation of  and  parameters. For a target N×N chroma block, assuming intra padding, total involved samples number is 2N.

Figure 1 : Neighboring samples used for parameters derivation in LM mode (L shape template).

## JCTVC-G129: decimation of the input samples of OLS

The contribution proposes to reduce computational complexity by subsampling and  before passing it on to OLS.

Figure 2: proposed downsampling prior to OLS computation.

Several subsampling ratio were explored, and it was finally recommended to use 1:2 subsampling for 16x16. However this does not address the 4x4 and 8x8 cases, meaning that in the worst case, no computational reduction is obtained at the decoder.

# Proposal description

## Using even or odd samples for LM mode

In the current contribution the parameters of LM mode are computed either from the even or from the odd neighboring samples (as illustrated in Figure 3 for 4x4 block). At the encoder, the two versions are tested. The best version is signaled to the decoder (2 LM codewords are used to signal if even or odd samples are used). The decoder performs the computation of the OLS parameters using the even-only or odd-only samples.

LM odd (LMo)                        LM even (LMe)

Figure 3: even or odd samples (grey) used for the OLS computation.

Chroma modes are signaled as follows (bins in bold font are bypass coded):

| Chroma mode | Codewords |
| --- | --- |
| DM | 0 |
| LMe | 100 |
| LMo | 101 |
| Planar | 110 |
| DC | 1110 |
| Vertical | 1111 |

Table 1 : List of chroma prediction modes and their codewords.

with LMe and LMo corresponding to the LM mode with even and odd samples, respectively.

In addition, when DM is one of DC, Planar or Vertical, Horizontal replaces it (see annex for exact text specification).

As depicted in the following table, this modification enables to reduce by 22% the number of multiplications used in the LM mode in the worst decoding case (all CUs are 4x4 and use LM mode).

|  | HM6.0 | Proposal |
| --- | --- | --- |
| OLS | 16+4=20 | 8+4=12 |
| Prediction | 16 | 16 |
| Total | 36 | 28 (22% less than HM) |

Table 2: comparison of the number of multiplications in LM mode between HM and proposal.

# Results

The following tables summarize the performance of the proposal, based on test conditions specified in JCTVC-H1100 but with LM mode always activated. It is observed that a gain is obtained on chroma components for all the configurations. The average gain over all configurations is of 0.6% for U and V.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -2.2% | -2.1% | 0.0% | -1.6% | -2.0% |
| Class B | 0.0% | -0.5% | -0.4% | 0.1% | -0.1% | -0.3% |
| Class C | 0.0% | -0.6% | -0.8% | 0.0% | -0.2% | -0.3% |
| Class D | 0.0% | -0.3% | -0.5% | 0.1% | -0.3% | -0.3% |
| Class E | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% |
| Overall | 0.0% | -0.7% | -0.8% | 0.0% | -0.4% | -0.6% |
|  | 0.0% | -0.7% | -0.7% | 0.0% | -0.4% | -0.6% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -2.4% | -1.8% | 0.0% | -2.5% | -2.4% |
| Class B | 0.0% | -0.7% | -0.3% | 0.0% | -0.3% | -0.3% |
| Class C | 0.1% | -0.5% | -0.7% | 0.0% | -0.4% | -0.7% |
| Class D | 0.0% | -0.5% | -0.9% | 0.1% | -0.3% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -1.0% | -0.9% | 0.0% | -0.8% | -0.9% |
|  | 0.0% | -1.0% | -0.8% | 0.0% | -0.8% | -0.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.5% | -0.1% | 0.1% | -0.7% | 0.1% |
| Class E | 0.0% | -0.5% | -1.1% | 0.0% | 0.2% | -0.6% |
| Overall | 0.0% | -0.3% | -0.4% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusion

As the proposal enables at the same time to save decoder computation operations for LM mode while providing slight coding gains for chroma, it is recommend to adopt this proposal in HM7.0.

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex – Text specification

Table 8-3 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_DC |
| Otherwise (2..34) | Intra_Angular |
| 35, 36 | Intra_FromLuma (used only for chroma) |

Table 8-4 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

HM6.0:

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 26 | 10 | 1 | X ( 0 <= X < 35 ) |
| 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 26 | 34 | 26 | 26 | 26 |
| 2 | 10 | 10 | 34 | 10 | 10 |
| 3 | 1 | 1 | 1 | 34 | 1 |
| 4 | LM | LM | LM | LM | LM |
| 5 | 0 | 26 | 10 | 1 | X |

Proposal:

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 26 | X ( 0 <= X < 35 ) |
| 0 | LMo | LMo | LMo | LMo |
| 1 | 10 | 0 | 0 | 0 |
| 2 | 1 | 10 | 1 | 1 |
| 3 | 26 | 26 | 10 | 26 |
| 4 | LMe | LMe | LMe | LMe |
| 5 | 0 | 26 | 1 | X |

with LMo corresponding to mode Intra_FromLuma 35, and LMe to Intra_FromLuma 36.

| Value of intra_chroma_pred_mode | chroma_pred_from_luma_enabled_flag = 1 |
| --- | --- |
| 5 | 0 |
| 4 | 100 |
| 0 | 1100 101 |
| 1 | 1101 |
| 2 | 1110 |
| 3 | 1111 |

##### 8.3.3.1.8	Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring samples p[ x, y ], with x, y = −1..2*nS−1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS−1.

This intra prediction mode is invoked when intraPredMode is equal to 35 or 36.

The variable T is defined as follows:

–	If intraPredMode is equal to 35, T is set equal to 0.

–	Otherwise (intraPredMode is equal to 36) , T is set equal to 1.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + Log2( nS ) − 14 )		(8-1)

pY’[ x, −1 ] = ( PLM[ 2x−1, −1 ] + 2*PLM[ 2x, −1 ] + PLM[ 2x+1, −1 ] + 2 ) >> 2, with x = 0T..nS−1	(8-2)

pY’[ −1, y ] = ( PLM[ −1, 2y ] + PLM[ −1, 2y+1 ] ) >> 1, with y = 0T..nS−1	(8-3)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x, y = 0T..nS−1	(8-4)

- Variables L, C, LL, LC and k2 are derived as follows:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		(8-5)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		(8-6)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(8-7)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(8-8)

L = y=0nS2−1pY'[−1, T+2y]+x=0nS2−1pY'[T+2x, −1]>>k3	(8-629)

C = y=0nS2−1p[−1, T+2y]+x=0nS2−1p[T+2x, −1]>>k3	(8-63)

LL = y=0nS2−1pY'[−1, T+2y]2+x=0nS2−1pY'[T+2x, −1]2>>k3	(8-64)

LC = y=0nS2−1pY'[−1, T+2y]∗p[−1, T+2y]+y=0nS2−1pY'[T+2x, −1]∗p[T+2x,−1]>>k3	(8-65)

k2 = Log2( (2*nS) >> k3 )		(8-10)