| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0188 |
| --- | --- |

| Title: | Use of chroma phase in LM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Christophe GISQUET Stephane PAUTET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33-2 99 87 96 14 edouard.francois@crf.canon.fr christophe.gisquet@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This proposal provides additional inputs related the chroma phase issue in the luma-based chroma prediction mode (LM mode), already addressed in contribution JCTVC-H0177. The issue consists in a possible misalignment of interpolated luma samples with the chroma samples. It is proposed to signal the chroma phase parameters and to adapt the luma interpolation process based on this chroma phase information.

This proposal reports further experiments based on the HM6.0. One set of filters is recommended to handle the different considered cases. The evaluation has been made on different versions of 4:2:0 sequences, generated from 4:4:4 sequences, whose chroma components have been downsampled with specific chroma phases. Using these recommended filters, it is reported that average gains around 4% on chroma components can be observed.

* the set of 4:4:4 sequences have been converted into several sets of 4:2:0 sequences with different chroma phases. One average BDR gain is measured for each set of 4:2:0 sequences.

# Introduction

In its current design, LM mode only considers 4:2:0 content with so-called ‘nominal’ luma and chroma samples position [2]. The issue consists in a possible misalignment of interpolated luma samples with the chroma samples. It is proposed to signal the chroma phase parameters and to adapt the luma interpolation process based on this chroma phase information. A solution to handle the right chroma phase in the luma-based chroma prediction mode (LM mode) has been already addressed in JCTVC-G245 and JCTVC-H0177. Chroma phase parameters are signaled in the SPS or PPS, and the luma interpolation filters used in the LM mode are adapted according to these parameters.

In this proposal, some additional experiments are presented, and a set of filters is recommended to handle different chroma phase settings in 4:2:0 content. Section 2 explains how chroma phase is handled in the current design of LM mode. Section 3 presents the proposed solution, the results of some experiments and the recommended set of filters. Section 4 describes the used test conditions and the final results obtained with the recommended set of filters.

# Chroma phase in current HM design

## Chroma phase definition

The chroma phase corresponds to the location of the chroma samples related to the luma samples. In Figure 1 different possible positions of the chroma samples related to the luma samples in a 4:2:0 content are considered. For each position, a couple of horizontal and vertical chroma phases (Cx,Cy), in unit of luma samples of a frame, is specified.

Figure 1: chroma phases in 4:2:0 format.

Chroma phase is already implicitly specified in the current HEVC specification [2], in the same way as in H.264/MPEG-4 AVC. The syntax element chroma_format_idc defines the chroma format sampling structure. In case of 4:2:0 content, corresponding to chroma_format_idc equal to 1, the nominal vertical and horizontal relative locations of luma and chroma samples in pictures correspond to (Cx=0, Cy=0.5). More generally, the nominal chroma samples positions are specified in the WD as follows:

4:2:0                                                 4:2:2                                                4:4:4

Figure 2: Nominal locations of luma and chroma samples in WD.

In addition, in H.264/MPEG-4 AVC, alternative chroma sample relative locations may be indicated in case of 4:2:0 content in Video Usability Information.

Chroma phases are also specified in the SVC extension of H.264/MPEG-4 AVC, in order to accurately control the inter-layer chroma prediction.

## Chroma phase in current LM mode design

In the LM mode, the nominal position is also explicitly considered. The interpolated luma samples RecL’[x,y] used to derive the linear model parameters and to predict the chroma signal are virtually located at the nominal position, as illustrated in Figure 3. The filters used to generate the collocated interpolated luma samples are built based on this assumption:

- To generate the left border and inner luma samples, vertical filter [1,1]/2 is used

  - RecL’[-1,y] = 	( RecL[-2,2y] + RecL[-2,2y+1] ) >> 1	      for the left border samples**

  - RecL’[x,y] = 	( RecL[2x,2y] + RecL[2x,2y+1] ) >> 1	      for the inner samples

- To generate the top border luma samples, horizontal filter [1,2,1]/4 is used

  - RecL’[x,-1] = 	( RecL[2x-1,-1] + 2 RecL[2x,-1] + RecL[2x+1,-1] + 2 ) >> 2

Figure 3: position of luma-chroma samples in LM mode.

**for the LCU border, the immediately left border samples are considered (this enables reducing the storage needs).

# Proposal: LM mode generalization with chroma phase

As already described in JCTVC-G245, it is suggested to generalize the LM mode to other cases than the nominal chroma samples positioning. The 4 cases depicted in Figure 1 are considered. In case of 4:2:0 content, the chroma phase relative to the luma samples position is signaled in the Sequence Parameter Set, as described in section 3.1. Depending on the chroma phase, different filters are used to generate the interpolated luma samples used to predict the chroma samples.

## Syntax changes

It is first proposed to insert syntax elements related to chroma phases into the Sequence Parameter Set or in the Picture Parameter set.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 1 ¦ ¦ chroma_format_idc = = 2 ) { |  |
| chroma_phase_x | u(1) |
| } |  |
| if( chroma_format_idc = = 1 ) { |  |
| chroma_phase_y | u(1) |
| } |  |
| max_temporal_layers_minus1 | u(3) |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 1 ¦ ¦ chroma_format_idc = = 2 ) { |  |
| chroma_phase_x | u(1) |
| } |  |
| if( chroma_format_idc = = 1 ) { |  |
| chroma_phase_y | u(1) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

chroma_phase_x specifies the horizontal phase shift of the chroma components in units of half luma samples of a frame (possible values 0,1). If chroma_phase_x is not present, it shall be inferred to be equal to 0.

chroma_phase_y specifies the vertical phase shift of the chroma components in units of half luma samples of a frame (possible values 0,1). If chroma_phase_y is not present, it shall be inferred to be equal to 0.

## Evaluation of different sets of filters for the luma signal interpolation process

Basically, a generic bilinear filtering has been considered for generating the interpolated top, left and inner luma signal [JCTVC-G245].

Let (Cx,Cy) being defined as (chroma_phase_x, chroma_phase_y).

- For the left border,

  - RecL’[-1,y] = 	( RecL[-1,2y] + Cy.RecL[-1,2y+1] ) >> Cy

- For the inner samples,

  - RecL’[x,y] = 	( RecL[2x,2y] + Cx.RecL[2x+1,2y] + 
		  Cy.RecL[2x,2y+1] + Cx.Cy.RecL[2x+1,2y+1] ) >> (Cx+Cy)

- For the top border,

  - RecL’[x,-1] = 	( RecL[2x-1,-1] + (2+Cx).RecL[2x,-1] + 
		  (2+Cx).RecL[2x+1,-1] ) + Cx.RecL[2x+2,-1] ) >> (2+Cx)

However some concerns about the added complexity were raised at the 7th HEVC meeting. In JCTVC-H0177, a limited set of luma interpolation filters has been identified and recommended, as depicted in the following table.

| Filter | (Cx,Cy) | Top | Left | Inner |
| --- | --- | --- | --- | --- |
| 0 | (0, 0) | [1,2,1]/4 horiz | [1,2,1]/4 vertic | [1,2,1]/4 horiz |
| 1 | (0.5, 0) | [1,1]/2 horiz | [1,2,1]/4 vertic | [1,1]/2 horiz |
| 2 (default HM design) | (0, 0.5) | [1,2,1]/4 horiz | [1,1]/2 vertic | [1,1]/2 vertic |
| 3 | (0.5, 0.5) | [1,1,1,1]/4 horiz | [1,1,1,1]/4 horiz | 1,11,14 bidim |

It has been observed that these filters are relevant even for chroma phases not explicitly fitting them [JCTVC-H0177]. For instance, for a set of sequences with chroma phase (Cx=0.75,Cy=0), 4% gain in chroma and 0.2% in luma are obtained using the interpolation filter 1, corresponding to chroma phase (Cx=0.5,Cy=0). This shows that it is not necessary to specify as many filters as there are chroma phases configurations.

In the current contribution, we have additionally tested the configuration (Cx=0,Cy=0.25), which can correspond to a 4:2:0 interlaced case, as depicted in the figure below extracted from the H.264/AVC specification.

Figure 4: Nominal sampling locations of 4:2:0 samples in top and bottom fields.

For this case, we have tested the bi-linear filter solution mentioned initially [JCTVC-G245]. The following results are observed, using the HM as anchor.

|  | IIHE |  |  | IILC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class O | 0.0% | 0.2% | 0.1% | 0.0% | 0.4% | 0.2% |
| Class P | 0.0% | -0.3% | -0.9% | 0.0% | -0.3% | -0.8% |
| All | 0.0% | -0.1% | -0.4% | 0.0% | 0.1% | -0.3% |

In fact, the HM filter performs almost as well as the bi-linear filter in this case. This confirms that it is not necessary to use many different filters, for the various chroma phases to be considered. It is possible to keep a limited set of 4 filters to handle various cases.

In this proposal, we have further simplified Filter 4. New evaluation in HM6 have shown that using [1,1]/2 instead of [1,1,1,1]/4 for filtering top and left luma samples, but keeping the bidimensional filter [1,1 / 1,1]/4 for inner samples, leads to a minor loss (an average of 0% Y, 0.3% U, 0.1% V on considered test sequences, see ‘Results’ section) while simplifying the design.

Finally the news proposed filters are the following:

| Chroma format | Filter | (Cx,Cy) | Top | Left | Inner |
| --- | --- | --- | --- | --- | --- |
| 4:2:0 | 0 | (0, 0) | [1,2,1]/4 horiz | [1,2,1]/4 vertic | [1,2,1]/4 horiz |
|  | 1 | (0.5, 0) | [1,1]/2 horiz | [1,2,1]/4 vertic | [1,1]/2 horiz |
|  | 2 (default HM) | (0, 0.5) | [1,2,1]/4 horiz | [1,1]/2 vertic | [1,1]/2 vertic |
|  | 3 | (0.5, 0.5) | [1,1]/2 horiz | [1,1]/2 vertic | 1,11,14 bidim |
| 4:2:2 | 0 | (0, 0) | [1,2,1]/4 horiz | No filt | [1,2,1]/4 horiz |
|  | 1 | (0.5, 0) | [1,1]/2 horiz | No filt | [1,1]/2 horiz |

# Results

## Test conditions

To evaluate the proposed solution, new test sequences with specific chroma phases have been used. A set of 8 native RGB 4:4:4 sequences, grouped into 2 classes, are used:

- Class P: 1080p sequences used for the AVC professional extension (man_in_restaurant, playing_cards, rolling_tomatoes, table_setting), [3]. Each sequence is made of 60 frames.

- Class S: 2 1080p sequences, (ducks_takeoff, 251 frames, in_to_tree, 224 frames), 2 2160p (3940x2160) sequences, (crowd_run, 61 frames, park_joy, 61 frames) from SVT [4].

Each sequence has then been converted into 4 different YUV 4:2:0 versions, corresponding to 4 different horizontal/vertical chroma phases (Cx=0,Cy=0), (Cx=0.5,Cy=0), (Cx=0,Cy=0.5 – nominal case), (Cx=0.5,Cy=0.5). RGB 4:4:4 to YUV 4:2:0 conversion processes are described in JCTVC-H0177.

The related conversion tools (RGB 4:4:4 to YUV 4:4:4, YUV 4:4:4 to YUV 4:2:0) are provided within this contribution archive.

## Results

The modified HM6.0 software is provided in attachment to this contribution (file JCTVC-I0xxx_HM6.0_LM-with-ChromaPhase_Canon.zip). The following define enables to use chroma phase in LM mode:

LM_CHRPHASE_CXCY

Additionally, 2 defines CHRPHASE_CX and CHRPHASE_CY are used to specify the horizontal and vertical chroma phase of the HM.

The following table summarizes the results (BD-rate variations in %) obtained with the filters provided in section 3Error: Reference source not found. For each chroma phase case, the corresponding modified HM is compared to the default HM.

| Cx=0,Cy=0 | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class O | -0.1% | -1.8% | -1.2% | -0.1% | -1.6% | -0.7% |
| Class P | 0.0% | -1.2% | -2.8% | -0.1% | -1.3% | -2.7% |
| Overall | -0.1% | -1.5% | -2.0% | -0.1% | -1.5% | -1.7% |

| Cx=0.5,Cy=0 | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class O | -0.2% | -3.5% | -1.8% | -0.2% | -3.5% | -1.6% |
| Class P | 0.0% | -1.3% | -3.9% | 0.0% | -1.4% | -4.0% |
| Overall | -0.1% | -2.4% | -2.8% | -0.1% | -2.4% | -2.8% |

| Cx=0.5,Cy=0.5 | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class O | -0.3% | -3.8% | -2.7% | -0.2% | -2.1% | -1.2% |
| Class P | 0.0% | -0.5% | -1.9% | 0.0% | -0.5% | -1.8% |
| Overall | -0.1% | -2.2% | -2.3% | -0.1% | -1.3% | -1.5% |

# Conclusion

It has been shown that taking into account the chroma phase in the LM mode may provide a noticeable improvement of the performance of the LM mode. In addition, it has been shown that a limited set of 4 related luma interpolation filters was sufficient to handle various cases.

We suggest the JCTVC group to consider these filters for adoption in the HM.

# References

- Edouard FRANCOIS , Christophe GISQUET, Stephane PAUTET, “Non-CE6a: Use of chroma phase in LM mode” JCTVC-H0177, 8th Meeting: San Jose, USA, 1–10 February, 2012.

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand., “WD4: Working Draft 4 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1103, 8th Meeting: San Jose, USA, 1–10 February, 2012.

- Tom McMahon., “Film-Originated Test Sequences”, Joint Video Team (JVT) of ISO/IEC JPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6), JVT-J042, 10th Meeting: Waikoloa, Hawaii, USA, 8-12 December, 2003.

- “The SVT High Definition Multi Format Test Set”, Feb.2006, http://media.xiph.org/svt/SVT_MultiFormat_v10.pdf

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).