| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0189 |
| --- | --- |

| Title: | On APS referring and updating |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nael OUEDRAOGO Edouard FRANCOIS Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33-2 99 87 96 14 nael.ouedraogo@crf.canon.fr edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution relates to APS. APS has been introduced to embed parameters related to Quantization matrices, deblocking filter, SAO and ALF, expected to vary more frequently than the parameters present in the PPS. In this contribution, four different proposals are submitted. The first proposal aims at enabling to refer to multiple APSs within a slice by using a direct reference for each coding tool. The second proposal, relating to the APS update, consists in adding a new flag in the APS NALU to indicate if the APS is fully reset or not. The third proposal relates to the possibility to give two levels of priority to the APS NALUs, based on the flag nal_ref_flag value. The fourth proposal defines a reset mechanism of the APS buffer for easier management of APSs in random access scenarios.

# Introduction

APS has been introduced to embed parameters related to pictures and that are expected to vary more frequently than the parameters present in the PPS. Currently, APS may contain parameters related to the Quantization scaling matrices QSM, the deblocking filter DBF (on/off flag, strength parameters), the SAO (segmentation and offsets) and the ALF (segmentation and filters coefficients). APS validity is not limited to a single picture and one APS may be valid for several pictures. An APS buffer is used (in the current design, up to 64 APSs can be managed), and each slice can refer to one of the APSs of this APS buffer.

To convey the data related to the APS, an APS NALU has been specified. The APS NALU contains an APS id, which enables to create a new APS (if it does not exist yet) or to overwrite an existing APS, with the parameters contained in the APS NALU.

Even if the APS is rather a picture-level set, different slices in a same picture could in principle refer to different APSs: each slice may contain an APS_id different from other slices of the same picture.

It is also possible to signal the loop filter parameters directly in the slice, instead of referring to an APS. For DBF and ALF, the signaling can be made in the Slice Header. For SAO, the parameters can be interleaved in the Slice Data (parameters signaled into each LCU). It can be expected that interleaving would also apply to ALF parameters.

These different possibilities give a lot of flexibility, but also make the signaling more complex and may cause some inefficiency. The following table summarizes the multiple syntax elements related to the 4 tools concerned by the APS.

|  | QM | DBF |
| --- | --- | --- |
| SPS | scaling_list_enable_flag | deblocking_filter_in_aps_enabled_flag |
| PPS |  | deblocking_filter_control_present_flag |
| APS | aps_scaling_list_data_present_flag | aps_deblocking_filter_flag if(aps_deblocking_filter_flag) disable_deblocking_filter_flag |
| Slice |  | if( deblocking_filter_control_present_flag ) if( deblocking_filter_in_aps_enabled_flag ) inherit_dbl_params_from_aps_flag if( !inherit_dbl_params_from_aps_flag ) disable_deblocking_filter_flag |

|  | SAO | ALF |
| --- | --- | --- |
| SPS | sample_adaptive_offset_enabled_flag | adaptive_loop_filter_enabled_flag if(adaptive_loop_filter_enabled_flag) alf_coef_in_slice_flag |
| PPS |  |  |
| APS | aps_sao_interleaving_flag* if( !aps_sao_interleaving_flag ) aps_sample_adaptive_offset_flag** if( aps_sample_adaptive_offset_flag ) aps_sao_param( ) | aps_adaptive_loop_filter_flag*** if( aps_adaptive_loop_filter_flag ) alf_param( ) |
| Slice | if( sample_adaptive_offset_enabled_flag ) slice_sao_interleaving_flag* slice_sample_adaptive_offset_flag** | if( adaptive_loop_filter_enabled_flag ) slice_adaptive_loop_filter_flag*** |

* 	slice_sao_interleaving_flag must be equal to aps_sao_interleaving_flag when present

** 	slice_sample_adaptive_offset_flag must be equal to aps_sample_adaptive_offset_flag when present

***	slice_adaptive_loop_filter_flag must be equal to aps_adaptive_loop_filter_flag when present

The following issues can be mentioned:

- The current syntax is rather complex and implies many different syntax elements, partly redundant.

- It is not possible for a slice to refer to different APSs. Several proposals have addressed this issue [JCTVC-G1026, H0505, H0381, H0069]. So far no solution has been adopted regarding the referring to multiple APSs.

- If for a given picture, only the SAO parameters of a first existing APS should be changed, it is necessary to create a new APS or to overwrite an APS, mostly redundant with the first existing APS.

- There is a potential issue for random access scenarios, to identify at random access points the valid APSs that should be referred. Because the APSs are expected to change much more frequently than the SPS or PPS, transmitting them out of band does not look as a satisfying solution.

- All APSs are considered of same priority, though it is highly probable that some APSs, being potentially relevant for many pictures, are of higher priority than other ones.

In this contribution, four proposals are submitted to address these issues:

- The first proposal, described in section 2, aims at enabling to refer to multiple APSs within a slice by using a direct reference for each coding tool.

- The second proposal, described in section 3,  relating  to the APS update, consists in adding a new flag in the APS NALU to indicate if the APS is fully reset or not.

- The third proposal, described in section 4, relates to the possibility to give two levels of priority to the APS NALUs, based on the flag nal_ref_flag value.

- The fourth proposal, described in section 5, defines a reset mechanism of the APS buffer for improved error resilience, especially in random access scenarios.

# Proposal 1 – referring to multiple APS

The first proposal aims at enabling to refer in the slice to multiple APSs of the APSs buffer. The proposal is very close to the principle presented in H0505. The main difference is that the APS referring is made in the slice header. Each slice may refer, for each tool (i.e. QSM, DBF, SAO and ALF), to one APS. For each tool, an identifier is signaled. When this identifier is equal to 0, this means that the set of parameters, if any, is not defined from an APS but in the slice NAL unit. When it is greater than 0, the parameters from the APS of aps_id equal to the identifier minus 1 are used.

## Syntax & semantic

The proposed changes are depicted in the following table.

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| […] |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sample_adaptive_offset_flag | u(1) |
| […] |  |
| } |  |
| if( scaling_list_enable_flag) |  |
| scaling_list_param_id | ue(v) |
| if( deblocking_filter_in_aps_enabled_flag) |  |
| deblocking_filter _ param_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag) |  |
| sao_ param_id | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_ param_id | ue(v) |
| […] |  |
| } |  |
| […] |  |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_param_id = = 0) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| […] |  |
| } |  |
| […] |  |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_param_id = = 0 ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && alf_param_id > 0) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| […] |  |
| } |  |

The semantic of the new syntax elements is defined as follows:

scaling_list_param_id equal to 0 specifies that the scaling list parameters are the default ones. Otherwise scaling list parameters from APS of id (scaling_list_param_id - 1) shall be used. When scaling_list _param_id  is not present, it is inferred to be equal to 0.

deblocking_filter_ param_id equal to 0 specifies that deblocking parameters in the slice header shall be used. Otherwise deblocking parameters from the APS of id (deblocking_filter _param_id - 1) shall be used. When deblocking_filter_param_id is not present, it is inferred to be equal to 0.

sao_param_id equal to 0 specifies that SAO parameters interleaved in the slice shall be used. Otherwise SAO parameters from the APS of id (sao_param_id - 1) shall be used. When sao_param_id  is not present, it is inferred to be equal to 0.

alf_param_id equal to 0 specifies that ALF parameters in the slice header shall be used. Otherwise ALF parameters from the APS of id (alf_param_id - 1) shall be used. When alf_param_id  is not present, it is inferred to be equal to 0.

## Benefits

The proposed solution firstly simplifies the syntax. The following syntax elements can be removed:

- inherit_deblocking_params_from_aps_flag (which indicates whether DBF parameters are defined in APS),

- slice_sao_interleaving_flag and aps_sao_interleaving_flag (which both indicate whether LCU based representation is used for SAO parameters),

- alf_coef_in_slice_flag (which specifies if ALF parameters are in slice or APS NAL unit).

In addition, it gives the flexibility for each slice to refer to multiple APSs within a slice, with a simple referring mechanism.

# Proposal 2 – APS update

A Parameter Set NAL unit which has a same identifier as a previous one is classically replacing entirely the previous Parameter Set NAL unit parameters. APS NAL unit may contain a large amount of data that is likely to be stable during the video stream typically QSM.  Moreover, the number of APS NAL units may be quite important in the sequence and thus the value of aps_id is likely to loop. For instance, a previous APS NAL unit defines QSM, DBF and SAO parameters, and a new APS with a same aps_id, contains only SAO parameters.

This proposal adds aps_reset_flag syntax element to indicate how the new APS updates the previous APS NAL unit. If aps_reset_flag is equal to 1, only parameters defined in the new APS NAL unit replace the ones defined in the previous APS NAL unit. Thus, for the previous example, the updated APS contains QSM and DBF of previous APS and SAO parameters of the new APS. Consequently, only necessary parameters have been coded in the APS NAL unit which save bitrate.

In particular applications, conditional replacement of previous APS parameter is not convenient. For this reasons, if aps_reset_flag is equal to 0, then all parameters of previous APS are replaced by the new one. Thus, the updated APS NAL unit of the example contains only SAO parameters. To maintain the QSM and DBF parameters of the previous APS in the APS buffer, the new APS must either duplicate the QSM and DBF parameters or employ a different aps_id.

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_reset_flag | u(1) |
| aps_scaling_list_data_present_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| aps_deblocking_filter_flag | u(1) |
| if(aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| aps_sample_adaptive_offset_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ) |  |
| aps_sao_param( ) |  |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_adaptive_loop_filter_flag ) |  |
| alf_param( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Benefits

The benefit of this proposal is a simple method to update parameters of APS NAL unit. In addition, the conditional replacement could be easily deactivated when robustness against errors is required.

# Proposal 3 – APS for non-reference pictures

The Figure 1 shows an example of hierarchical coding, with a number of pictures specified as non-reference pictures (I5, I6, I7 and I8). In this example, different APSs are used for each hierarchical level. Pictures I0 and I1 of lower level 0 use their own APS. Then a different APS is used for each level (a number of filter parameters such as SAO parameters strongly depend on the quantization parameter and it looks logixal to use at least different APSs for different temporal levels). In the example, the 4 non-reference pictures use a same APS (APS of id 4), that is not referred by any other picture. Loop filters (SAO, ALF, DBF) for non-reference pictures is actually equivalent to a post filtering, since non-reference pictures, by definition, are not used as reference for coding other pictures.

Figure 1: example of APS usage in hierarchical pictures structure.

This leads to the following question: is it necessary to embed all the ALF, SAO related syntax in the main stream for these non-reference pictures? In case for instance of streaming applications with limited bandwidth, it should be possible to drop these data when bandwidth does not allow to transmit all data.

In the current design, nal_ref_flag of APS NAL units is always set to 1. The proposed solution consists in using nal_ref_flag to indicate if the APS is ‘dropable’ or not.

It is proposed to use the following rules for defining nal_ref_flag value of APS NAL units: When nal_ref_flag of an APS is equal to 0, only non-reference slice can refer to this APS. Otherwise, any slice can refer to an APS with nal_ref_flag equal to 1.

Hence, nal_ref_flag gives the priority of the APS: APS with nal_ref_flag equal to 0 are of low priority, and can be dropped in priority if bandwidth is limited. The impact of their loss will be only for non-reference pictures.

## Benefits

This solution enables to define APS with 2 levels of priority. APS of low importance, for instance because they are only use by non-reference pictures, can then be easily detected and possibly dropped in case of limited bandwidth.

# Proposal 4 – APS buffer reset

Advanced HEVC encoders could use several APS NAL units during a video sequence. In video streaming, Parameter Sets (SPS, PPS) are generally transmitted out of band on a reliable channel to ensure that they are correctly received by the decoder.  Indeed, data stored in SPS or PPS is essential for the decoding of the frames. In addition, the content of SPS and PPS is generally very stable during one video sequence. In most cases, one SPS and one PPS NAL unit is sufficient for the whole video sequence.

With introduction of APS NAL unit in HEVC, these principles have changed. APS are not all strictly indispensable for the decoding process. Indeed, in case of loss of an APS NAL unit, the whole frame is decodable but decoding error is introduced with visual artifacts. Moreover, the number of APS NAL units could be important. For instance, at least one APS NAL unit could be used per frame. For these reasons, sending APS NAL units out of band on a reliable channel might be not realistic or practically not employed. In-band transmission of APS NAL units is thus likely to be used.

Figure 2: APS Referencing and random access issue

When accessing at a resynchronization frame in random position in the bitstream, the state of the APS buffer has to be restored such that all APS NAL units that will be used by subsequent slices are available and decoded. For instance, the Figure 2 illustrates a set of a slice and APS NAL units and orange arrows indicate that the slice refers to one APS NAL unit. In that example, the first slice NAL unit refers to the the APS NAL unit #1. When accessing to the IDR frame identified with the blue arrow, the decoder should have in his APS buffer the APS NAL unit #1.

It seems too complex to parse all slice headers that follow the random access frame to identify which APS NAL units are employed and need to be decoded and stored in the APS buffer. In addition, parsing the whole bitstream for identifying valid APS NAL unit is probably not very convenient when the random access point is far from the beginning of the sequence.

Consequently, we propose to refresh periodically the APS buffer to enable use of simple method to randomly access to one picture. Typically, the APS buffer may be refreshed at each IDR or CDR frame or with a predetermined buffering period stored in SPS or PPS NAL unit. In practical, it is proposed to add two new syntax elements in the Sequence Parameter Sets as described in the table below:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| aps_buffer_reset_at_idr_cdr_flag | u(1) |
| if(aps_buffer_reset_at_idr_cdr_flag = = 0 ) |  |
| aps_buffer_period | ue(v) |
| […] |  |

aps_buffer_reset_at_idr_cdr_flag equal to 1indicates that the APS buffer is refreshed at each IDR frame. When equal to 0, the APS buffer refresh process depends on aps_buffer_period value.

aps_buffer_period equal to 0 indicates that APS buffer should not be reset. Any APS NAL unit may be referred to in one VCL NAL unit. When greater or equal to 1, it indicate the maximum number of frames for which each APS NAL unit is valid.

When decoding an IDR slice NAL unit and aps_buffer_reset_at_idr_cdr_flag is equal to 1, all prior APS NAL units should be considered as deactivated and should not be referred to by subsequent NAL units. For instance, the Figure 3 represents the video stream of Figure 2 when aps_buffer_reset_at_idr_cdr_flag: since APS buffer is reset at each IDR, the APS NAL unit #2 is inserted just before the slice that refers it. In that case, it is ensure that needed APS NAL unit will be available after the random access point.

When Figure 2 is equal to 0 and aps_buffer_period is equal to 0, all APS NAL units active and non-active may be refer by any NAL unit of the bitstream. In that case, all APS NAL unit sent prior to the random accessed IDR in the Figure 2 should be stored in the APS buffer.

Figure 3: APS referencing when  aps_buffer_reset_at_idr_cdr_flag is equal to 1.

When aps_buffer_reset_at_idr_cdr_flag is equal to 0 and aps_buffer_period is greater or equal to 1, APS NAL unit activation is limited to a maximum number of frames equal to aps_buffer_period. Consequently, one NAL unit cannot refer to an APS NAL unit that was available before N lasts access units (where N is equal to aps_buffer_period). In that case, the decode needs to store all APS NAL unit prior to the random access point and within the aps_buffer_period interval as depicted in the Figure 4.

Figure 4: APS referencing when aps_buffer_reset_at_idr_cdr_flag is equal to 0 and aps_buffer_period greater than 0.

## Benefits

The proposed solution offers a simple mechanism to identify thee set of valid APSs for each random access point. This avoids complex solutions requiring for instance full parsing of the stream to identify the valid APSs, or the usage of out-of-band transmission of the APSs.

# Conclusion

This contributions aims at proposing simple mechanisms for multiple APS referring and for better robustness to APSs loss.

The first proposal consists in using explicit identifiers in the slice header referring for each tool. These identifiers enable to simply signal the APS to be used for each considered tool. This solution can save several syntax elements from the current design and give more flexibility by allowing to refer to multiple APS within a slice.

The second proposal relates to the APS update mechanism. It specifies a new syntax element in the APS NALU to enable/disable the APS reset. In addition, the conditional replacement could be easily deactivated when robustness against errors is required.

The third proposal consists in using the nal_ref_flag of the APS NALU to indicate the priority level of the APS. This enables to identify APSs that are of low priority and that can be preferably dropped if available bandwidth is too limited.

The fourth proposal aims at addressing the issue of APSs availability in random access scenarios. The proposed mechanism is based on the possibility to regularly reset the APS buffer, either for each IDR, or with a given period specified in a new syntax element inserted in the SPS.

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).