| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0193 M24432 |
| --- | --- |

| Title: | Non-CE1: LCU SAO Enable Flag Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Texas Instruments Inc., USA Do-Kyoung Kwon Texas Instruments Inc., USA In Suk Chong Qualcomm Inc., USA Marta Karczewicz Qualcomm Inc., USA | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-214-480-1562 wskim@ti.com +1-214-480-1479 d-kwon@ti.com +1-858-845-5477 ichong@qualcomm.com +1-858-658-5673 martak@qualcomm.com |
| Source: | Texas Instruments Inc. Qualcomm Inc. |  |  |

_____________________________

# Abstract

In HM-6.0, SAO parameter can be signaled for each LCU, and SAO can be turned off for each LCU by setting SAO type as 0 for each color component. This contribution proposes to insert a new code in each LCU to enable or disable SAO for three color components, which is decoupled from SAO type. It is also proposed not to signal a merge flag when SAO is turned off in a neighboring LCU.

Experimental results reportedly show that the proposed method can improve coding efficiency by 1.5%, 0.8%, and 0.5% for LCU size of 16, 32, and 64, respectively, for luma component compared to the CE1 anchor in case of low-delay B main, when interleaving mode is used. Similar or larger gain is observed for chroma components.

# Introduction

In HM-6.0 [1], SAO data can be copied from neighboring LCUs (either left or upper LCU) by setting merge flags, or new SAO parameters (i.e., sao_type_idx and sao_offset) can be sent to the decoder. And, sao_type_idx can be set to 0 to signal SAO off, 1-4 to signal edge offset (EO), or 5 to signal band offset (BO).

In this proposal, we propose to decouple SAO on/off signaling from sao_type_idx for each color component for each LCU. If SAO is off for a color component in a LCU, a LCU SAO enable flag is set to 0, and no other information is sent. Otherwise, a LCU SAO enable flag is set to 1, and other syntax elements such as merge flags or SAO parameters are sent. Now that merge flag is sent only when current LCU’s SAO is enabled, merge flag is not signaled when SAO is turned off in a neighboring LCU.

These schemes are applied to both LCU interleaved mode and APS mode. Furthermore, for LCU interleaved mode, LCU SAO enable flags of all color components are jointly coded.

# Proposed Method

2.1 Interleaving mode case

2.1.1 Combined SAO flag for Y/Cb/Cr

At the encoder side, using the RDO scheme in HM-6.0, it is determined for each color component to turn on or off SAO for each LCU. This generates three flags for three color components. A combination index is generated using these flags as:

Combination index = (lcu_sao_enable_flagY<<2) + (lcu_sao_enable_flagCb<<1) + lcu_sao_enable_flagCr,

where flag value 1 implies SAO on, and flag value 0 implies SAO off for each color component.

Then, a code index is assigned to signal a combination of these flags. Table 1 shows all the possible combinations of flags, corresponding combination index, and the mapping to the code index, which is to be coded into a bitstream. The mapping table is generated based on the statistics of test sequences according to the common test condition [2] using LCU interleaving mode. After coding those SAO on/off flags, other related syntax elements such as merge flags (i.e., merge_left and/or merge_up flag) or SAO parameters (i.e., sao_type_idx and sao_offset) are coded.

Table 1. Mapping from LCU SAO enable flags to a lcu_sao_enable_idx index.

| LCU SAO enable flag |  |  | Combination index | Code index (lcu_sao_enable_idx) |
| --- | --- | --- | --- | --- |
| Y | Cb | Cr |  |  |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 5 |
| 0 | 1 | 0 | 2 | 6 |
| 0 | 1 | 1 | 3 | 7 |
| 1 | 0 | 0 | 4 | 1 |
| 1 | 0 | 1 | 5 | 2 |
| 1 | 1 | 0 | 6 | 3 |
| 1 | 1 | 1 | 7 | 4 |

We propose to apply this method for LCU interleaved SAO, where SAO parameter of each LCU is coded as part of slice data using CABAC. LCU_SAO_enable_idx is coded prior to any other SAO parameters. Unary binarization is used to code LCU_SAO_enable_idx.

At the decoder side, LCU_SAO_enable_idx is decoded and SAO on/off flag for each color component is interpreted from it. If the flag is 0, no other SAO parameters are decoded for that color component. If it is 1, SAO parameters for that color component are further decoded.

As SAO off is signaled using LCU_SAO_enable_idx, the current SAO type 0 becomes unnecessary. Therefore, it is proposed to change the SAO type table as in Table 2, where SAO off is removed and values of sao_type_idx for EO and BO are reduced by 1.

Table 2. Proposed SAO type table.

| - sao_type_idx | - SAO type |
| --- | --- |
| - 0 | - 1D 0-degree edge |
| - 1 | - 1D 90-degree edge |
| - 2 | - 1D 135-degree edge |
| - 3 | - 1D 45-degree edge |
| - 4 | - Band |

2.1.2 Efficient Merge Flag Signaling

If an upper or left LCU is available, SAO parameters can be copied from those of upper or left LCU using merge up or merge left flag. When SAO is turned off in the upper or left LCU, it becomes not necessary to copy SAO parameters from it.

By use of LCU SAO enable flag as proposed in section 2.1.1, unnecessary signaling of merge flags can be effectively removed by checking neighboring LCU’s LCU SAO enable flag. For each color component, SAO parameters or merge flags are signaled only when LCU SAO enable flag is equal to 1. In this case, if upper/left LCU’s SAO enable flag is 0, we propose not to send merge up/left flag.

2.2 APS mode case

Now that sao_enable_flag is decoupled from sao_type_idx in the interleaving mode, we also want to apply unified scheme both in APS and LCU interleaved modes. In APS mode, we decouple SAO enable flag for each color component, then merge_up flag or SAO parameters are sent only when SAO enable flag is on.

2.2.1 Decoupling SAO flag for Y/Cb/Cr

At the encoder side, using the RDO scheme in HM-6.0, it is determined for each color component to turn on or off SAO for each LCU. This generates three flags for three color components (lcu_sao_enable_flagY, lcu_sao_enable_flagCb, lcu_sao_enable_flagCr).

After coding those lcu_sao_enable_flag, if the flag is on, other related syntax elements such as merge flags (i.e., merge_up flag) or SAO parameters (i.e., sao_type_idx and sao_offset) are coded for each color component.

2.2.2 Efficient Merge Flag Signaling

If an upper LCU is available, SAO parameters can be copied from those of upper LCU using merge-up scheme. When SAO is turned off in the upper LCU, it becomes not necessary to copy SAO parameters from it.

By use of LCU SAO enable flag as proposed in section 2.2.1, unnecessary signaling of a merge flag can be effectively removed by checking upper LCU’s sao_enable_flag. For each color component, SAO parameters or merge flag are signaled only when lcu_sao_enable_flag is equal to 1. In this case, if upper LCU’s SAO flag is 0, we propose not to send merge up flag.

# Experimental Results

To evaluate the coding efficiency of the proposed method, the CE1 common test condition [3] is used for the interleaving mode, and the common test condition in [2] with changing SAO mode to APS mode for all configurations (i.e. CE1 Test 0.1) is used for the APS mode.

Results for the interleaving mode are summarized in Table 3. Compared to CE1 anchor, bitrate reduction of 1.5%, 0.8%, and 0.5% for LCU size of 16, 32, and 64, respectively, is achieved for luma component. Similar or larger gains are observed for chroma components. Test results for all other configurations with LCU size 16/32/64 can be found in the attached spread sheets.

Larger gain in smaller LCU size is observed. This is because SAO parameter bitrate becomes burdensome when LCU size is small, resulting in more LCUs not using SAO. In this case, the proposed method can improve coding gain by efficiently signalling SAO on/off combination of color components.

Note that the harmonization results of the proposed method with other SAO proposals can be found in the CE1 report.

Results for the APS mode is summarized in Table 4 for LCU size of 64x64. There is no difference at all. Therefore, it is beneficial to apply the proposed scheme to unify syntax and semantics for both interleaving mode and APS mode.

Table 3. BD-rate results of the proposed method in the interleaving mode compared to CE1 anchor.

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -0.9% | -0.5% | -0.8% | -0.9% | -0.5% |
| Class C | -0.6% | -0.5% | -0.4% | -0.7% | -0.7% | -0.3% |
| Class D | -0.9% | -0.8% | -0.8% | -0.9% | -0.1% | 0.1% |
| Class E | -4.2% | -3.4% | -3.6% | -4.1% | -3.5% | -3.6% |
| Overall | -1.5% | -1.3% | -1.2% | -1.4% | -1.2% | -0.9% |
|  | -1.5% | -1.3% | -1.2% | -1.4% | -1.2% | -0.9% |
| Class F | -1.7% | -2.0% | -1.6% | -1.8% | -1.0% | -1.2% |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 106% |  |  | 102% |  |  |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.5% | -0.8% | -0.3% | -0.3% | 0.0% |
| Class C | -0.4% | -0.5% | -0.6% | -0.4% | -0.4% | -0.3% |
| Class D | -0.7% | -0.7% | -0.7% | -0.8% | -0.6% | -0.6% |
| Class E | -2.4% | -2.4% | -2.5% | -2.3% | -1.9% | -2.3% |
| Overall | -0.8% | -0.9% | -1.0% | -0.8% | -0.7% | -0.7% |
|  | -0.8% | -0.9% | -1.0% | -0.8% | -0.7% | -0.7% |
| Class F | -0.9% | -0.5% | -1.3% | -0.8% | -0.7% | -0.6% |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.7% | -0.8% | -0.2% | -0.3% | 0.0% |
| Class C | -0.2% | -0.6% | -0.4% | -0.3% | -0.3% | -0.3% |
| Class D | -0.4% | -0.5% | -0.9% | -0.4% | -0.6% | -0.4% |
| Class E | -1.6% | -2.2% | -1.5% | -1.5% | -1.1% | -1.4% |
| Overall | -0.5% | -0.9% | -0.9% | -0.5% | -0.5% | -0.5% |
|  | -0.5% | -0.9% | -0.9% | -0.5% | -0.6% | -0.6% |
| Class F | -0.5% | -0.4% | -0.7% | -0.4% | 0.2% | -0.6% |
| Enc Time[%] | 92% |  |  | 97% |  |  |
| Dec Time[%] | 85% |  |  | 97% |  |  |

Table 4. BD-rate results of the proposed method in the APS mode compared to CE1 Test 0.1.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

# Conclusion

It is proposed to signal a new codeword to efficiently turn on or off SAO for each color component for each LCU. The proposed method is implemented for both interleaving and APS mode cases. For the interleaving mode, experimental results show coding efficiency improvement of 1.5%, 0.8%, and 0.5% for LCU size of 16, 32, and 64, respectively, for luma component. Similar or larger gain is observed for chroma components. For the APS mode, no difference is observed, which makes the proposed method beneficial for both modes with the unified syntax and semantics.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1003, Feb. 2012.

- F. Bossen, “Common test conditions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1100, Feb. 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb., 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD Text Changes (changes marked in yellow)

### 7.3.4 Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) { |  |
| lcu_sao_enable_idx | ae(v) |
| if(LcuSaoEnableFlag[ 0 ][ xCtb ][ yCtb ]) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| } |  |
| if(sao_cb_enable_flag && LcuSaoEnableFlag[ 1 ][ xCtb ][ yCtb ]) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag && LcuSaoEnableFlag[ 2 ][ xCtb ][ yCtb ]) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| } |  |

#### 7.3.4.1 Sample adaptive offset unit CABAC syntax

| sao_unit_cabac( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 & & CtbAddrInSlice != 0 && LcuSaoEnableFlag[ cIdx ][ rx - 1][ ry ] ) |  |
| sao_merge_left_flag | ae(v) |
| if( !sao_merge_left_flag ) |  |
| if( ry > 0 & & ( AddrUp > 0 ¦ ¦ slice_loop_filter_across_slices_enabled_flag ) && LcuSaoEnableFlag[ cIdx ][ rx][ ry - 1]) ) |  |
| sao_merge_up_flag | ae(v) |
| if( !sao_merge_up_flag ) |  |
| sao_offset_cabac( rx, ry, cIdx ) |  |
| } |  |

#### 7.4.2.8 Sample adaptive offset VLC semantics

sao_type_idx[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-1 of current SAO unit at position rx and ry for the colour component cIdx.

…

Table 7-1 – Specification of the edge type for SAO

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - Edge type (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge |
| - 1 | - 1D 90-degree edge |
| - 2 | - 1D 135-degree edge |
| - 3 | - 1D 45-degree edge |
| - 4 | - Band |

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the pixel range when sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 4.

sao_offset[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current SAO unit at position rx and ry for the colour component cIdx.

…

It is a requirement of bitstream conformance that when sao_type_idx[ cIdx ][ rx ][ ry ] is not equal to 4,

…

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is less than 4 and i is larger than 1,offsetSign is set to −1.

- Otherwise (sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 4 or i is less than 2), offsetSign is set to 1.

### 7.4.4 Slice data semantics

lcu_sao_enable_idx indicates the LcuSaoEnableFlag values of three color components as specified in Table 7-x.

Table 7-x – Specification of the LcuSaoEnableFlag values of three color components

| - lcu_sao_enable_idx | - LcuSaoEnableFlag[ cIdx ][ rx ][ ry ] |  |  |
| --- | --- | --- | --- |
|  | - cIdx = 0 | - cIdx = 1 | - cIdx = 2 |
| - 0 | - 0 | - 0 | - 0 |
| - 1 | - 1 | - 0 | - 0 |
| - 2 | - 1 | - 0 | - 1 |
| - 3 | - 1 | - 1 | - 0 |
| - 4 | - 1 | - 1 | - 1 |
| - 5 | - 0 | - 0 | - 1 |
| - 6 | - 0 | - 1 | - 0 |
| - 7 | - 0 | - 1 | - 1 |

#### 8.7.2.1 Modification process for sample adaptive offset

…

The modified picture buffer for the colour component cIdx is derived as follows.

–	If LcuSaoEnableFlag[ cIdx ][ rx ][ ry ] equals 0, no modification occurs.

–	Otherwise, the following ordered steps apply:

##### 8.7.2.1.1 Modification process for luma and chroma samples

…

- If saoTypeIdx is equal to one of the values of 0, 1, 2, or 3, the following ordered steps apply:

…

Table 8-2 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ] [ rx ][ ry ] | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |

#### 7.3.2.6 Sample adaptive offset parameter syntax

| aps_sao_param( ) { | Descriptor |
| --- | --- |
| sao_cb_enable_flag | u(1) |
| sao_cr_enable_flag | u(1) |
| sao_num_lcu_in_width_minus1 | ue(v) |
| sao_num_lcu_in_height_minus1 | ue(v) |
| sao_one_luma_unit_flag | u(1) |
| if(sao_one_luma_unit_flag) |  |
| lcu_sao_enable_flag[ 0 ] | u(1) |
| if (lcu_sao_enable_flag [ 0 ]) |  |
| sao_offset_vlc( 0, 0, 0 ) |  |
| if( sao_cb_enable_flag ){ |  |
| sao_one_cb_unit_flag | u(1) |
| if( sao_one_cb_unit_flag ){ |  |
| lcu_sao_enable_flag[ 1 ] | u(1) |
| if (lcu_sao_enable_flag [ 1 ]) |  |
| sao_offset_vlc( 0, 0, 1 ) |  |
| } |  |
| } |  |
| if( sao_cr_enable_flag ){ |  |
| sao_one_cr_unit_flag | u(1) |
| if( sao_one_cr_unit_flag ){ |  |
| lcu_sao_enable_flag[ 2 ] | u(1) |
| if (lcu_sao_enable_flag [ 2 ]) |  |
| sao_offset_vlc( 0, 0, 2 ) |  |
| } |  |
| } |  |
| for( ry = 0; ry <= sao_num_lcu_in_height_minus1; ry++ ) { |  |
| for( rx = 0; rx <= sao_num_lcu_in_width_minus1; rx++ ) { |  |
| if( aps_sample_adaptive_offset_flag && ! sao_one_luma_unit_flag ) { |  |
| if( ry > 0 && rx = = 0 ) |  |
| sao_repeat_row_flag[ 0 ] | u(1) |
| sao_unit_vlc(rx, ry, 0 ) |  |
| } |  |
| if( sao_cb_enable_flag && ! sao_one_cb_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 1 ] | u(1) |
| sao_unit_vlc(rx, ry, 1 ) |  |
| } |  |
| if( sao_cr_enable_flag & & !sao_one_cr_unit_flag ) { |  |
| if( ry > 0 & & rx = = 0 ) |  |
| sao_repeat_row_flag[ 2 ] | u(1) |
| sao_unit_vlc( rx, ry, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.3.2.7Sample adaptive offset unit VLC syntax

| sao_unit_vlc( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| if( !sao_repeat_row_flag[ cIdx ] ) { |  |
| if( rx = = 0 ¦ ¦ run[ cIdx ][ rx ][ ry ] < 0 ) |  |
| if( ry = = 0 ) { |  |
| sao_run_diff | u(v) |
| saoRun[ cIdx ][ rx ][ ry ] = sao_run_diff |  |
| } else { |  |
| sao_run_diff | se(v) |
| saoRun[ cIdx ][ rx ][ ry ] = sao_run_diff + saoRun[ cIdx ][ rx ][ ry – 1 ] |  |
| } |  |
| saoRun[ cIdx ][ rx + 1 ][ ry ] = saoRun[ cIdx ][ rx ][ ry ] – 1 |  |
| lcu_sao_enable_flag | u(1) |
| if (lcu_sao_enable_flag) { |  |
| if( (rx = = 0 ¦ ¦ saoRun[ cIdx ][ rx ][ ry ] < 0) && LcuSaoEnableFlag[ cIdx ][ rx][ ry - 1] ) |  |
| if( ry > 0 ) |  |
| sao_merge_up_flag | u(1) |
| if( !sao_merge_up_flag ) |  |
| sao_offset_vlc( rx, ry, cIdx) |  |
| } |  |
| } else |  |
| saoRun[ cIdx ][ rx ][ ry ] = saoRun[ cIdx ][ rx ][ ry − 1 ] |  |
| } |  |