| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0194 |
| --- | --- |

| Title: | Cross check of JCTVC-I0148 on LM clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports the verification results of the proposal JCTVC-I0148 on LM Mode Clean-Up [1] submitted by HiSilicon Technologies, Altera Europe Limited, The Hong Kong University of Science & Technology, Intel Corp and Canon Research Centre France. The proponents provide their source codes with the results. The crosschecker checked source codes and provide cross check results to the proponents. It succeeded and produced as same result as those the proponent provided. The spreadsheet contains the detailed data is accompanied.

# Problem statement and Methods Proposed

The proponents identified several discrepancies between WD and HM and proposed to fix them. And also they clarified and simplified a few functions by reusing predefined functions. The list below shows the problems and the corresponding actions.

| Problem | Solution |
| --- | --- |
| Input parameters should specify both Luma and Chroma neighbouring samples and their locations. | Change WD |
| C and L should be swapped in (8-75) | Change WD |
| Log2 function in Equations of (8-69) to (8-74) produces a floating point operation | Change Floor(log2(x)) to GetFloorLog2(x) for both WD and HM |
| In (8-72), k1 can be zero, (k1-1) can be negative, and the operation 1 << ( k1 − 1 ) is undefined. | Change WD |
| The operation “+ ( 1 << ( k1 − 1 ) ) >> k1” in (8-72) is not used in HM6.0 | Change WD |
| (8-73) in HM6.0 uses the function of CountLeadingZerosOnes(x), which is different from Log2(x) in the WD | Change Floor(log2(x)) to GetFloorLog2(x) for both WD and HM |
| CountLeadingZerosOnes(x) in HM is not defined in WD | Change CountLeadingZerosOnes(x) to 14 - Floor(log2(abs( x) + (sign(x) -1)/2 )) for HM Change WD accordingly |
| GetMSB (x) in HM is not defined in WM | Change GetMSB (x) to GetFloorLog2(x)+1 for HM Change WD accordingly |
| The calculation of iCountShift in HM6.0 could be simplified as g_aucConvertToBit[ uiWidth ] + 3 | Change HM |

# Results

## Simulation environment

The encoding environment for cross-checking the method is as following:

OS: Linux version 2.6.18-274.el5 (gcc version 4.1.2 20080704 (Red Hat 4.1.2-50))

CPU: Intel Xeon Quad Core E31230 (SANDY BRIDGE3.2GHz)

Memory : 12GB DDR3 UDIMM Memory

The decoding environment for cross-checking is as following:

OS: Win XP 64 bit

CPU: i7 875K 2.93Hz

Memory: DDR3 16G

## Simulation conditions and results

The simulation follows the test conditions of [2] for All Intra HE10 case. LMChroma is set as 1 for All Intra Main case. There was no mismatch between reconstructed sequences at the encoder side and decoded sequences at the decoder. The results are same as the proponent provided. The spreadsheet contains the detailed data of the simulation is accompanied.

|  | All Intra Main (LMChroma=1) |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# References

- L. Liu, R. Allen, X. Zhang, P. Kapsenberg, E. Francois, “LM Mode Clean-Up”, JCTVC-H0148, 9th JCT-VC Meeting, Geneva, CH, Apr. 2012

- F. Bossen, “Common HM test conditions”, JCTVC-H1100, 8th JCT-VC Meeting, San Jose, USA, Mar. 2012.