| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0195 M24434 |
| --- | --- |

| Title: | CE1 Test 4.3 : Removal of Pixel Access Constraints for SAO Interleaving Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1562 wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HM-6.0, SAO interleaving mode is supported, which enables SAO parameter estimation for each LCU. Because the deblock filter requires to access pixels in the neighboring LCUs, certain number of bottom and right boundary pixel lines in the current LCU cannot be used for SAO parameter estimation, until processing of bottom and right LCUs are finished. For low delay processing, those unavailable LCU boundary lines are not used for SAO parameter estimation in the CE1 software. This contribution provides information how much performance loss would be caused by removal of LCU boundary lines for SAO parameter estimation.

Experimental results reportedly show that the boundary line removal causes coding efficiency loss by 0.4%, 0.3%, and 0.1% for LCU size of 16, 32, and 64, respectively, for luma component and 0.8% on average for chroma components in case of low-delay B main.

Related contribution, JCTVC-I0200, studies the coding efficiency when non-deblocked pixels are used for SAO parameter estimation for the boundary lines.

# Introduction

In HM-6.0 [1] sample adaptive offset (SAO) interleaving mode is supported, which enables SAO parameter estimation for each LCU. SAO processing occurs after deblock filtering process. Therefore, it would be natural to use deblocked pixels to estimate SAO parameters. However, when a LCU is processed to estimate SAO parameters, certain number of boundary lines may not be deblock filtered yet. For low delay processing, LCU boundary lines are removed for SAO parameter estimation in CE1 [2], when SAO interleaving mode is used. This contribution provides information how much performance loss would be caused by removal of LCU boundary lines for SAO parameter estimation.

# SAO Parameter Estimation in Interleaving Mode

When deblock filtering is performed for each LCU for low-delay processing, it requires neighboring LCUs, i.e., right and bottom LCUs to complete the deblock filtering process of the current LCU. This means that certain number of lines are not deblock filtered yet when SAO parameters are estimated for the current LCU. The number of lines depends on the filter tap size. In addition, one more boundary line may not be used for estimation, because edge offset (EO) also requires processing with window size of 3 by 3.

In HM-6.0, when slice interleaving mode is used for SAO signaling, bottom 4 lines are removed for SAO parameter estimation for each LCU, where 3 lines are removed due to deblock filtering, and additional 1 line is removed due to SAO EO. In CE1, right boundary lines are also removed for each LCU for SAO parameter estimation, where total 5 lines are removed (4 lines due to deblock filtering, 1 line due to SAO EO). By removal of these boundary lines, SAO parameter is estimated only using deblock filtered pixels in the current LCU. This is illustrated in Fig. 1.

Fig. 1. Boundary line removal in CE1 anchor.

Since boundary lines are removed for parameter estimation, it is expected that the accuracy of estimated parameter would decrease, and it would decrease more if LCU size is smaller, since the portion of removed area would occupy more in the total area of a smaller LCU. CE1 uses 3 LCU sizes, 64, 32, and 16, and Table 1 below shows the occupancy ratio of the removed area in a LCU according to its size.

Table 1. Occupancy ratio of removed pixels according to LCU sizes.

| LCU size | number of pixels in LCU | number of removed pixels in LCU | occupancy ratio of removed pixels (%) |
| --- | --- | --- | --- |
| 1616 | 256 | 124 | 48.4 |
| 3232 | 1024 | 268 | 26.2 |
| 6464 | 4096 | 556 | 13.6 |

It can be noticed that about half of area is removed in LCU size of 16, and about a quarter in case of LCU size of 32. This implies there can be performance loss due to inaccurate parameter estimation. It is also possible to cause artifact by applying SAO parameters estimated using only deblock filtered pixels to the area excluded during parameter estimation.

# Experimental Results

To evaluate the coding efficiency of the proposed method, the CE1 common test condition [2] is used. Note that only the low-delay B configuration with the interleaving mode is tested.

Results are summarized in Table 2, where modified CE1 anchor without boundary line removal is set to an anchor, and performance degradation by removal of boundary lines is measured in terms of BD-rate. It can be noticed performance degradation of 0.4%, 0.3%, and 0.1% for LCU size of 16, 32, and 64, respectively, occurs for luma component and 0.8% on average for chroma components in case of low-delay B main.

Table 2. BD-rate results of SAO parameter estimation with boundary line removal.

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4% | 0.1% | -0.2% | 0.2% | 0.0% | 0.1% |
| Class C | 0.3% | 0.5% | 1.0% | 0.2% | 0.6% | 0.3% |
| Class D | 0.1% | -0.4% | -0.2% | 0.1% | -0.5% | -0.1% |
| Class E | 0.7% | -0.2% | -0.3% | 0.3% | 0.3% | 0.2% |
| Overall | 0.4% | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% |
|  | 0.3% | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% |
| Class F | 1.6% | 0.9% | 2.2% | 1.3% | 1.9% | 2.3% |
| Enc Time[%] | 105% |  |  | 109% |  |  |
| Dec Time[%] | 106% |  |  | 113% |  |  |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.9% | 1.4% | 0.2% | 0.3% | 0.3% |
| Class C | 0.3% | 1.4% | 2.0% | 0.3% | 0.5% | 0.9% |
| Class D | 0.2% | 1.1% | 1.2% | 0.2% | 0.2% | 0.4% |
| Class E | 0.4% | 1.1% | 0.9% | 0.4% | 0.2% | 0.3% |
| Overall | 0.3% | 1.1% | 1.4% | 0.3% | 0.3% | 0.5% |
|  | 0.3% | 1.1% | 1.4% | 0.3% | 0.3% | 0.5% |
| Class F | 1.0% | 1.8% | 3.8% | 0.9% | 2.6% | 2.1% |
| Enc Time[%] | 107% |  |  | 105% |  |  |
| Dec Time[%] | 112% |  |  | 108% |  |  |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.9% | 1.2% | 0.1% | 0.7% | 0.7% |
| Class C | 0.1% | 0.8% | 1.0% | 0.1% | 0.4% | 0.8% |
| Class D | 0.1% | 0.7% | 0.2% | 0.1% | 0.6% | 0.9% |
| Class E | 0.1% | 2.0% | 1.7% | 0.1% | 0.4% | 0.2% |
| Overall | 0.1% | 1.0% | 1.0% | 0.1% | 0.5% | 0.7% |
|  | 0.1% | 1.1% | 1.0% | 0.1% | 0.6% | 0.8% |
| Class F | 0.4% | 1.4% | 1.6% | 0.3% | 0.6% | 1.0% |
| Enc Time[%] | 109% |  |  | 111% |  |  |
| Dec Time[%] | 115% |  |  | 112% |  |  |

# Conclusion

It is investigated how much coding efficiency degradation occurs due to LCU boundary line removal for SAO parameter estimation. Experimental results show coding efficiency loss of 0.4%, 0.3%, and 0.1% for LCU size of 16, 32, and 64, respectively, for luma component and 0.8% on average for chroma components in case of low-delay B main.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1003, Feb. 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb., 2012.