| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0196 M24435 |
| --- | --- |

| Title: | CE1 Test 4.4 and 4.5: Removal of SAO Merge Flags in Interleaving Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1562 wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HM-6.0 [1] SAO interleaving mode is supported, which signals SAO parameter for each LCU as part of slice data. To improve coding efficiency merge schemes are supported, which provide copying schemes of SAO parameters from the upper or left LCU. This contribution investigates the coding efficiency of merge schemes.

Experimental results reportedly show that removal of merge up scheme causes coding efficiency loss of 0.0%, 0.1%, and 0.1% for LCU size of 16, 32, and 64, respectively, for luma component and 0.2% on average for chroma components in case of low-delay B main. Removal of both merge up and merge left schemes results in coding efficiency improvement by 0.5%, 0.4%, and 0.3% for LCU size of 16, 32, and 64, respectively, for luma component and 0.1% on average for chroma components in case of low-delay B main.

It is concluded that removal of merge schemes provides coding gain within the current CE1 anchor design based on HM-6.0. It is recommended to further investigate the performance of merge schemes with other promising CE1 or AhG6 proposals.

# Introduction

In HM-6.0 [1] sample adaptive offset (SAO) interleaving mode is supported, which enables SAO parameter estimation for each LCU. SAO processing occurs after deblock filtering process. Therefore, it would be natural to use deblocked pixels to estimate SAO parameters. However, when a LCU is processed to estimate SAO parameters, certain number of boundary lines may not be deblock filtered yet. For low delay processing, LCU boundary lines are removed for SAO parameter estimation in CE1 [2], when SAO interleaving mode is used. This contribution provides information how much performance loss would be caused by removal of LCU boundary lines for SAO parameter estimation.

# Merge Schemes in SAO Interleaving Mode

When slice interleaving mode is used to signal SAO parameters, sao_merge_left_flag is signalled first for each LCU, followed by sao_merge_up_flag if sao_merge_left_flag is 0. When one of these flags equals to 1, SAO parameter for the current LCU is copied from left or upper LCU. Otherwise, SAO parameter for the current LCU is signalled.

Considering interleaving mode requires separate syntax and parsing implementation, it would be worth to investigate validity of these merge schemes in terms of coding efficiency and implementation complexity within the frame work of SAO interleaving scheme.

In terms of complexity, the merge left scheme requires a buffer to store the SAO parameter of left LCU both at the encoder and decoder sides. The merge up scheme requires a buffer to store the SAO parameter of the upper LCU both at the encoder and decoder sides, which requires a line buffer of SAO parameter.

CE1 anchor is modified to investigates coding efficiency from these merge schemes. Current CE1 anchor based on HM-6.0 includes both merge up and merge left schemes. First, merge up scheme is removed from CE1 anchor, and coding efficiency is evaluated according to CE1 common test condition [2]. Then, both merge up and left schemes are removed together and the performance is tested. Note that removal of merge up scheme is defined as Test 4.4 in CE1, and removal of merge up and left is defined as Test 4.5 in CE1.

# Experimental Results

To evaluate the coding efficiency of the proposed method, the CE1 common test condition [2] is used. Note that only the low-delay B configuration with the interleaving mode is tested.

Table 2 shows the results of removal of merge up scheme compared to CE1 anchor (CE1 Test 4.4). It is observed coding efficiency loss of 0.0%, 0.1%, and 0.1% for LCU size of 16, 32, and 64, respectively, for luma component and 0.2% on average for chroma components in case of low-delay B main.

Table 3 shows the results of removal of both merge up and left schemes compared to CE1 anchor (CE1 Test 4.5). It is observed coding efficiency improvement by 0.5%, 0.4%, and 0.3% for LCU size of 16, 32, and 64, respectively, for luma component and 0.1% on average for chroma components in case of low-delay B main.

Based on these results, little gain is observed from merge up scheme when it is used with merge left scheme. However, better coding gain is achieved when both merge schemes are removed. This implies in case of interleaving mode where SAO parameter is estimated for each LCU, merge schemes may not work efficiently due to lack of correlation between SAO parameters of neighboring LCUs, while it may waste bits to signal not to use these schemes for each LCU.

However, since its performance can be improved if prediction and signalling of these flags are made wisely, it is recommended to further investigate the performance of merge schemes with other promising CE1 or AhG6 proposals.

Table 1. Test 4.4: BD-rate results of removal of merge up scheme compared to CE1 anchor.

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | -0.2% | 0.2% |
| Class D | -0.1% | -0.4% | -0.3% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.4% | 0.2% | -0.1% | -0.3% | -0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class F | 0.2% | -0.2% | 0.4% | 0.2% | -1.6% | -1.5% |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 95% |  |  |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | -0.2% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.4% |
| Class D | 0.1% | -0.3% | 0.2% | 0.0% | -0.3% | -0.5% |
| Class E | 0.1% | -0.1% | 0.0% | 0.1% | 0.2% | 0.2% |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class F | 0.3% | 1.4% | 0.2% | 0.3% | 0.3% | 0.4% |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.6% | 0.3% | 0.1% | 0.2% | 0.4% |
| Class C | 0.1% | 0.3% | 0.5% | 0.1% | 0.3% | 0.4% |
| Class D | 0.1% | 0.4% | 0.4% | 0.2% | 0.4% | -0.1% |
| Class E | 0.0% | 0.3% | 0.8% | 0.2% | 0.3% | 0.6% |
| Overall | 0.1% | 0.4% | 0.5% | 0.1% | 0.3% | 0.3% |
|  | 0.1% | 0.3% | 0.5% | 0.1% | 0.2% | 0.2% |
| Class F | 0.1% | 0.5% | 0.3% | 0.3% | 1.0% | 0.7% |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  |

Table 2. Test 4.5: BD-rate results of removal of merge up and left schemes compared to CE1 anchor.

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | -0.2% | -0.1% | -0.3% | -0.3% |
| Class C | -0.3% | -0.2% | -0.2% | -0.3% | -0.3% | 0.0% |
| Class D | -0.8% | -0.8% | -0.4% | -0.7% | -0.3% | -0.6% |
| Class E | -0.9% | -0.9% | -0.9% | -0.9% | -1.0% | -0.7% |
| Overall | -0.5% | -0.5% | -0.4% | -0.5% | -0.4% | -0.4% |
|  | -0.5% | -0.5% | -0.4% | -0.5% | -0.4% | -0.4% |
| Class F | 0.3% | -0.3% | 0.3% | 0.1% | -1.9% | -1.7% |
| Enc Time[%] | 105% |  |  | 100% |  |  |
| Dec Time[%] | 110% |  |  | 99% |  |  |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.3% | 0.1% |
| Class C | -0.1% | 0.1% | 0.4% | -0.1% | 0.0% | 0.4% |
| Class D | -0.5% | -0.7% | 0.1% | -0.6% | -0.2% | -1.2% |
| Class E | -1.2% | -1.2% | -1.1% | -1.0% | -0.7% | -0.9% |
| Overall | -0.4% | -0.3% | -0.1% | -0.3% | -0.1% | -0.3% |
|  | -0.4% | -0.3% | -0.1% | -0.3% | -0.1% | -0.3% |
| Class F | 0.6% | 1.6% | 1.0% | 0.7% | -0.8% | -0.5% |
| Enc Time[%] | 102% |  |  | 108% |  |  |
| Dec Time[%] | 103% |  |  | 108% |  |  |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.9% | 1.0% | 0.2% | 0.3% | 0.7% |
| Class C | -0.1% | 0.3% | 0.7% | 0.0% | 0.5% | 0.4% |
| Class D | -0.1% | 0.4% | 0.5% | 0.0% | 0.2% | 0.0% |
| Class E | -1.3% | -0.3% | 0.2% | -1.0% | -0.3% | -0.7% |
| Overall | -0.3% | 0.4% | 0.6% | -0.1% | 0.2% | 0.2% |
|  | -0.3% | 0.3% | 0.6% | -0.1% | 0.1% | 0.1% |
| Class F | 0.4% | 0.7% | 1.0% | 0.8% | 1.4% | 1.3% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

# Conclusion

In this contribution the performance of merge schemes for SAO parameter signaling in interleaving mode is investigated. Experimental results show little gain when merge up is used with merge left, and more gain (0.3% on average for luma) is achieved when both merge up and left are removed. Therefore, it would be reasonable to remove these merge schemes for the SAO interleaving design as in HM-6.0 or CE1 anchor. It is recommended to further investigate the performance of merge schemes with other promising CE1 or AhG6 proposals.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1003, Feb. 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb., 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).