| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0198 |
| --- | --- |

| Title: | On Tiles and Wavefront tools for parallelism |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Viéron Jérôme, Thiesse Jean-Marc 26 Burospace, Route de Gisy, 91570 Bièvres, FRANCE | Tel: Email: | +33 16 935 8998 j.vieron@ateme.com, jm.thiesse@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution proposes a preliminary assessment of Tiles and Wavefront tools for parallelism. Both tools seem to be efficiently adapted for high parallelism applications. While Wavefront does not present any noticeable subjective issue, some visual artifacts have been reported for Tiles in Chroma components and Tiles boundaries. From these observations and because Wavefront technic is usually used by encoder designers, this tool is expected to be a part of HEVC Main profile.

# Introduction

As a world-leading provider of video compression solutions for Broadcast and Broadband market, ATEME is highly interested on new tools developed within HEVC for high parallelism support. In order to anticipate the development of our HEVC based products, both Tiles and Wavefront have been respectively evaluated both objectively and subjectively on various sequences including ATEME own video materials.

This document reports results of our investigation and recommendations related to Profile definition are formulated.

# Parallelism tools assessment

## Objective evaluation

Objective results of both Tiles and Wavefront are first reported on following tables (tables 1, 2, 3).

Configuration: HM6.0, Tiles uses configuration of Class D size tiles [1], while WPP uses one substream per LCU line (Qualcomm contribution JCTVC-H0517). Results are reported for classes A, B ,E since parallel processing is less relevant for low resolution sequences.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,8% | 0,1% | 1,3% | -0,1% | -0,5% | 0,2% |
| Class B | 1,1% | 1,1% | 1,2% | 0,7% | -1,2% | -1,8% |
| Class E | 2,6% | 1,5% | 1,8% | 2,0% | -5,8% | -7,3% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1,8% | 2,0% | 2,2% | 1,7% | 4,1% | 4,1% |
| Class B | 1,4% | 1,4% | 1,6% | 1,4% | 2,1% | 1,3% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 1,3% | 1,3% | 1,1% | 0,3% | -2,1% | -2,8% |
| Class E | 2,2% | 1,7% | 1,3% | 2,0% | -3,2% | -3,5% |

Figure 1. Tiles performance compared to WPP anchor at same degree of parallelism.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,9% | 0,0% | 1,2% | 0,0% | -0,3% | 0,1% |
| Class B | 1,3% | 1,2% | 1,2% | 0,8% | -1,1% | -1,9% |
| Class E | 3,1% | 1,6% | 2,0% | 2,5% | -5,8% | -7,1% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 2,8% | 2,9% | 3,3% | 2,6% | 5,3% | 5,4% |
| Class B | 2,5% | 2,2% | 2,5% | 2,4% | 2,9% | 2,2% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 2,8% | 2,5% | 2,7% | 1,7% | -0,9% | -1,3% |
| Class E | 9,1% | 6,5% | 6,5% | 9,1% | 1,9% | 1,4% |

Figure 2. Tiles performance vs Ref.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | -0,1% | -0,1% | 0,1% | 0,2% | 0,0% |
| Class B | 0,2% | 0,1% | 0,1% | 0,2% | 0,1% | 0,0% |
| Class E | 0,5% | 0,2% | 0,2% | 0,5% | 0,1% | 0,2% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,9% | 0,9% | 1,0% | 0,9% | 1,2% | 1,2% |
| Class B | 1,0% | 0,8% | 0,9% | 1,0% | 0,8% | 0,9% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 1,4% | 1,2% | 1,6% | 1,4% | 1,3% | 1,5% |
| Class E | 6,8% | 4,8% | 5,2% | 7,0% | 5,4% | 5,2% |

Figure 3. Wavefront performance vs Ref.

The overall performance of Wavefront is higher for all tested configurations. Bitrate overhead of tile ranges from 0.8% to 2.6% compared to Wavefront.

## Subjective evaluation

Tests have been conducted on various video sequences (including ATEME own material) in order to evaluate the subjective performance. HEVC Tiles and Wavefront tools are configured for same parallelism performance. Class D configuration is used for Tiles while one substream by line is used for Wavefront in order to experiment its higher parallelism configuration available.

On subjective evaluation of both tools, none visible artifact has been reported for Wavefront while Tiles yield to some issues summarized below:

- Tiles artifacts are mainly associated with Intra coding. These artifacts appear on Intra only encoded sequence and in frames following Intra refresh frames in RA encoding.

- Reported artifacts belong to Chroma component, especially in Tile boundaries for flat areas such as blue sky or black background. In RA configuration, such artifacts on Intra frame result in blocking and ghosting effects for depending frames.

- Tiles artifacts are particularly visible with animated movies where chroma quality is really relevant.

# Conclusion

It is recommended to adopt the Wavefront tool in HEVC Main profile due to its high parallelism capacity without main artifact issue. Also, the developer-aware framework proposed within Wavefront Parallel Processing tool will be helpful for quick design of efficient HEVC implementation and hopefully the HEVC adoption by the market.

# References

[1] A. Fuldseth, M. Horowitz, S. Xu, A. Segall, M. Zhou, JCTVC-F335, Torino, July 2011

# Patent rights declaration(s)

ATEME does not have any current or pending patent rights relating to the technology described in this contribution.