| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0199 |
| --- | --- |

| Title: | Non-CE1: Decoupling SAO on/off from SAO type with neighbor-based contexts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Chih-Ming Fu, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Woo-Shik Kim, Do-Kyoung Kwon Texas Instruments 12500 TI Blvd MS 8649, Dallas, TX, USA 75243 In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chihming.fu, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com Woo-Shik Kim +1-214-480-1562 {wskim, d-kwon}@ti.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com |
| Source: | MediaTek Inc., Texas Instruments Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the interleaving mode of SAO, each LCU has one SAO type for each color component, and the SAO type can represent SAO off, EO, or BO. In this proposal, SAO on/off signaling is decoupled from the SAO type. SAO on/off flag for each color component is sent before the other syntax elements of the color component, and the context formation method for the SAO on/off flag coding is proposed, which uses the SAO on/off flags of left LCU and the upper LCU as follows: CtxIdx = LeftFlag + UpperFlag = 0, 1, or 2. The three context models are shared by the three color components. Although the flag of upper LCU is looked up, the size of the flag line buffer would be small. In some implementation design, this line buffer may not be needed, if the line buffer required during SAO processing due to deblocking can be shared. Another context formation method can be CtxIdx = LeftFlag = 0 or 1, which reduces one context.

In addition to the new SAO on/off flag coding, the constrained merge syntax from JCTVC-I0193 is included as follows. The merge-left or merge-up flag is not coded and inferred as zero when the left or upper LCU is SAO-off, which is reported to achieve 0.1% gain.

Simulation results reportedly show that the 3-context SAO on/off flag coding in combination with the constrained merge syntax can achieve 0.7%/1.1%/1.0% gains of Y/Cb/Cr for Main-LDB and 0.7%/0.6%/0.6% gains of Y/Cb/Cr for HE10-LDB compared to the CE1 anchor. The 2-context SAO on/off flag coding in combination with the constrained merge syntax can achieve 0.6%/1.0%/0.9% gains of Y/Cb/Cr for Main-LDB and 0.6%/0.6%/0.5% gains of Y/Cb/Cr for HE10-LDB.

# Introduction

In HM-6.0, SAO uses LCU-based syntax. The LCU-based SAO syntax can support picture-based SAO encoding for better coding efficiency. It can also support LCU-based SAO encoding for less computing resource in some platforms or for sub-picture latency requirements in some applications. The LCU-based SAO syntax can be signaled in APS for one picture or interleaved in slice data. This switching capability is achieved by using sao_interleaving_flag both in APS and slice header. When LCU-based SAO encoding is applied, each LCU derives its own SAO parameters. In order to reduce side information, sao_merge_left_flag and sao_merge_upper_flag can be used to copy SAO parameters from neighbors. If merging is not applied, sao_type_idx is used to represent the SAO type for the current LCU. There are six values of sao_type_idx, as shown in Table 1, where 0, 1-4, and 5 stand for SAO-off, EO, and BO, respectively. In this contribution, we propose to decouple SAO on/off from sao_type_idx and insert SAO on/off flag before the other SAO syntax elements in each LCU. In this way, it could allow switching off SAO for each LCU without sending any other SAO parameters, so the overall coding efficiency can be improved. The rest of this document is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Table 1.  Original SAO type index

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Off |
| - 1 | - 1D 0-degree edge |
| - 2 | - 1D 90-degree edge |
| - 3 | - 1D 135-degree edge |
| - 4 | - 1D 45-degree edge |
| - 5 | - Band |

# Proposed method

In the proposed method, first, we remove SAO-off from sao_type_idx. Therefore, the number of allowed values for sao_type_idx is reduced from 6 to 5, as shown in Table 2. Second, separate on/off flag called sao_lcu_enable_flag is added in front of other SAO syntax elements to allow turning on/off SAO for each LCU, as shown in Table 3. The sao_lcu_enable_flag is coded using CABAC with three contexts, and the three contexts are shared by three components. The context index is defined as 0 if both left and upper LCUs are SAO-off, defined as 1 if one of the two neighboring LCUs is SAO-on and the other is SAO-off, and defined as 2 if both two neighboring LCUs are SAO-on.

Table 2.  Proposed SAO type index

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge |
| - 1 | - 1D 90-degree edge |
| - 2 | - 1D 135-degree edge |
| - 3 | - 1D 45-degree edge |
| - 4 | - Band |

Table 3.  Proposed syntax change

| sao_unit_cabac( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| sao_lcu_enable_flag[cIdx][rx][ry] | ae(v) |
| if( sao_lcu_enable_flag[cIdx][rx][ry] == 1 ) { |  |
| if( rx > 0 & & CtbAddrInSlice != 0 && sao_lcu_enable_flag[cIdx][rx - 1][ry] ) |  |
| sao_merge_left_flag | ae(v) |
| if( !sao_merge_left_flag ) |  |
| if( ry > 0 & & ( AddrUp > 0 ¦ ¦ slice_loop_filter_across_slices_enabled_flag ) && sao_lcu_enable_flag[cIdx][rx][ry - 1] ) |  |
| sao_merge_up_flag | ae(v) |
| if( !sao_merge_up_flag ) |  |
| sao_offset_cabac( rx, ry, cIdx ) |  |
| } |  |
| } |  |

| sao_offset_cabac( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =4 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| } |  |

Although the flag of upper LCU is looked up, the size of the flag line buffer would be small. In some implementation design, this line buffer may not be needed, if the line buffer required during SAO processing due to deblocking can be shared. The SAO on/off flag line buffer is already required due to deblocking. The bottom pixel rows of the upper LCU cannot be processed by SAO until the deblocking of the boundary between the current LCU and the upper LCU is done, so it is necessary to store SAO on/off flags of the upper LCU row.

Another simplified context formation that only uses two contexts is proposed as follows. The context index is defined as 0 if the left LCU is SAO-off and defined as 1 if the left LCU is SAO-on. This can reduce complexity.

We also propose to use the constrained merge syntax presented in JCTVC-I0193 [1]. The merge-left or merge-up flag is not coded and inferred as zero when the left or upper LCU is SAO-off, which is reported to achieve 0.1% gain. Our proposal is combined with the constrained merge syntax for simulation.

# Simulation results

Experiments were conducted according to the CE1 test conditions [2]. Three different LCU sizes are tested under Main-LDB and HE10-LDB conditions. The anchor is SAO-on with non-normative changes defined in CE1.

Table 3 shows the results of the 3-context SAO on/off coding in combination with the constrained merge syntax. In Main-LDB condition with LCU size equal to 64x64, overall BD-rates are -0.7%, -1.1%, and -1.0% for Y, Cb, and Cr, respectively. In HE10-LDB condition with LCU size equal to 64x64, overall BD-rates are -0.7%, -0.6%, and -0.6% for Y, Cb, and Cr, respectively.

Table 4 shows the results of the simplified 2-context SAO on/off coding in combination with the constrained merge syntax. In Main-LDB condition with LCU size equal to 64x64, overall BD-rates are -0.6%, -1.0%, and -0.9% for Y, Cb, and Cr, respectively. In HE10-LDB condition with LCU size equal to 64x64, overall BD-rates are -0.6%, -0.6%, and -0.5% for Y, Cb, and Cr, respectively.

# Conclusions

In this contribution, a new SAO on/off flag coding method is proposed for SAO interleaving mode. For each color component, one SAO on/off flag, decoupled from the SAO type, is sent before the other SAO syntax elements of the color component. The context formation can explore its two neighbors (left LCU and upper LCU) or only one neighbor (left LCU), and the three or two context models are shared by all luma and chroma components. The proposal is integrated with a constrained merge syntax, which reportedly achieves 0.1% gain. Overall, the Y/Cb/Cr gains are 0.7%/1.1%/1.0% for Main-LDB with LCU size equal to 64x64 using 3-context SAO on/off coding, and the corresponding gains are 0.6%/1.0%/0.9% when 2-context SAO on/off coding is used.

# References

- W.-S. Kim, D.-K. Kwon, I. S. Chong, M. Karczewicz, “Non-CE1: LCU SAO enable flag coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0193, April, 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1101, February, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 3.  Results of the 3-context SAO on/off coding with the constrained merge syntax. Run times are inaccurate.

Table 4.  Results of the 2-context SAO on/off coding with the constrained merge syntax. Run times are inaccurate.