| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0199 |
| --- | --- |

| Title: | Non-CE1: Decoupling SAO on/off from SAO type with neighbor-based contexts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Chih-Ming Fu, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chihming.fu, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the interleaving mode of SAO, each LCU has one SAO type for each component, and the SAO type can represent BO, EO, or off. In this proposal, SAO on/off is decoupled from the SAO type. One SAO on/off flag for one component is sent before the rest syntax elements of the component, and the context formation method of the on/off flag uses the left LCU and the upper LCU as follows: CtxIdx = LeftFlag + UpperFlag = 0, 1, or 2. The three contexts are shared by the three components. Although the upper LCU is used, no additional line buffer is needed because the on/off flag line buffer is already required due to deblocking. Another context formation method can be CtxIdx = LeftFlag = 0 or 1, which reduces one context. In addition to the new SAO on/off coding, the constrained merge syntax from JCTVC-I0193 is included as follows. The merge-left or merge-up flag is not coded and inferred as zero when the left or upper LCU is SAO-off, which is reported to achieve 0.1% gain. Simulation results reportedly show that the 3-context SAO on/off coding in combination with the constrained merge syntax can achieve 0.7%/1.1%/1.0% gains of Y/Cb/Cr for Main-LDB and 0.7%/0.6%/0.6% gains of Y/Cb/Cr for HE10-LDB. The 2-context SAO on/off coding in combination with the constrained merge syntax can achieve 0.6%/1.0%/0.9% gains of Y/Cb/Cr for Main-LDB and 0.6%/0.6%/0.5% gains of Y/Cb/Cr for HE10-LDB.

# Introduction

In HM-6.0, SAO uses LCU-based syntax. The LCU-based SAO syntax can support picture-based SAO encoding for better coding efficiency. It can also support LCU-based SAO encoding for less computing resource in some platforms or for sub-picture latency requirements in some applications. The LCU-based SAO syntax can be signaled in APS for one picture or interleaved in slice data. This switching capability is achieved by using sao_interleaving_flag both in APS and slice header. When LCU-based SAO encoding is applied, each LCU derives its own SAO parameters. In order to reduce side information, sao_merge_left_flag and sao_merge_upper_flag can be used to copy SAO parameters from neighbors. If merging is not applied, sao_type_idx is used to represent the SAO type for the current LCU. There are six values of sao_type_idx, as shown in Table 1, where 0, 1-4, and 5 stand for SAO-off, EO, and BO, respectively. In this contribution, we propose to decouple SAO on/off from sao_type_idx and insert SAO on/off flag before the rest SAO syntax elements in each LCU. In this way, it could allow switching off SAO for one LCU without sending any other SAO parameters, so the overall coding efficiency can be improved. The rest of this document is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Table 1.  Original SAO type index

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Off |
| - 1 | - 1D 0-degree edge |
| - 2 | - 1D 90-degree edge |
| - 3 | - 1D 135-degree edge |
| - 4 | - 1D 45-degree edge |
| - 5 | - Band |

# Proposed method

In the proposed method, first, we remove SAO-off from sao_type_idx. Therefore, the number of allowed values for sao_type_idx reduced from 6 to 5, as shown in Table 2. Second, separate on/off flag called sao_enable_flag is added in front of other SAO syntax elements to allow turning on/off SAO for each LCU, as shown in Table 3. The sao_enable_flag is coded using CABAC with three contexts, and the three contexts are shared by three components. The context index is defined as 0 if both left and upper LCUs are SAO-off, defined as 1 if one of the two neighboring LCUs is SAO-on and the other is SAO-off, and defined as 2 if both two neighboring LCUs are SAO-on.

Table 2.  Proposed SAO type index

| - sao_type_idx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge |
| - 1 | - 1D 90-degree edge |
| - 2 | - 1D 135-degree edge |
| - 3 | - 1D 45-degree edge |
| - 4 | - Band |

Table 3.  Proposed sao_enable_flag

| sao_unit_cabac( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| sao_enable_flag | ae(v) |
| if( sao_enable_flag == 1 ) { |  |
| if( rx > 0 & & CtbAddrInSlice != 0 ) |  |
| sao_merge_left_flag | ae(v) |
| if( !sao_merge_left_flag ) |  |
| if( ry > 0 & & ( AddrUp > 0 ¦ ¦ slice_loop_filter_across_slices_enabled_flag ) ) |  |
| sao_merge_up_flag | ae(v) |
| if( !sao_merge_up_flag ) |  |
| sao_offset_cabac( rx, ry, cIdx ) |  |
| } |  |
| } |  |

Although the upper LCU is used in context formation for sao_enable_flag, there is no additional line buffer. The SAO on/off flag line buffer is already required due to deblocking. The bottom pixel rows of the upper LCU cannot be processed by SAO until the deblocking of the boundary between the current LCU and the upper LCU is done, so it is necessary to store SAO on/off flags of the upper LCU row.

Another simplified context formation that only uses two contexts is proposed as follows. The context index is defined as 0 if the left LCU is SAO-off and defined as 1 if the left LCU is SAO-on. This still can reduce some complexity although the SAO on/off line buffer cannot be removed.

In JCTVC-I0193 [1], constrained merge syntax is invented. The merge-left or merge-up flag is not coded and inferred as zero when the left or upper LCU is SAO-off, which is reported to achieve 0.1% gain. Our proposal is combined with the constrained merge syntax for simulation. However, decisions can be made separately for the proposed SAO on/off coding and the constrained merge syntax.

# Simulation results

Experiments were conducted according to the CE1 test conditions [2]. Three different LCU sizes are tested under Main-LDB and HE10-LDB conditions. The anchor is SAO-on with non-normative changes defined in CE1.

Table 3 shows the results of the 3-context SAO on/off coding in combination with the constrained merge syntax. In Main-LDB condition with LCU size equal to 64x64, overall BD-rates are -0.7%, -1.1%, and -1.0% for Y, Cb, and Cr, respectively. In HE10-LDB condition with LCU size equal to 64x64, overall BD-rates are -0.7%, -0.6%, and -0.6% for Y, Cb, and Cr, respectively.

Table 4 shows the results of the simplified 2-context SAO on/off coding in combination with the constrained merge syntax. In Main-LDB condition with LCU size equal to 64x64, overall BD-rates are -0.6%, -1.0%, and -0.9% for Y, Cb, and Cr, respectively. In HE10-LDB condition with LCU size equal to 64x64, overall BD-rates are -0.6%, -0.6%, and -0.5% for Y, Cb, and Cr, respectively.

# Conclusions

In this contribution, a new SAO on/off coding method is proposed for SAO interleaving mode. For each component, one SAO on/off flag, decoupled from the SAO type, is sent before the rest SAO syntax elements of the component. The context formation can explore its two neighbors (left LCU and upper LCU) or only one neighbor (left LCU), and the three or two contexts are shared by all luma and chroma components. The proposal is integrated with a constrained merge syntax, which reportedly achieves 0.1% gain. Overall, it is reported that the Y/Cb/Cr gains are 0.7%/1.1%/1.0% for Main-LDB with LCU size equal to 64x64 using 3-context SAO on/off coding, and the corresponding gains become 0.6%/1.0%/0.9% when 2-context SAO on/off coding is used

# References

- W.-S. Kim, D.-K. Kwon, I. S. Chong, M. Karczewicz, “Non-CE1: LCU SAO enable flag coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0193, April, 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1101, February, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 3.  Results of the 3-context SAO on/off coding with the constrained merge syntax. Run times are inaccurate.

[Results of LCU=32x32 will be updated right after simulation is done.]

Table 4.  Results of the 2-context SAO on/off coding with the constrained merge syntax. Run times are inaccurate.

[Results of LCU=16x16 will be updated right after simulation is done.]