| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0200 M24439 |
| --- | --- |

| Title: | Non-CE1: SAO Parameter Estimation Using Non-deblocked Pixels in Interleaving Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1562 wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HM-6.0 SAO parameters are estimated for each LCU at the encoder side, when interleaving mode is used. SAO processing occurs after deblock filtering process. Because the deblock filter requires to access pixels in the neighboring LCUs, certain number of bottom and right boundary pixel lines in the current LCU cannot be used for SAO parameter estimation, until processing of bottom and right LCUs are finished. For low delay processing, those unavailable pixels are not used for SAO parameter estimation in CE1 software, which reduces coding efficiency of SAO as reported in CE1 Test 4.3.

This contribution is an encoder only proposal, which proposes to use non-deblocked pixels for those unavailable pixels to estimate SAO parameters, where the number of unavailable lines is exactly determined according to SAO type.

Experimental results reportedly show that the proposed method can improve coding efficiency by 0.3/0.1/0.0 % (Y/Cb/Cr) in case of LCU size 16, 0.3/1.2/1.4 in case of LCU size 32, and 0.2/0.8/1.1 in case of LCU size 64, respectively, compared to the CE1 anchor in case of low-delay B main, when interleaving mode is used. In case of low-delay P main, the reported gains are 1.8/0.5/0.1 for LCU size 16, 0.9/1.7/1.9 for LCU size 32, and 0.3/1.2/1.5 for LCU size 64, respectively. Note that these average values do not include Class F sequences, in which larger gains are reportedly observed.

# Introduction

In HM-6.0 SAO parameters are estimated for each LCU at the encoder side, when interleaving mode is used. SAO processing occurs after deblock filtering process. Because the deblock filter requires to access pixels in the neighboring LCUs, certain number of bottom and right boundary pixel lines in the current LCU cannot be used for SAO parameter estimation, until processing of bottom and right LCUs are finished. For low delay processing, those unavailable pixels are not used for SAO parameter estimation in CE1 software, where 4 bottom lines and 5 right column lines for luma and 2 bottom line and 3 right column lines for chroma are removed for each LCU. This reduces coding efficiency of SAO as reported in CE1 Test 4.3.

For more accurate SAO parameter estimation without delay, this contribution proposes to use non-deblocked pixels for those unavailable pixels to estimate SAO parameters, where the number of unavailable lines is exactly determined according to SAO type.

# Proposed Method

## Number of unavailable lines according to SAO type

When deblock filter is used, it has been discussed among CE1 participants that 3 bottom lines and 4 right column lines are not available for SAO parameter estimation of the current LCU. In addition to these lines, one more line may not be available when edge offset is used. This depends on the direction of edge offset filter. Fig. 1 shows edge offset filter window shapes.

|  | C |  |  |  | C |  |  |  | C |  |  |  | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(a) 1-D 0-degree (b) 1-D 90-degree (c) 1-D 135-degree (d) 1-D 45-degree

Fig. 1. Pixel level range division into two bands and 16 sub-bands in each band.

From Fig. 2 it can be noticed one additional right column line is not available in (a), (c), and (d), and one additional bottom line is not available in (b), (c), and (d). In case of band offset, no additional line is unavailable. Table 1 shows number of unavailable lines according to SAO type.

In the proposed method, exact number of lines is set according to SAO type to use deblock-filtered pixels to estimate SAO parameters.

Table 1. Number of horizontal and vertical non-deblock-filtered pixel lines to be avoided for SAO parameter estimation.

| sao_type_idx | Luma |  | Chroma |  |
| --- | --- | --- | --- | --- |
|  | bottom lines | right column lines | bottom lines | right column lines |
| 0 | N/A | N/A | N/A | N/A |
| 1 | 3 | 5 | 1 | 3 |
| 2 | 4 | 4 | 2 | 2 |
| 3 | 4 | 5 | 2 | 3 |
| 4 | 4 | 5 | 2 | 3 |
| 5 | 3 | 4 | 1 | 2 |

## SAO parameter estimation using non-deblocked pixels

We propose to use non-deblocked pixels for unavailable pixels to estimate SAO parameters, where the number of unavailable lines is exactly determined according to SAO type as specified in section 2.1.

Fig. 2 illustrates this for luma component, where black colored pixels are already deblock-filtered when SAO estimation is performed for the current LCU. Blue colored pixels are not available due to edge offset, and red colored pixels are not available due to deblock filtering. Blue colored pixels may not exist depending on SAO type. For example, for band offset, there will be no blue pixel. In the proposed method, SAO parameter estimation is performed using deblock-filtered pixels for black colored pixels and using non-deblock-filtered pixels for blue and red colored pixels.

Fig. 2. Illustration of available and non-available pixels in LCU for luma component.

# Experimental Results

To evaluate the coding efficiency of the proposed method, the CE1 common test condition [2] is used. Note that only the interleaving mode is used.

During implementation of the proposed method on top of the CE1 software, we have found a bug, which is applicable in general with or without the proposed method. Specifically, the bug is that the picture boundary is not correctly considered during the SAO parameter estimation of SAO type 1 (horizontal EO). As this bug fix relates to only one type of SAO in the picture boundary area, its impact on the performance is negligible. Experimental results show 0.0% BD rate change in most of the cases, where the full results can be found in the attached spread sheets. Note that we applied this bug fix to both the anchor and the proposed method for fair comparison.

Results of the proposed method are summarized in Table 2. Compared to CE1 anchor with bug fix, bitrate reduction of 0.3/0.1/0.0 % (Y/Cb/Cr) in case of LCU size 16, 0.3/1.2/1.4 in case of LCU size 32, and 0.2/0.8/1.1 in case of LCU size 64, respectively, is achieved in case of low-delay B main with the interleaving mode. In case of low-delay P main, the gains are 1.8/0.5/0.1 for LCU size 16, 0.9/1.7/1.9 for LCU size 32, and 0.3/1.2/1.5 for LCU size 64, respectively. Note that these average values do not include Class F sequences, in which larger gains are observed. Full test results can be found in the attached spread sheets.

Table 2. BD-rate results summary of the proposed method compared to CE1 anchor with bug fix.

|  | LCU size | average excluding class F |  |  | average of class F |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | Cb | Cr | Y | Cb | Cr |
| low-delay B main | 16 | -0.3% | -0.1% | 0.0% | -1.4% | -2.2% | -1.4% |
|  | 32 | -0.3% | -1.2% | -1.4% | -0.9% | -0.6% | -2.9% |
|  | 64 | -0.2% | -0.8% | -1.1% | -0.4% | -1.3% | -1.7% |
| low-delay P main | 16 | -1.8% | -0.5% | -0.1% | -1.9% | -0.8% | -0.4% |
|  | 32 | -0.9% | -1.7% | -1.9% | -1.0% | -1.9% | -3.2% |
|  | 64 | -0.3% | -1.2% | -1.5% | -0.5% | -1.1% | -1.1% |

Table 3. BD-rate results of the proposed method compared to CE1 anchor with bug fix.

- LCU size 16

|  | All Intra Main |  |  |  |  |  | All Intra HE10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |
| Class A |  | 0.1% |  | 0.3% |  | 0.3% |  | 0.1% |  | 0.0% |  | -0.2% |
| Class B |  | 0.1% |  | 0.4% |  | 0.4% |  | 0.0% |  | 0.1% |  | 0.1% |
| Class C |  | 0.1% |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.1% |  | 0.2% |
| Class D |  | 0.1% |  | 0.3% |  | 0.3% |  | 0.0% |  | 0.2% |  | 0.2% |
| Class E |  | 0.2% |  | 0.6% |  | 0.6% |  | 0.1% |  | 0.2% |  | 0.2% |
| Overall |  | 0.1% |  | 0.4% |  | 0.3% |  | 0.0% |  | 0.1% |  | 0.1% |
|  |  | 0.1% |  | 0.4% |  | 0.3% |  | 0.0% |  | 0.1% |  | 0.1% |
| Class F |  | -0.7% |  | 0.2% |  | -0.1% |  | -1.2% |  | 0.3% |  | 0.3% |
| Enc Time[%] |  | 98% |  |  |  |  |  | 90% |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  |  |  | 91% |  |  |  |  |
|  |  | Random Access Main |  |  |  |  |  | Random Access HE10 |  |  |  |  |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |
| Class A |  | -0.2% |  | -0.4% |  | -0.2% |  | 0.0% |  | -0.1% |  | -0.3% |
| Class B |  | -0.4% |  | -0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.2% |
| Class C |  | -0.1% |  | -0.3% |  | -0.4% |  | -0.1% |  | 0.1% |  | 0.2% |
| Class D |  | 0.0% |  | -0.1% |  | -0.1% |  | 0.0% |  | 0.2% |  | 0.1% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.2% |  | -0.2% |  | -0.1% |  | 0.0% |  | 0.1% |  | 0.1% |
|  |  | -0.2% |  | -0.2% |  | -0.1% |  | 0.0% |  | 0.1% |  | 0.1% |
| Class F |  | -0.8% |  | -0.6% |  | -1.2% |  | -0.9% |  | 0.1% |  | -0.2% |
| Enc Time[%] |  | 102% |  |  |  |  |  | 97% |  |  |  |  |
| Dec Time[%] |  | 106% |  |  |  |  |  | 93% |  |  |  |  |
|  |  | Low delay B Main |  |  |  |  |  | Low delay B HE10 |  |  |  |  |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -0.3% |  | -0.1% |  | 0.3% |  | -0.2% |  | -0.2% |  | -0.2% |
| Class C |  | -0.3% |  | -0.4% |  | -0.9% |  | -0.2% |  | -0.5% |  | -0.3% |
| Class D |  | -0.1% |  | 0.2% |  | 0.3% |  | 0.0% |  | 0.3% |  | 1.0% |
| Class E |  | -0.6% |  | 0.2% |  | 0.3% |  | -0.3% |  | 0.0% |  | 0.1% |
| Overall |  | -0.3% |  | -0.1% |  | 0.0% |  | -0.2% |  | -0.1% |  | 0.1% |
|  |  | -0.3% |  | 0.0% |  | 0.0% |  | -0.2% |  | -0.1% |  | 0.1% |
| Class F |  | -1.4% |  | -2.2% |  | -1.4% |  | -1.4% |  | -2.2% |  | -2.1% |
| Enc Time[%] |  | 103% |  |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] |  | 102% |  |  |  |  |  | 90% |  |  |  |  |
|  |  | Low delay P Main |  |  |  |  |  | Low delay P HE10 |  |  |  |  |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -2.5% |  | -0.4% |  | 0.1% |  | -0.6% |  | -0.3% |  | -0.6% |
| Class C |  | -1.6% |  | -0.7% |  | -1.2% |  | -0.8% |  | -0.4% |  | -0.5% |
| Class D |  | -1.1% |  | -0.8% |  | 0.2% |  | -0.8% |  | 0.0% |  | -0.5% |
| Class E |  | -2.1% |  | 0.3% |  | 0.5% |  | -0.5% |  | -0.3% |  | 0.0% |
| Overall |  | -1.8% |  | -0.5% |  | -0.1% |  | -0.7% |  | -0.2% |  | -0.4% |
|  |  | -1.9% |  | -0.5% |  | -0.2% |  | -0.7% |  | -0.3% |  | -0.5% |
| Class F |  | -1.9% |  | -0.8% |  | -0.4% |  | -1.6% |  | -0.5% |  | -1.0% |
| Enc Time[%] |  | 101% |  |  |  |  |  | 97% |  |  |  |  |
| Dec Time[%] |  | 97% |  |  |  |  |  | 87% |  |  |  |  |

- LCU size 32

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.3% | -0.1% | -0.2% | -0.4% |
| Class B | 0.0% | -0.3% | -0.7% | 0.0% | -0.1% | -0.3% |
| Class C | 0.0% | -0.7% | -0.9% | -0.1% | -0.4% | -0.5% |
| Class D | 0.0% | -0.4% | -0.6% | -0.1% | -0.1% | -0.2% |
| Class E | 0.0% | -0.5% | -0.4% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.4% | -0.6% | 0.0% | -0.2% | -0.3% |
|  | 0.0% | -0.4% | -0.6% | 0.0% | -0.2% | -0.3% |
| Class F | -0.1% | -1.0% | -1.2% | -0.2% | -0.7% | -0.8% |
| Enc Time[%] | 97% |  |  | 92% |  |  |
| Dec Time[%] | 97% |  |  | 90% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -1.2% | -0.9% | -0.1% | -0.4% | -0.7% |
| Class B | -0.2% | -1.0% | -1.2% | -0.1% | -0.3% | -0.5% |
| Class C | -0.1% | -0.9% | -1.2% | -0.1% | -0.4% | -0.5% |
| Class D | -0.1% | -0.8% | -0.8% | -0.1% | -0.5% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -1.0% | -1.0% | -0.1% | -0.4% | -0.6% |
|  | -0.2% | -1.0% | -1.0% | -0.1% | -0.4% | -0.6% |
| Class F | -0.3% | -1.2% | -1.5% | -0.4% | -0.6% | -1.0% |
| Enc Time[%] | 106% |  |  | 104% |  |  |
| Dec Time[%] | 111% |  |  | 96% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.8% | -1.3% | -0.2% | -0.5% | -0.2% |
| Class C | -0.3% | -1.5% | -1.9% | -0.3% | -0.6% | -0.9% |
| Class D | -0.2% | -1.4% | -1.0% | -0.2% | -0.4% | 0.0% |
| Class E | -0.3% | -1.4% | -1.1% | -0.4% | -0.1% | -0.2% |
| Overall | -0.3% | -1.2% | -1.4% | -0.3% | -0.4% | -0.3% |
|  | -0.2% | -1.1% | -1.2% | -0.2% | -0.4% | -0.4% |
| Class F | -0.9% | -0.6% | -2.9% | -0.7% | -1.6% | -3.2% |
| Enc Time[%] | 105% |  |  | 102% |  |  |
| Dec Time[%] | 105% |  |  | 86% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.2% | -1.4% | -1.7% | -0.3% | -0.7% | -0.6% |
| Class C | -0.7% | -1.8% | -2.2% | -0.4% | -0.9% | -1.0% |
| Class D | -0.5% | -1.8% | -1.9% | -0.3% | -0.2% | -0.6% |
| Class E | -1.2% | -1.9% | -1.7% | -0.6% | -0.6% | -0.1% |
| Overall | -0.9% | -1.7% | -1.9% | -0.4% | -0.6% | -0.6% |
|  | -0.9% | -1.7% | -1.9% | -0.4% | -0.6% | -0.7% |
| Class F | -1.0% | -1.9% | -3.2% | -0.8% | -1.2% | -2.0% |
| Enc Time[%] | 103% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 84% |  |  |

- LCU size 64

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.7% | -0.8% | 0.0% | -0.4% | -0.7% |
| Class B | 0.0% | -0.5% | -0.7% | 0.0% | -0.3% | -0.5% |
| Class C | 0.0% | -0.4% | -0.5% | 0.0% | -0.3% | -0.3% |
| Class D | 0.0% | -0.4% | -0.5% | 0.0% | -0.2% | -0.3% |
| Class E | 0.0% | -0.8% | -1.1% | 0.0% | -0.2% | -0.5% |
| Overall | 0.0% | -0.6% | -0.7% | 0.0% | -0.3% | -0.5% |
|  | 0.0% | -0.5% | -0.7% | 0.0% | -0.3% | -0.5% |
| Class F | -0.1% | -0.4% | -0.5% | -0.1% | -0.4% | -0.6% |
| Enc Time[%] | 109% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.9% | -1.0% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | -0.7% | -0.7% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.5% | -0.6% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.3% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.6% | -0.6% | 0.0% | 0.0% | 0.0% |
|  | -0.1% | -0.6% | -0.6% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.5% | -0.7% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 113% |  |  | 119% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.9% | -0.9% | -0.1% | -0.6% | -1.0% |
| Class C | -0.1% | -0.7% | -1.2% | -0.1% | -0.5% | -0.7% |
| Class D | -0.1% | -0.4% | -0.3% | -0.1% | -0.8% | -0.5% |
| Class E | -0.2% | -1.5% | -2.1% | -0.1% | -0.5% | -1.0% |
| Overall | -0.2% | -0.8% | -1.1% | -0.1% | -0.6% | -0.8% |
|  | -0.2% | -0.8% | -1.0% | -0.1% | -0.6% | -0.7% |
| Class F | -0.4% | -1.3% | -1.7% | -0.3% | -1.2% | -1.1% |
| Enc Time[%] | 119% |  |  | 115% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -1.4% | -1.6% | -0.1% | -0.9% | -0.9% |
| Class C | -0.2% | -0.7% | -1.0% | -0.1% | -0.4% | -0.5% |
| Class D | -0.2% | -0.6% | -1.1% | -0.1% | -0.5% | -0.3% |
| Class E | -0.3% | -2.6% | -2.7% | -0.2% | -0.7% | -1.0% |
| Overall | -0.3% | -1.2% | -1.5% | -0.1% | -0.6% | -0.7% |
|  | -0.3% | -1.2% | -1.5% | -0.1% | -0.6% | -0.7% |
| Class F | -0.5% | -1.1% | -1.1% | -0.4% | -0.5% | -1.7% |
| Enc Time[%] | 115% |  |  | 110% |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  |

# Conclusion

It is proposed to use non-deblocked pixels to estimate SAO parameters for LCU boundary lines, where exact number of unavailable lines is set according to SAO type. The proposed method is implemented for LCU interleaving case, and experimental results show coding efficiency improvement of 0.3/0.1/0.0 % (Y/Cb/Cr) in case of LCU size 16, 0.3/1.2/1.4 in case of LCU size 32, and 0.2/0.8/1.1 for LCU size of 64, respectively. In case of low-delay P main, the gains are 1.8/0.5/0.1 for LCU size 16, 0.9/1.7/1.9 for LCU size 32, and 0.3/1.2/1.5 for LCU size 64, respectively. These average values do not include Class F sequences, in which larger gains are observed.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1003, Feb. 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb., 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).