| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0205 |
| --- | --- |

| Title: | AHG5: A throughput study of transform coefficient coding proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vivienne Sze Madhukar Budagavi | Email: | sze@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution provides a study of the throughput of proposed tools related to transform coefficient coding in AHG5 and CE3.

# Introduction

CABAC is a well known throughput bottleneck in the video codec implementations (particularly at the decoder-side).  This is due to the many feedback loops in the CABAC, several of which are in the context selection. Fig. 1 highlights the feedback loops within the CABAC in the decoder.  First, the value of the context is updated based on the previously decoded bins.  Second, the context to be chosen depends on the previously decoded bins as well as various factors such as macroblock type, binIdx, transform size, location in transform, etc.  At the decoder, this second feedback loop passes through the bin decoder.  Thus parallelism is a particular challenge at the decoder.   Note that the context update and range update feedback loops are simpler that the context selection loops and thus do not affect throughput as severely.

These loops make the CABAC difficult to parallelize, which makes it difficult to achieve the high throughput necessary for high resolution and frame-rate videos.  Furthermore, since high throughput can be traded-off for power savings using voltage scaling, the serial nature of CABAC limits the battery life for video codecs that reside on mobile devices.  This limitation is a critical concern, as a significant portion of future video codecs will be battery-operated.

Fig.1. The CABAC is composed of three key blocks: Binarizer, Context Modeler and Bin Coder (i.e. Arithmetic Coder). Feedback loops are highlighted with dashed lines.

## Speculative Computations for multi-bin decoding

A common technique used in practice is speculative computation which aims to increase the throughput of CABAC.   However, this comes as a cost of increased number of computations per bin.    For a two bin per cycle example: in order to decode bin2 in parallel with bin1 requires 2x computations for bin2 to account for the case that bin1=0 and bin1=1.  In particular, the number of context selection computations increases.  The number of operations increases exponentially as we increase the number of bins that are to be processed in parallel. (e.g. for N bins to be decoded in parallel, we need up to 1+2+4+…+2(N-1) context selection computations)

To reduce the cycle time (i.e critical path), the CABAC needs to be pipelined. For instance, the 4-stage pipeline shown in Fig. 2 is commonly used for AVC CABAC implementations.  Due to the feedback loops, each inserted pipeline stage between context selection and bin decoder increases the number of context selection computations by 2x.  For the architecture in Fig. 2, the number of context selections operations grows to by a factor of 4x, which means a 2-bin per cycle pipelined solution requires 12x context selection computations.

Fig. 2 Common pipelined architecture for CABAC. Context modeling blocks in grey.  Feedback loops highlight with dashed lines.

Thus the total increase in computations for context selection for an M-stage pipelined architecture decoding N-bins per cycles is 2(M-1) *(1+2+4+…+2(N-1) ). Often optimizations are done in an attempt to reduce the total computations.

# HM-6.0 tools for transform coefficient coding

The following tools are currently being used in HM-6.0.  This section will describe throughput implications for each of these tools.

Using neighboring dependencies

- For context selection of significant_coeff_flag

  - Dependency on previously decoded bin is avoided (2-bins/cycle).

Removing dependencies based on position

- Removing neighboring dependencies for high frequency positions

  - Reduces number of bins using neighboring dependency to 37.5% in worst case

  - Negligible impact on coding efficiency

Ordering of bins

- Grouping bypass bins

  - Reduces cycles required to process bypass bins; grouping bypass bins maximize the high throughput of bypass bins.

  - Negligible impact on coding efficiency

# Proposed tools for transform coefficient coding

The following tools are proposed for transform coefficient coding.  This section will describe the throughput implications for each of these tools.

Using neighboring dependencies

- For context selection of coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag

  - Dependency on previously decoded bin always possible

- To select Rice Parameter for coeff_abs_level_remaining

  - Increases storage requirements

- With multiple scans

  - Increases combinations of neighboring dependencies for context selection

- With smaller TU

  - In the worst case, all bins are coded with neighboring dependencies

Removing dependencies based on position

- Removing some neighboring dependencies at certain positions to enable 3 or 4 bin parallelism

  - Reduces dependencies across multiple bins; however, more checks are required to determine if dependency should be removed. Certain methods assume specific mapping of positions (bins) to cycle, which limits architecture design decisions (e.g. 4 bin parallelism that assumes a certain mapping can increase the implementation complexity of a 3 bin/cycle architecture)

Ordering of bins

- Interleaving context coded bins

  - Increases speculative calculations and context switches

  - Note that this can also reduce scans at encoder; however, encoder can potentially use single scan with HM-6.0 since coefficients are signaled in small groups of 16 which can be accessed in parallel.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).