| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0207 |
| --- | --- |

| Title: | CE3: Cross-check of JCTVC-I0324 on parallelization with bit-plane coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vivienne Sze | Email: | sze@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution reports cross-check results from Texas Instruments for I0324.  The BD-rate results under common conditions match results provided by Qualcomm.

# Summary based on Analysis of Source Code

Using H0228 as a starting point, significant_coeff_flag, coeff_abs_greater1_flag and coeff_abs_greater2_flag are separated into bit-planes (not interleaved).  Neighboring dependencies are removed from the template, to enable 3-bin and 4-bin parallelism.

# Experiment Results

The HM-6.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations

3-bins/cycle parallelism

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.2% | -0.1% | -0.5% | -0.2% |
| Class B | 0.1% | 0.0% | -0.1% | 0.1% | -0.1% | -0.2% |
| Class C | 0.0% | 0.3% | 0.2% | 0.0% | 0.3% | 0.3% |
| Class D | -0.2% | 0.2% | 0.1% | -0.2% | 0.2% | 0.2% |
| Class E | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.3% |
| Overall | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | -0.2% |
| Enc Time[%] | 110% |  |  | 113% |  |  |
| Dec Time[%] | 97% |  |  | 101% |  |  |

4-bins/cycle parallelism

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.2% | 0.1% | 0.2% | -0.1% | 0.1% |
| Class B | 0.4% | 0.4% | 0.3% | 0.3% | 0.2% | 0.1% |
| Class C | 0.1% | 0.5% | 0.5% | 0.1% | 0.4% | 0.5% |
| Class D | 0.0% | 0.5% | 0.4% | 0.0% | 0.5% | 0.4% |
| Class E | 0.1% | 0.3% | 0.2% | 0.1% | 0.0% | 0.1% |
| Overall | 0.2% | 0.4% | 0.3% | 0.2% | 0.2% | 0.2% |
|  | 0.2% | 0.4% | 0.3% | 0.2% | 0.2% | 0.2% |
| Class F | 0.2% | 0.3% | 0.3% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  | 112% |  |  |
| Dec Time[%] | 96% |  |  | 101% |  |  |