| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 7 May, 2012 | Document: JCTVC-I0212 WG11 Number: m24453 |
| --- | --- |

| Title: | CE2.c.1 Reduction of number of filter classes in ALF BA mode, by changing class mapping table and using HM6.0 encoding algorithm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In HEVC CD, ALF block-based filter adaptation with directional features (BA-DIR) produces up to 16 filters. Using HM6.0 encoding algorithm, this document investigates reduction of number of BA-DIR filter classes to 7 or 4 by changing class mapping table. In addition, reduction of number of operations in BA-DIR calculation is investigated based on JCTVC-H0415. Simulations report 0.1% ~ 0.2% BD-rate losses when the number of BA-DIR filter classes is reduced, while additional 0.1% ~ 0.2% BD-rate losses are reported when using JCTVC-H0415 on top of BA-DIR class reduction.

# Text changes

Reduction of number of filter classes in BA:

- For 7 BA classes:

#### 7.3.3.6 Adaptive loop filter info syntax

| alf_info( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| if(NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| …… |  |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if( alf_no_filters_minus1 = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_minus1 > 1 ) |  |
| for( i = 1; i < 7; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ][ ry ][ rx ][ i ] | u(1) |

Table 8-1. avgVarTab values for different avgVar and directions.

| avgVar | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Direction=0 | 0 | 1 | 1 | 2 | 2 | 2 |
| Direction=1 | 0 | 3 | 3 | 4 | 4 | 4 |
| Direction=2 | 0 | 5 | 5 | 6 | 6 | 6 |

- For 4 BA classes:

#### 7.3.3.6 Adaptive loop filter info syntax

| alf_info( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| if(NumALFFiltersInStoredBuffer[cIdx] > 0) |  |
| …… |  |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if( alf_no_filters_minus1 = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_minus1 > 1 ) |  |
| for( i = 1; i < 4; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ][ ry ][ rx ][ i ] | u(1) |

Table 8-2. avgVarTab values for different avgVar and directions.

| avgVar | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Direction=0 | 0 | 1 | 1 | 1 | 1 | 1 |
| Direction=1 | 0 | 2 | 2 | 2 | 2 | 2 |
| Direction=2 | 0 | 3 | 3 | 3 | 3 | 3 |

Additionally, apply JCTVC-H0415 to reduce number of operations for BA-DIR calculation:

The figure below illustrate how BADIR calculation is changed

- 8.7.3.3 Derivation process for filter index array for luma samples

...

...

The filter index array fIdx is specified in the follows:

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x−1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-1)
	varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y−1 ] – s’’[ xC+x, yC+y+1 ] |	(8-2)

- The variables varTempH1[ x, y ], varTempV1[ x, y ], and varTemp3[ x, y ] with x, y = 0..( (nS) − 1 )>>2 are derived as

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with (i, j) = (0,1),(1,1),(2,1)	(8-3)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with (i, j) =(1,0),(1,1),(1,2)	(8-4)
	varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 2	(8-5)

# Encoding algorithm

Coding conditions:

- HE10 with ALF and SAO both using picture-based encoding in HM6.0

- Max number of filters per frame = 16

- Apply “Tickets #400” for ALF picture-based encoding:

TEncAdaptiveLoopFilter.cpp, line 492

Int maxDepthInFilters = (Int)(logf((float)(m_iALFMaxNumberFilters ))/logf(4.0));

Anchor as in HM6.0: 16 BA classes, the encoder can choose among:

- Whole frame as 1 region, with up to 16 BA classes.

- A frame is divided into 4 regions, each has up to 4 BA classes (merged from 16 BA classes).

- A frame is divided into 16 regions, each has up to 1 filter classes (essentially no BA).

For 7 BA classes, the encoder can choose among:

- Whole frame as 1 region, with up to only 7 BA classes.

- A frame is divided into 4 regions, each has up to 4 BA classes (merged from the 7 BA classes).

- A frame is divided into 16 regions, each has up to 1 filter classes (essentially no BA).

For 4 BA classes, the encoder can choose among:

- Whole frame as 1 region, with up to only 4 BA classes.

- A frame is divided into 4 regions, each has up to 4 BA classes.

- A frame is divided into 16 regions, each has up to 1 filter classes (essentially no BA).

# Results

The simulations were run in the 64-bit Linux cluster environment, with heterogeneous cores.

Reduction of number of filter classes in BA:

| - 7 BA classes |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.1% | 0.1% | 0.1% | 0.2% |
|  | Enc Time | 96 | 97 | 97 | 99 |
|  | Dec Time | 93 | 94 | 93 | 96 |

| - 4 BA classes |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.1% | 0.2% | 0.1% | 0.2% |
|  | Enc Time | 96 | 98 | 98 | 100 |
|  | Dec Time | 95 | 97 | 96 | 100 |

Additionally, apply JCTVC-H0415 together with 7 BA and 4 BA classes:

| - 7 BA classes + H0415 |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.2% | 0.3% | 0.2% | 0.3% |
|  | Enc Time | 96 | 97 | 97 | 99 |
|  | Dec Time | 93 | 94 | 92 | 96 |

| - 4 BA classes + H0415 |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.2% | 0.3% | 0.2% | 0.4% |
|  | Enc Time | 97 | 98 | 98 | 100 |
|  | Dec Time | 96 | 97 | 96 | 100 |

** Please note that the run-time is not accurate because during the time when we conduction simulations, the Linux cluster cores being assigned to our queue were heterogeneous.

# Conclusions

Using HM6.0 encoding algorithm, this document subtest investigates reduction of number of BA-DIR filter classes to 7 or 4 by changing class mapping table. In addition, reduction of number of operations for BA-DIR calculation is investigated based on JCTVC-H0415.

Simulations report 0.1% ~ 0.2% BD-rate losses when the number of BA-DIR filter classes is reduced.

On top of losses introduced by reducing BA-DIR classes, using JCTVC-H0415 3HV to replace current BA computations results in additional 0.1~0.2% BD-rate losses.

H0415, 8th JCT-VC meeting

HM5.0 (15BA)

- H0415: 0.1 / 0.1 / 0.1 / 0.2

CE2.c, 9th JCT-VC meeting

HM6.0

- 7BA classes:  0.1 / 0.1 / 0.1 / 0.2

- 7BA + H0415: 0.2 / 0.3 / 0.2 / 0.3 (additional loss 0.1 / 0.2 / 0.1 / 0.1)

HM6.0

- 4BA classes:  0.1 / 0.2 / 0.1 / 0.2

- 4BA + H0415: 0.2 / 0.3 / 0.2 / 0.4 (additional loss 0.1 / 0.1 / 0.1 / 0.2)

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Non-CE8: ALF BA feature computation only accessing 3x3 area”, JCTVC-H0415, San José, CA, USA, Feb. 2012