| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 7 May, 2012 | Document: JCTVC-I0213_r1 WG11 Number: m24454 |
| --- | --- |

| Title: | CE2.e.1 Reduction of depth of LCU division for filtering control |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In HEVC CD, PIC-mode of ALF has filtering on/off control up to the 3rd-level of LCU quadtree division depth. This document investigates reduction of LCU quadtree division depth for ALF filtering control. Simulations with 16 filter per picture, under HM6.0 which incorporates combination of ALF BA and RA, report 0.1% BD-rate losses with LCU quadtree division depth 0 (LCU-based on/off control), and 0.0% BD-rate losses with LCU quadtree division depth up to just 1. As supplementary results, additional simulations under single filter scenario (no BA and no RA) report similar numbers as above.

# Simulation setup and results

Coding conditions

- HE10 with ALF and SAO both using picture-based encoding in HM6.0

- Apply “Tickets #400” for ALF picture-based encoding:

TEncAdaptiveLoopFilter.cpp, line 492

Int maxDepthInFilters = (Int)(logf((float)(m_iALFMaxNumberFilters ))/logf(4.0));

The anchor is:

- Max depth CU filtering control = 3 (i.e. can be 0, 1, 2, 3)

TEncAdaptiveLoopFilter.cpp, line 1526

for (ctrlDepth=0; ctrlDepth<4; ctrlDepth++)

## With 16 filters per picture, HM6.0 combination of BA and RA

The simulations were run in the 64-bit Linux cluster environment.

| - Max depth CU filtering control = 0 (i.e. LCU-based filtering control) for (ctrlDepth=0; ctrlDepth<1; |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.0% | 0.1% | 0.1% | 0.1% |
|  | Enc Time | 91 | 97 | 98 | 97 |
|  | Dec Time | 99 | 99 | 101 | 101 |

| - Max depth CU filtering control = 1 for (ctrlDepth=0; ctrlDepth<2; |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Enc Time | 94 | 99 | 99 | 98 |
|  | Dec Time | 100 | 100 | 101 | 101 |

| - Max depth CU filtering control = 2 for (ctrlDepth=0; ctrlDepth<3; |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Enc Time | 97 | 99 | 99 | 99 |
|  | Dec Time | 99 | 99 | 100 | 101 |

## With one filter per picture (no BA and no RA)

| - Max depth CU filtering control = 0 (i.e. LCU-based filtering control) for (ctrlDepth=0; ctrlDepth<1; |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.0% | 0.1% | 0.1% | 0.1% |
|  | Enc Time | 92 | 97 | 98 | 97 |
|  | Dec Time | 101 | 101 | 101 | 101 |

| - Max depth CU filtering control = 1 for (ctrlDepth=0; ctrlDepth<2; |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Enc Time | 95 | 99 | 100 | 98 |
|  | Dec Time | 101 | 100 | 102 | 100 |

| - Max depth CU filtering control = 2 for (ctrlDepth=0; ctrlDepth<3; |  | AI-HE | RA-HE | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- |
|  | Average | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Enc Time | 97 | 99 | 99 | 99 |
|  | Dec Time | 100 | 100 | 100 | 100 |

- Conclusions:

This document investigates reduction of LCU quadtree division depth for ALF filtering control.

- 16 filter per picture, under HM6.0 which incorporates combination of ALF BA and RA

- 0.1% BD-rate losses with LCU quadtree division depth 0 (LCU-based on/off control).

- 0.0% BD-rate losses with LCU quadtree division depth up to just 1.

- As supplementary results, additional simulations under single filter scenario (no BA and no RA) report similar numbers as above.

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References