| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 7 May, 2012 | Document: JCTVC-I0214 WG11 Number: m24455 |
| --- | --- |

| Title: | AHG6: Cross-check of JCTVC-H0290 on restriction on ALF filtering area |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for AHG6 JCTVC-H0290 on restriction on ALF filtering area. The case with the maximum ALF filtering area of 70% is cross-checked. Rate-PSNR numbers match exactly to the results in JCTVC-H0290. As for software study, the restriction of maximum filtering area is implemented as encoder constraint via filtering area budget control and adaptive thresholding with respect to SSD reduction. The computation of SSD reduction does not increase HM6.0 encoder complexity since it is already utilized in the ALF RD decision.

# Cross-check results

Coding conditions

- HE10 with ALF and SAO both using picture-based encoding in HM6.0

- Apply “Tickets #400” for ALF picture-based encoding:

TEncAdaptiveLoopFilter.cpp, line 492

Int maxDepthInFilters = (Int)(logf((float)(m_iALFMaxNumberFilters ))/logf(4.0));

Simulations were conducted using 64-bit Linux cluster environment.

| Maximum ALF filtering area of 70%, provided by JCTVC-I0290 [1] |  | AI-10 | RA-10 | LB-10 | LP-10 |
| --- | --- | --- | --- | --- | --- |
|  | Class A | 0.3% | 0.4% |  |  |
|  | Class B | 0.2% | 0.1% | 0.0% | 0.0% |
|  | Class C | 0.2% | 0.1% | 0.0% | 0.0% |
|  | Class D | 0.1% | 0.2% | 0.1% | 0.0% |
|  | Class E | 0.4% |  | 0.0% | 0.1% |
|  | Average | 0.2% | 0.2% | 0.0% | 0.0% |
|  | Enc Time | 100% | 100% | 100% | 100% |
|  | Dec Time | 97% | 98% | 100% | 99% |

# Software study

- The restriction of maximum filtering area is implemented as encoder constraint via filtering area budget control and adaptive thresholding with respect to SSD reduction.

- The computation of SSD reduction does not increase HM6.0 encoder complexity since it is already utilized in the ALF RD decision.

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “AHG6: Restriction on ALF filtering area,” JCTVC-I0290, Geneva, Switzerland, Apr.-May. 2012