| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 7 May, 2012 | Document: JCTVC-I0215_r2 WG11 Number: m24456 |
| --- | --- |

| Title: | AHG6: ALF baseline Option 2 BA with reduced number of filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This cobtribution investigates reducing of number of filters in ALF block-based filter adaptation with directional features (BA-DIR), in the context of ALF baseline Option 2 BA created in AHG6. Two approaches are tested: 1. Force encoder to merge initial 16 BA-DIR classes into fewer than a maximum allowed number of filters (similar to the methods in CE2.c.1-1), and 2. Modify the class mapping table in CD Text (similar to the approach in CE2.c.1-2). Results of method 1 show 0.0% BD-rate changes by forcing to merge below 8 filter, and 0.1% losses by forcing to merge below 4 filter. Results of method 2 show 0.0% BD-rate changes by modifying the class mapping in CD to 10 classes or to 7 classes.

# Intorduction

In AHG6 of the 9th JCTVC meeting, after studying different aspects of ALF conducted in CE2 [1], ALF baseline options were created [2]. In Option 2, filter coefficients are estimated using picture based optimization and are transmitted in APS. Two filter adaptation methods are currently being evaluated under ALF Option 2: Block-based filter adaptation with directional features (BA-DIR), and region-based filter adaptation by partitioning a picture (RA). BA-DIR classifies blocks at 4x4 basis, leading to a need of fast memory for filter coefficients to support the switching among filters at 4x4 block basis. Reducing the size of such filter coefficient memory is desirable. In this document, reducing the number of BA-DIR filters is investigated. Two approaches are tested: 1. Forcing encoder to merge initial BA-DIR classes into fewer than a maximum allowed number of filters (similar to the methods in CE2.c.1-1 [3]), and 2. Modifying the class mapping table in CD Text (similar to the approach in CE2.c.1-2 [4]).

# Methods to reduce number of classes in BA-DIR

## Forcing encoder to merge BA-DIR classes

This method specifies the maximum number of filter per picture, such that the encoder is forced to merge the initial 16 BA-DIR classes into fewer than the specified number of filters. In the software of AHG6 ALF Option 2 BA, this can be done by changing ALF_LONG_TERM_MAX_NUM_FILTERS in:

deriveFilterInfo(compIdx, m_alfPictureCorr[compIdx], alfParam, ALF_LONG_TERM_MAX_NUM_FILTERS);

Forcing up to 8 filters per picture:

#define ALF_LONG_TERM_MAX_NUM_FILTERS      8   //!< number of filters in APS

Forcing up to 4 filters per picture:

#define ALF_LONG_TERM_MAX_NUM_FILTERS      4   //!< number of filters in APS

## Modifying the class mapping table in CD Text

- For 10 BA classes:

#### 7.3.3.6 	Adaptive loop filter picture info syntax

| alf_picture_info(cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if( alf_no_filters_minus1 = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_minus1 > 1 ) |  |
| for( i = 1; i < 10; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ][ i ] | u(1) |

#### 8.7.3.3 	Derivation process for filter index array for luma samples

…

…

- The variable avgVar is derived as

varTab[16] = {0, 1, 2, 2, 3, 3, 3, 4, 4, 4, 4, 5, 5, 5, 5, 5}	(8-1)

varTab[8] = {0, 1, 1, 2, 2, 2, 2, 3,}		(8-2)
	avgVar = varTab[Clip3( 0, 15 7, (varTemp3[ x >> 2 ][ y >> 2 ] * 1024 ) >> (3 + BitDepthY) )]	(8-3)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)−1 is derived as

fIdx[ x ][ y ] = avgVarTab[direction][avgVar]		(8-4)

where avgVarTab[direction][avgVar] is defined in .

Table 8-1. avgVarTab values for different avgVar and directions.

| avgVar | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Direction=0 | 0 | 1 | 2 | 3 | 4 | 5 |
| Direction=1 | 0 | 6 | 7 | 8 | 9 | 10 |
| Direction=2 | 0 | 11 | 12 | 13 | 14 | 15 |

| avgVar | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Direction=0 | 0 | 1 | 2 | 3 |
| Direction=1 | 0 | 4 | 5 | 6 |
| Direction=2 | 0 | 7 | 8 | 9 |

- For 7 BA classes:

#### 7.3.3.6 	Adaptive loop filter picture info syntax

| alf_picture_info(cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if( alf_no_filters_minus1 = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_minus1 > 1 ) |  |
| for( i = 1; i < 7; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ][ i ] | u(1) |

#### 8.7.3.3 	Derivation process for filter index array for luma samples

…

…

- The variable avgVar is derived as

varTab[16] = {0, 1, 2, 2, 3, 3, 3, 4, 4, 4, 4, 5, 5, 5, 5, 5}	(8-5)

varTab[8] = {0, 1, 1, 1, 1, 1, 1, 2,}		(8-6)
	avgVar = varTab[Clip3( 0, 15 7, (varTemp3[ x >> 2 ][ y >> 2 ] * 1024 ) >> (3 + BitDepthY) )]	(8-7)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)−1 is derived as

fIdx[ x ][ y ] = avgVarTab[direction][avgVar]		(8-8)

where avgVarTab[direction][avgVar] is defined in .

Table 8-2. avgVarTab values for different avgVar and directions.

| avgVar | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Direction=0 | 0 | 1 | 2 | 3 | 4 | 5 |
| Direction=1 | 0 | 6 | 7 | 8 | 9 | 10 |
| Direction=2 | 0 | 11 | 12 | 13 | 14 | 15 |

| avgVar | 0 | 1 | 2 |
| --- | --- | --- | --- |
| Direction=0 | 0 | 1 | 2 |
| Direction=1 | 0 | 3 | 4 |
| Direction=2 | 0 | 5 | 6 |

# Simulation results

The simulations were conducted in the 64-bit Linux cluster environment.

LCU size 64, both ALF and SAO are using CLU mode.

## RA-16 compared against BA-DIR

| Replace BA(16) with RA-16 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | AI-10 | RA-10 | LB-10 | LP-10 |
| Class A | 0.5% | 0.9% |  |  |
| Class B | 0.2% | 0.4% | 0.1% | 0.4% |
| Class C | 0.1% | 0.1% | 0.0% | 0.2% |
| Class D | 0.1% | 0.0% | 0.1% | 0.1% |
| Class E | 0.2% |  | 0.1% | 0.1% |
| Average | 0.2% | 0.4% | 0.1% | 0.2% |
| Enc Time | 100% | 100% | 100% | 100% |
| Dec Time | 99% | 98% | 99% | 100% |

## Forcing encoder to merge BA-DIR classes

| Force encoder to merge below 8 filters |  |  |  |  |  | Force encoder to merge below 4 filters |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI-10 | RA-10 | LB-10 | LP-10 |  |  | AI-10 | RA-10 | LB-10 | LP-10 |
| Class A | 0.0% | 0.0% |  |  |  | Class A | 0.2% | 0.3% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |  | Class B | 0.1% | 0.2% | 0.1% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% |  | Class C | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% |  | Class D | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% |  | 0.0% | 0.0% |  | Class E | 0.1% |  | 0.2% | 0.1% |
| Average | 0.0% | 0.0% | 0.0% | 0.0% |  | Average | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time | 100% | 100% | 100% | 100% |  | Enc Time | 100% | 100% | 100% | 100% |
| Dec Time | 100% | 100% | 100% | 101% |  | Dec Time | 100% | 100% | 100% | 102% |

## Modifying the class mapping table in CD Text

| Modify class mapping in CD to 10 filters |  |  |  |  |  | Modify class mapping in CD to 7 filters |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI-10 | RA-10 | LB-10 | LP-10 |  |  | AI-10 | RA-10 | LB-10 | LP-10 |
| Class A | 0.1% | 0.1% |  |  |  | Class A | 0.1% | 0.1% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |  | Class B | 0.0% | 0.1% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% |  | Class C | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% |  | Class D | 0.0% | -0.1% | 0.0% | -0.1% |
| Class E | 0.0% |  | 0.1% | 0.0% |  | Class E | 0.0% |  | 0.2% | 0.0% |
| Average | 0.0% | 0.0% | 0.0% | 0.0% |  | Average | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time | 101% | 100% | 100% | 100% |  | Enc Time | 100% | 101% | 101% | 102% |
| Dec Time | 99% | 100% | 99% | 101% |  | Dec Time | 99% | 102% | 103% | 105% |

# Conclusions

This document investigates reducing of number of filters in ALF block-based filter adaptation with directional features (BA-DIR), in the context of ALF baseline Option 2 BA created in AHG6. Two approaches are tested:

1. Force encoder to merge initial BA-DIR classes into fewer than a maximum allowed number of filters (similar to the methods in CE2.c.1-1):

- Force to merge below 8 filters: 0.0% BD-rate changes.

- Force to merge below 4 filters: 0.1% BD-rate changes.

2. Modify the class mapping table (similar to the approach in CE2.c.1-2).

- Modify the class mapping table to 10 classes: AI / RA / LDB / LDP: 0.0% / 0.0% / 0.0% / 0.0%

- Modify the class mapping table to 7 classes: AI / RA / LDB / LDP: 0.0% / 0.0% / 0.0% / 0.0%

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “CE2”, JCTVC-Ixxxx, Geneva, Switzerland, Apr.-May 2012

- “AHG6: Baseline options for ALF”, JCTVC-I0157, Geneva, Switzerland, Apr.-May 2012

- “CE2.c.1: Reducing number of filters of BA (Encoder only)”, JCTVC-I0250, Geneva, Switzerland, Apr.-May 2012

- “CE2.c.1 Reduction of number of filter classes in ALF BA mode, by changing class mapping table and using HM6.0 encoding algorithm”, JCTVC-I0157, Geneva, Switzerland, Apr.-May 2012