| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0216v2 |
| --- | --- |

| Title: | AHG7: Reducing HEVC worst-case memory bandwidth by restricting bi-directional 4x8 and 8x4 prediction units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-978-719-1348 timhellman@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This proposal recommends adding a profile & level-independent limit on the types of motion compensation prediction units to alleviate worst-case motion compensation bandwidth.  It claims that the present draft of the standard results in worst-case bandwidth that is too high for practical implementations.  It notes that the AVC standard has a level limit on the number of motion vectors per MB pair, but the proposal claims that restricting 4x8 and 8x4 PUs to uni-prediction all pictures would be a preferred method to address the same concerns for HEVC.  The proposal claims luma losses of 0.2 – 0.4% for class A and class B sequences, and 0.2 to 0.8% overall.

# Introduction

The AVC standard specifies a profile-independent level limit on the maximum number of motion vectors.  The limit is 16 motion vectors per two consecutive macroblocks (MaxMvsPer2Mb) for Levels 3.1 and above.   This limit was defined to limit worst-case bandwidth requirements when decoding large pictures.  This submission recommends adding a level-independent limit to HEVC to address the same concerns.  Rather than restricting the number of motion vectors, however, simply limiting 4x8 and 8x4 PUs to use only uni-directional prediction achieves a reasonable maximum bandwidth with very little impact on PSNR for HD-sized sequences, and also results in a simpler encoder implementation than a vector limit.  Contribution [2] recommended study of such a limit.  This contribution provides experimental results.

The limit on bidirectional PUs could be achieved by specifying an encoder restriction.   However, this proposal recommends changing the parsing syntax when the restriction is in place, as doing so reduces the coding loss and also makes it trivial to detect when a faulty encoder is attempting to evade the restriction.

AVC’s level-specific limit is based on the assumption that low-resolution pictures can afford higher relative peak bandwidth, because the overall number of pixels is lower.  However, one very common uses for low-resolution streams is to combine many together into a mosaic.  In this case the aggregate overall bandwidth will be much higher than for a single picture, simply because the MV limit isn’t applied to the individual streams.  Therefore this proposal recommends applying the limit at all levels.

# Motion Compensation Bandwidth

Motion compensation memory bandwidth is determined by the PU block size, filter extent and the memory alignment (access characteristics of DDR DRAMs).  For a 32-bit DRAM interface (which is typical for embedded systems), the minimum data access size is 32 bytes (8 ‘beats’ of 4 bytes each), on an aligned 32-byte boundary.  Thus, in the worst case, a 4-pixel (byte) fetch ends up straddling two 32-byte boundaries, and requires a 64-byte memory access!  Since reference pictures are typically stored in a raster format, the DRAM alignment means that the ‘thin and tall’ prediction units (4x8) use more bandwidth than the ‘short and fat’ (8x4) PUs.  The larger filter extent of HEVC (8 tap Y, 4 tap C) versus AVC (6 tap Y, 2 tap C) means that HEVC uses more bandwidth than AVC even for the same PU size.

Decoder motion compensation is by far the largest user of memory bandwidth in embedded decoder implementations, such as set-top boxes.  It is not uncommon to allocate 1 Gbyte/second or more of memory bandwidth to an AVC HD decoder to guarantee proper decoding of worst-case compliance streams.  These streams consist of multiple pictures populated entirely with the worst-case PU size.  While memory bandwidth does increase with each generation of DDR devices, so does the expectations on decoder frame size and frame rate.  Therefore, providing a reasonable limit on motion compensation behavior for large picture sizes is extremely important to facilitate cost-effective implementations and ensure HEVC is successful in the marketplace.

The only motion compensation PU limit presently specified for HEVC is the elimination of 4x4 PUs.  Without any further limits, the worst-case bandwidth for HD pictures is more than double the corresponding worst-case for AVC.  That’s likely to be unacceptable for many applications.  Thus, an additional limit is needed.

- Proposed Solution

4x8 and 8x4 bi-directional partition sizes produce the highest peak normalized bandwidth.  A picture composed of all 4x8 bi-directional partitions produces the worst-case, with all 8x4 bi-directional partitions following closely behind (all worst-case alignment, data from [3]).  Figure 1 shows the worst-case bandwidth for an HD picture composed entirely of the listed PU sizes.  Removing 4x8 and 8x4 bi-directional partitions results in a near halving of the worst-case bandwidth, with bi-directional 8x8 now becoming the worst-case.  The actual bandwidth savings is 44%.  It is interesting to note that this also results in a motion vector limit quite similar to that of AVC (8 vectors per 16x16 block), but in a manner that’s much easier for an encoder to enforce.

Figure 1 – Worst-case Bandwidth for different PU sizes (in MB/Sec)

Also of interest is a comparison of the worst-case HEVC bandwidth to that of AVC with its 16 MV restriction.  As shown in Figure 2, unconstrained HEVC has a worst-case that’s more than double the bandwidth of AVC.  With the small bidirectionals removed, the worst-case is only 25% larger than AVC, which is a reasonable increase.

Figure 2 - Bandwidth Comparsion: HEVC to AVC

Enabling this restriction with a flag allows for future flexibility.  This contribution recommends changing the ‘inter_4x4_enabled_flag’ to ‘disable_inter_4x8_8x4_bidir_flag’ for this purpose.  4x4 motion compensation is a bandwidth disaster, and it has not been shown to provide significant coding gain in any scenario.  It therefore makes sense to permanently disable 4x4 inter mode.  The supplied text changes in section 8 largely consist of the simpifications from removing the inter_4x4_enabled flag.

The disable_inter_4x8_bidir_flag, when set, prevents bidirectional vectors for 4x8 and 8x4 PUs.  This is accomplished by inferring values of 0 for merge_flag and inter_pred_flag, as shown.

| } else { /* MODE_INTER */ |  |
| --- | --- |
| disable_bidir = (log2CbSize == 3 && disable_inter_4x8_8x4_bidir_flag && (PartMode == PART_Nx2N ¦¦ PartMode == PART_2NxN)) ? 1 : 0; |  |
| if (!disable_bidir ¦¦ slice_type != B) |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B && !disable_bidir) |  |
| inter_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |

The merge_flag restriction is necessary because merge mode in a B slice can produce a bi-directional vector.  It would be possible, of course, to keep the merge_flag and simply require the encoder not to select a bidirectional vector, but this is error-prone.  Moreover, experimental data showed no significant coding gain difference between keeping the merge_flag with an encoder restriction and simply not parsing it at all.

# Experimental Results

As this technique only affects B slices, I only and low-delay P sequences are unaffected and therefore are not included.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | 0.6% | 0.5% | 0.3% | 0.4% | 0.3% |
| Class B | 0.3% | 0.3% | 0.3% | 0.2% | 0.1% | 0.2% |
| Class C | 0.5% | 0.8% | 0.6% | 0.4% | 0.5% | 0.4% |
| Class D | 0.7% | 0.7% | 0.9% | 0.6% | 0.7% | 0.6% |
| Class E |  |  |  |  |  |  |
| Overall | 0.5% | 0.6% | 0.6% | 0.4% | 0.4% | 0.4% |
| Enc Time[%] | 89% |  |  | 104% |  |  |
| Dec Time[%] | 78% |  |  | 86% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.0% | 0.2% | 0.3% | 0.1% | 0.3% |
| Class C | 0.7% | 0.9% | 0.8% | 0.6% | 0.7% | 0.8% |
| Class D | 0.8% | 0.7% | 0.7% | 0.7% | 0.9% | 0.5% |
| Class E | 0.5% | 0.5% | 0.5% | 0.1% | 0.4% | 0.4% |
| Overall | 0.6% | 0.5% | 0.5% | 0.4% | 0.5% | 0.5% |
| Enc Time[%] | 99% |  |  | 108% |  |  |
| Dec Time[%] | 88% |  |  | 80% |  |  |

The following table presents a summary of luma losses with syntax changes (copied from above) compared to an encoder-only restriction:

|  | Random Access Main: Syntax Change (copied from above) |  |  | Random Access Main: Encoder-only restriction |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | 0.6% | 0.5% | 0.8% | 1.9% | 1.5% |
| Class B | 0.3% | 0.3% | 0.3% | 0.6% | 0.7% | 0.8% |
| Class C | 0.5% | 0.8% | 0.6% | 1.6% | 2.2% | 2.3% |
| Class D | 0.7% | 0.7% | 0.9% | 2.5% | 3.2% | 3.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.5% | 0.6% | 0.6% | 1.4% | 1.9% | 1.9% |

As shown, the losses are significantly larger without the change to the syntax.  Additionally, as mentioned above, changing the syntax makes it significantly easier to detect non-compliant encoders.

# Conclusions

As discussed above, the need for a high-resolution bandwidth reduction is paramount.  The technique presented here has acceptably small losses, especially for the higher resolution class A & B (0.2-0.4% luma), and requires only a small change to the parsing syntax.  This contribution recommends adoption of the described technique with a mandate that the disable_inter_4x8_8x4_bidir_flag be set to 1 for all levels.  If the lower resolution losses are not considered acceptable, then restricting levels 3.1 and higher (as in AVC) is recommended.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1003_dK, Feb. 2012.

- T. Hellman, W.Wan, “Motion Compensation Restrictions to Alleviate Memory Bandwidth Concerns for High Resolution Video”, Document of Joint Collaborative Team on Video Coding, JCTVC-H0441, Feb. 2012.

- K. Kondo, T. Suzuki, “Non-CE3: Report on a restriction for small block”, Document of Joint Collaborative Team on Video Coding, JCTVC-G392, Nov, 2011

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

Only the first three changes relate to the 4x8/ 8x4 restriction.  The rest correspond to the permanent removal of 4x4 inter prediction.

## Section 7.3.2.1- Sequence parameter set RBSP syntax

| temporal_id_nesting_flag | u(1) |
| --- | --- |
| if( log2_min_coding_block_size_minus3 = = 0 ) |  |
| inter_4x4_enabled_flag | u(1) |
| disable_inter_4x8_8x4_bidir_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |

## Section 7.3.7 – Prediction Unit Syntax

| } else { /* MODE_INTER */ |  |
| --- | --- |
| disable_bidir = (log2CbSize == 3 && disable_inter_4x8_8x4_bidir_flag && (PartMode == PART_Nx2N ¦¦ PartMode == PART_2NxN)) ? 1 : 0; |  |
| if (!disable_bidir ¦¦ slice_type != B) |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B && !disable_bidir) |  |
| inter_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |

## Section 7.4.2.1- Sequence parameter set semantics

Replace:

inter_4x4_enabled_flag specifies whether inter prediction can be applied to blocks having the size of 4 by 4 luma samples.

With:

disable_inter_4x8_8x4_bidir_flag specifies whether bi-directional inter prediction can be applied to blocks having the size of 4 by 8 and 8 by 4 luma samples.  If not present, this flag’s value is inferred to be 0.  Inter prediction blocks of size 4 by 4, both unidirection and bidirectional, are prohibited through syntax coding restrictions.

## Section 7.4.6 – Coding Unit Semantics

- Otherwise (PredMode is equal to MODE_INTER), the following applies

- If log2CbSize is greater than Log2MinCbSize, part_mode shall be in the range of 0 to 2, inclusive and in the range of 4 to 7, inclusive.

- Otherwise if log2CbSize is equal to 3 and inter_4x4_enabled_flag is equal to 0, ,the value of part_mode shall be in the range of 0 to 2, inclusive.

- Otherwise (log2CbSize is greater than 3 or inter_4x4_enabled_flag is equal to q), the value of part_mode shall be in the range of 0 to 3, inclusive.

## Section 9.2.2.7 – Binarization process for part_mode

Input to this process is a request for a binarization for the syntax element part_mode and a variable cLog2CbSize specifying the current CU size.

Output of this process is the binarization of the syntax element.

The binarization for part_mode is given by nTable 9-1 depending on PredMode, cLog2CbSize and inter_4x4_enabled_flag  and Log2MinCbSize.

nTable 9-1 – Binarization for part_mode

| PredMode | Value of part_mode | PartMode | Bin string |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | cLog2CbSize > Log2MinCbSize | cLog2CbSize = = Log2MinCbSize |  |
|  |  |  |  | cLog2CbSize = = 3 && !inter_4x4_enabled_flag | cLog2CbSize > 3 ¦ ¦ inter_4x4_enabled_flag |
| MODE_INTRA | 0 | PART_2Nx2N | - | 1 | 1 |
|  | 1 | PART_NxN | - | 0 | 0 |
| MODE_INTER | 0 | PART_2Nx2N | 1 | 1 | 1 |
|  | 1 | PART_2NxN | 011 | 01 | 01 |
|  | 2 | PART_Nx2N | 001 | 00 | 001 |
|  | 3 | PART_NxN | - | - | 000 |
|  | 4 | PART_2NxnU | 0100 | - | - |
|  | 5 | PART_2NxnD | 0101 | - | - |
|  | 6 | PART_nLx2N | 0000 | - | - |
|  | 7 | PART_nRx2N | 0001 | - | - |

## Section 9.2.3.1- Derivation process for ctxIdx

Table 9-37

| part_mode (cLog2CbSize = = Log2MinCbSize && cLog2CbSize > 3 ¦ ¦ inter_4x4_enabled_flag) | Error: Reference source not found | 3 | 0 | 1 | 2 | na | na |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 6 | 0 | 1 | 2 | na | na |

## A.3.2 – Main Profile

- Sequence parameter sets shall have inter_4x4_enabled_flag equal to 0 only.

- Sequence parameter sets shall have disable_inter_4x8_8x4_bidir_flag equal to 0 only.