| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0217 |
| --- | --- |

| Title: | Generic HEVC high level syntax for scalability and adaptation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Skupin, V. George, T. Schierl Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 314 23881 robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The proposal provides a generic high level syntax concept to signal scalability. Syntax elements of the NAL unit header and the Video Parameter Set (VPS) are proposed that allow signaling of scalability by means of so-called layer scenarios in a backwards-compatible manner. A layer scenario describes the interpretation of an operation point identifier in the NAL unit header that represents the operation point of a NAL unit within the scalability dimensions in use. Furthermore, the proposal includes VPS extension data syntax that describes the operation points within the scalability dimensions explicitly or by reference and allows signaling adaptation by deletion of complete VPS extension data syntax blocks.

# Introduction

Besides recording, storing and playout of video, the transmission thereof is a major application in today’s communication networks. The unsteady nature of specific communication channels requires means of adaptation to be provided by the high level syntax of HEVC to perform well in different environments. Straight-forward adaptation of video bitstreams within the network during transmission, e.g. by Media-Aware Network Elements (MANEs), is one of the major challenges. Such devices usually only have limited resources in terms or processing power and have to operate with very low delay which prevents in-depth bitstream analysis.

The demands of this setup were already identified within the standardization of H.264/AVC. The encapsulation of video data into Network Abstraction Layer (NAL) units and the design decision to signal many important but rather invariant parameters outside the video bitstream in so called Parameter Sets reflect this understanding. A lack of extensibility of the base specification of H.264/AVC led to complicated approaches to extend the H.264/AVC high level syntax in a backwards-compatible way when extensions were specified. For HEVC, a flexible and extensible high level syntax should be taken care of in the base specification. Therefore, the high level syntax has to provide devices, such as MANEs, with the necessary information in an accessible form, e.g. not entropy decoded, and suitable for signaling adaptation in a straight-forward way.

Our proposal takes on the proposal of a VPS as described in [1]. The normative and required VPS is intended to provide information on the video bitstream, such as operation point descriptions and dependencies, above the level that sequence parameters sets address.  It allows a MANE to signal adaptation and remove associated operation points within the scalability dimensions from the VPS syntax. Operation points within the scalability dimensions with redundant profile and level identifiers can be signaled by reference to decrease redundancy. Utilizing the VPS extension data syntax to signal individual operation point descriptions allows for straight-forward deletion thereof. Keeping the operation point descriptions in the VPS in line with the actual (perhaps formerly adapted) content of the video bitstream is of importance for MANEs to facilitate further adaptation.

In order to generically signal the operation point of a NAL unit within the scalability dimensions in use on the NAL unit header level, we propose to use the second byte of the NAL unit header for signaling a 6-bit operation point identifier and 2 bits scenario identifier. The latter allows a MANE to interpret the operation point identifier as one, two or either three separate unsigned integer variables describing the operation point of the NAL unit within the scalability dimensions, which are identified in detail within the VPS syntax. The following sections give an explanatory overview on some of the concepts in the proposal section.

## new_profile_level_flag

The new_profile_level_flag is used to signal that a certain operation point within the scalability dimensions of the video (e.g. temporal or other) uses a different profile or level than the formerly described operation points. In loops that signal profile and level or further parameters of operation points, the new_profile_level_flag is used to signal whether a new profile and level with respect to the formerly described operation points is present or to that the profile and level of a specific operation point has to be inherited from formerly described ones, e.g. by reference.

## vps_extension_data()

Profile and level identifier or other operation point specific parameters in parameter sets are given via while(more_rbsp_data()) loops of VPS extension data. This allows for straight-forward adaptation, i.e. the exclusion or cut out of certain operation points as well as the adding thereof, without changing other syntax elements within the given syntax table, i.e. within the VPS syntax. Using the while(more_rbsp_data()) syntax has the advantage of omitting the signaling of absolute number of layers and thereby eases the adaptation of the parameter sets. The proposal does not block the use of the VPS extension syntax in future HEVC extensions and maintains a method to extend the VPS syntax.

## Layer scenarios

Layer scenario represents a way in which the second byte of the NAL unit header can be used to allow for generic and adaptive signaling of one or multi-dimensional operation point identifiers.

In the layer scenario approach, a 2 bits scenario identifier is signaled in the NAL unit header. The scenario identifier allows interpretation of the remaining 6 bits of the NAL unit header that serve as an operation point identifier. The operation point identifier can either be interpreted as one- or many-dimensional in case the video features one or multiple dimensions of scalability, such as temporal and spatial, spatial and mutliview or other combinations. A detailed description of the scalable dimension and the operation points within is given by means of the VPS, whereas the NAL unit header only provides information on how to interpret the operation point identifier of the NAL unit header syntax in order to derive one or more operation point identifiers. This allows any device in possession of the VPS to completely interpret the operation point of a NAL unit within the known scalability dimensions, whereas devices that do not have access to the VPS can still perform correct adaptation of the video bitstream but without knowledge of the actual scalability dimensions.

# Proposals

## Proposed NAL unit header syntax

The second byte of the NAL unit header is used to signal a so called layer scenario and an operation point identifier that signals the operation point within the scalability dimensions (e.g. temporal or others) in the context of the layer scenario. Table 1 gives the proposed NAL unit header syntax for HEVC with the new syntax elements layer_scenario and layer_id.

Table 1: Proposed NAL unit syntax.

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| layer_scenario | u(2) |
| layer_id | u(6) |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

layer_scenario signals the so called scenario of scalability dimensions and allows the correct interpretation of the operation point identifier as a one or multi-dimensional variable.

layer_id signals the operation point of the NAL unit within scalability dimension and has to be interpreted according to the layer_scenario syntax element.

The semantics of all syntax elements of Table 1 that are not described remain unchanged with respect to the current working draft.

Table 2 gives an overview on the values of layer_scenario and presents the corresponding interpretation of layer_id.

Table 2: Overview of layer_scenario values.

| layer_scenario | Number of scalability dimensions | interpretation of layer_id | number of possible operation points per dimension |
| --- | --- | --- | --- |
| 0 | 0 - 1 | one reserved_one_1bit and one 5-bit unsigned integer | 32 |
| 1 | 2 | two 3-bit unsigned integer | 8 |
| 2 | 2 | one 2-bit and one 4-bit unsigned integer | 4 (first dimension) and 16 (second dimension) |
| 3 | 3 | three 2-bit unsigned integer | 4 |

In the case of no or a single dimension of scalability, i.e. layer_scenario with a value of zero, the value of layer_id can be interpreted as an unsigned integer, listing the operation points in consecutive order according to their decoding dependencies and importance. In case of no scalability, all NAL units shall have a layer_id value of zero to signal that there is no meaningful hierarchical adaptation procedure and all NAL units are of the same importance. The MSB of layer_id is interpreted as one reserved bit with a value of one in this case to prevent start code emulation in the NAL unit header.

When layer_scenario has a value of one, the value of layer_id can be interpreted as two 3-bit unsigned integer variables. The first three bits giving the operation point of the NAL unit in the first scalability dimension, e.g. temporal, and the second 3-bits give the operation point of the NAL unit in the second scalability dimension, e.g. spatial.

When layer_scenario has a value of two, the value of layer_id can be interpreted as a 2-bit unsigned integer variable giving the operation point in the first scalability dimension, followed by a 4-bit unsigned integer variable describing the operation point in the second scalability dimension. This scenario can be beneficial when the amount of layers of one dimension is larger than in the other dimension.

When layer_scenario has a value of three, the value of layer_id can be interpreted as three 2-bit unsigned integer variables giving the operation point according to the three scalability dimensions in use, e.g. temporal, spatial and multiview scalability.

## Proposed Video Parameter Set syntax

The syntax of the Video Parameter Set has to be adjusted in order to signal profile and level for each operation point in the possibly multi- dimensional scalability space and provide means for adaptation in a straight-forward way to reflect the current level of adaptation of the video bitstream. This is achieved by using the vps_extension_data() syntax element for profile and level signaling of NAL units that use any kind of scalability, e.g. temporal, spatial or others. Furthermore, it provides detailed information for the interpretation of the layer_id syntax element within the NAL unit header.

The proposed VPS syntax given in Table 3 signals profile and level for NAL units that do not use any kind of scalability, i.e. video data coded according to the HEVC base specification with the lowest temporal level (operation point with layer_id = 0). Profile and level identifiers for higher operation points (layer_id >= 1) within the scalability dimensions are given by means of the VPS extensions data syntax described below.

Table 3: Proposed VPS syntax.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(8) |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| scalability_type | u(7) |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data() | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

video_parameter_set_id identifies the video parameter set.

scalability_type allows detailed understanding of the scalability dimensions described with the layer_id syntax element within the NAL unit header syntax. Table 4 specifies the actual scalability dimensions signaled by layer_id depending of the value of scalability_type. For the HEVC base specification, only temporal scalability has to be defined. Future HEVC scalability extension consequently specify values for their specific needs, e.g. “1: temporal scalability, 2: spatial scalability” for a spatial scalability extension of HEVC.

The semantics of all syntax elements of Table 3 that are not described remain unchanged.

Table 4: scalability_type and detailed description of scalability dimension described by layer_id.

| scalability_type | Scalability dimensions described by layer_id |
| --- | --- |
| 0 | none |
| 1 | 1: temporal scalability |
| 2 .. 127 | unspecified |

Descriptions of operation points that use scalability in any dimensions (e.g. temporal level greater than 0, spatial scalability or others, i.e. layer_id >= 1) are given by means of the vps_extension_data() syntax as given in Table 5. Each operation point is identified via its layer_id. Using the vps_extension_data() syntax allows for adaptation in a straight-forward way, as operation points are explicitly referenced and can be omitted by excluding a fixed number of bytes. Whenever the omitted vps_extension_data() syntax is not referenced by other operation point description, no bits of the VPS have to be changed apart from the omitted ones, which simplifies on-the-fly adaptation considerably.

Despite the definition of the vps_extension_data() syntax in this proposal, the extension syntax can be facilitated in possible future HEVC extension by making it depended on the scalability_type syntax element of the VPS syntax. Thereby, it is possible to allow future HEVC extensions to use the VPS extension syntax structure to signal operation point specific parameters apart from profile and level identifiers or further parameters.

Table 5: Proposed VPS extension data syntax.

| vps_extension_data() { | Descriptor |
| --- | --- |
| layer_id | u(6) |
| new_profile_level_flag | u(1) |
| reserved_zero_1bit | u(1) |
| if ( new_profile_level_flag == 1 ) { |  |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| } else { |  |
| ref_layer_id | u(6) |
| reserved_zero_2bits | u(2) |
| } |  |
| } |  |

layer_id identifies the described operation point. The value of layer_id shall be greater than or equal to 1.

new_profile_level_flag allows for either explicit signaling of profile and level of the described operation or implicit signaling by referencing another operation point description by its layer_id using the syntax element ref_layer_id.

profile_idc, reserved_zero_8bits and level_idc describe the profile and level that applies to the operation point specified by layer_id.

ref_layer_id points to an operation point description (i.e. a vps_extension_data() block) via an operation point identifier that is to be used for the current operation point described by layer_id.

## Proposed Sequence parameter set syntax

As the signaling of profile and level identifier is taken care of by means the video parameter set, the corresponding syntax elements within the sequence parameter set are removed. The current video parameter set is identified within the sequence parameter set by means of its identifier video_parameter_set_id. Table 6 gives the proposed sequence parameter set syntax.

Table 6: Proposed sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| … |  |
| } |  |

video_parameter_set_id identifies the video parameter set that is referred to by the sequence parameter set.

The semantics of all syntax elements that are not described in Table 6 remain unchanged.

# References

- Jill Boyce, Wonkap Jang, Danny Hong, Stephan Wenger, Ye-Kui Wang, Ying Chen, “High level syntax hooks for future extensions”, JTC1/SC29/WG11 JCTVC-H0388, San Jose, USA, 2012

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).