| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0218v2 |
| --- | --- |

| Title: | Changes to the Hash SEI calculation for improved usability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-978-719-1348 timhellman@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution claims that the two hash SEI messages, as presently defined, are difficult for a real-time encoder or decoder to use.  It states that three independent post-decode memory passes are necessary to compute the hashes.  It recommends two changes: computing separate MD5 hashes for each component, rather than just one per picture, and adding a checksum as a third hash type.  It claims that the first change allows real-time decoders to compute the MD5 at display time, and the second allows real-time computation of the hash at both encode time and decode time.

# Introduction

HEVC introduces a standard picture hash SEI message.  This is a valuable feature that allows rapid system compliance and QA testing, both in development and in the field.  Having standardized hashes is preferable to existing ad-hoc solutions.  However, the two hashes (MD5 and CRC) as presently defined would be nearly impossible to use in real-time systems, as they are order-dependent (pixels must be fed into the hash in the defined order), and operate on each color component in sequential raster order to produce a single hash value. The combination of these two properties means that the hash can only be computed after a picture has been fully decoded, and then only by doing 3 separate read passes over the memory.  This introduces an extra bandwidth and processing burden on the real-time system, and makes it less likely that either of these hash messages will actually be used.

# Proposed Solution

Having more than one hash allows an implementer to trade off low-complexity (CRC) and improved error detection (MD5).  This contribution proposes to keep that trade-off, but to change the MD5 calculation so that real-time video decoders can use it, and to add order-independent checksum as a third hash type, so that both real-time encoders and decoders can use it.

## MD5 Calculation Change

MD5’s pixel order dependency prevents it from being calculated at the output of a decoder, unless the calculation is done in decode order.  Unfortunately, there is no universal decode order; software decoders may decode an entire picture and then go back and filter it in raster order, while hardware decoders may process an LCU, or even a CU, to completion before moving to the next.  There is, however, one place where all video decoding systems process pixels in raster order – when they read them out for display.  In fact, this is where hash calculations are commonly done today.

Of course, luma and chroma components of pixels are read at the same time for display purposes, so the existing MD5, which computes the components sequentially, does not work here.  The simplest change would be to redo the calculation with Y, U and V interleaved at the pixel level, but the sub-sampling of chroma relative to luma makes this problematic.  The solution: compute 3 separate MD5 hashes, one for each component.  This makes the calculation independent of UV subsampling, and allows it to be computed in parallel with the video display process.  It works equally well for 4:4:4, 4:2:2 and 4:2:0 sub-samplings.

The calculation is modified as shown in section 6.

Three sets of MD5 bytes are computed: picture_md5[ cIdx].  The MD5 calculation time is proportional to the number of bytes, so splitting the calculation into three does not affect the overall computational load.

## Add a Checksum hash

Having an order-independent hash would be valuable, as it could be used by both real-time encoders and decoders without any bandwidth or processing penalties, since it could be performed at the output of the decoding process as reference pixels are written to memory.  A checksum (summing up the unsigned component values) is both simple to compute and order-independent.  Of course, its order-independence means that the checksum will not detect swapping errors, where pixels are accidentally put out in the wrong order.  This shortcoming can be somewhat mitigated by exclusive-ORing the pixel’s X/Y position with its value before performing the sum, as shown (luma shown for example):

lumaChecksum = 0;
              for( y = 0; y < lumaHeight; y++ ) {
	    for( x = 0; x < lumaWidth; x++ ) {
                             xor_mask = (x & 0xFF) ^ (y & 0xFF) ^ (x >> 8) ^ (y >> 8)
		lumaChecksum =+ (component_y[ x ][ y ] & 0xFF) ^ xor_mask
		if( BitDepthy > 8 )
			lumaChecksum =+ (component_y[ x ][ y ] >> 8) ^ xor_mask
                  }
	}

A 32-bit checksum value covers picture sizes up to 4Kx2K without wrap-around.  Larger pictures can be allowed to wrap; the chance of accidental checksum aliasing is extremely small.  Separate component checksums are not strictly necessary, but are defined for consistency with the other checksums.

# Conclusions

The recommended changes to the hash algorithms make the hashes much more useful in commercial systems with no increase in complexity.  This contribution recommends adopting them in the next draft revision.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1003_dK, Feb. 2012.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

## D.1.20 Decoded picture hash SEI message syntax

| decoded_picture_hash( payloadSize ) { | Descriptor |
| --- | --- |
| hash_type | u(8) |
| for(cIdx = 0; cIdx < 3; cIdx++) { |  |
| if( hash_type = = 0 ) { |  |
| for( i = 0; i < 16; i++) |  |
| picture_md5[ cIdx ] [ i ] | b(8) |
| } |  |
| else if( hash_type = = 1 ) |  |
| picture_crc[ cIdx ] | u(16) |
| else if( hash_type = = 2 ) |  |
| picture_checksum[ cIdx ] | u(32) |
| } |  |
| } |  |

## Replace all of D.2.20 with this:

D.2.20 Decoded picture hash SEI message semantics

This message provides a hash of the decoded picture in the current access unit.

Prior to computing the hash, the decoded picture data is arranged into three strings of bytes called pictureData[ cIdx ] of lengths pictureDataLen[ cIdx ] according to the following pseudo code process..

for( cIdx = 0; cIdx < 3; cIdx++) {
		dataLen = 0
		for( i = 0; i < componentWidth * componentHeight; i++ ) {
			pictureData[ cIdx ] [ dataLen++ ] = component[ cIdx ][ i ] & 0xFF
			if( BitDepthcomponent > 8 )
				pictureData[ cIdx ][ dataLen++ ] = component[ cIdx ][ i ] >> 8
		}
		pictureDataLen[ cIdx ] = dataLen
	}

where component[ cIdx ][ i ] is an array in raster scan order of decoded sample values in two's complement representation.

hash_type indicates the method used to calculate the checksum according to Table D-1.

Table D-1 – Interpretation of hash_type

| - hash_type | - Method |
| --- | --- |
| - 0 | - MD5 (RFC 1321) |
| - 1 | - CRC |
| - 2 | - Checksum |
| - 3..255 | - Reserved |

picture_md5[ cIdx ][ i ] is the 16-byte MD5 hash corresponding to colour component cIdx of the decoded picture. It is a requirement of bitstream conformance that the values of picture_md5[ cIdx ][ i ], when present, shall be equal to the value of digestVal[ cIdx ] obtained by performing the following pseudocode process using the MD5 functions defined in RFC 1321.

for( cIdx = 0; cIdx < 3; cIdx++) {
	MD5Init( context )
	MD5Update( context, pictureData[ cIdx ], pictureDataLen[ cIdx ] )
	MD5Final( digestVal[ cIdx ], context )
}

picture_crc[ cIdx ] is the cyclic redundancy check (CRC) corresponding to colour component cIdx of the decoded picture. It is a requirement of bitstream conformance that the value of picture_crc[ cIdx ], when present, shall be equal to the value of crcVal[ cIdx ] obtained by performing the following pseudo code process.

for( cIdx = 0; cIdx < 3; cIdx++) {
	crc = 0xFFFF
	pictureData[ cIdx ][ pictureDataLen[ cIdx ] ] = 0
	pictureData[ cIdx ][ pictureDataLen[ cIdx ] + 1 ] = 0
	for( bitIdx = 0; bitIdx < ( pictureDataLen[ cIdx ]  + 2 ) * 8; bitIdx++) {
		dataByte = pictureData[ cIdx ][ bitIdx >> 3 ]
		crcMsb = ( crc >> 15 ) & 1
		bitVal = ( dataByte >> (7 – ( bitIdx & 7 ) ) ) & 1
		crc = ( ( ( crc << 1 ) + bitVal ) & 0xFFFF ) ^ ( crcMsb * 0x1021 )
	}
	crcVal[ cIdx ] = crc
}

NOTE – The same CRC specification is found in Rec. ITU-T H.271.

picture_checksum[ cIdx ] is the checksum corresponding to colour component cIdx of the decoded picture.  It is a requirement of bitstream conformance that the value of picure_checksum[ cIdx ], when present, shall be equal to the value of checksum[ cIdx ] obtained by performing the following pseudo code process.

for( cIdx = 0; cIdx < 3; cIdx++) {
	sum = 0;
	for( y = 0; y < componentHeight; y++ ) {
		for( x = 0; x < componentWidth; x++ ) {
                             	xorMask = (x & 0xFF) ^ (y & 0xFF) ^ (x >> 8) ^ (y >> 8)
			sum = (sum + (component[ cIdx ][ x ][ y ] & 0xFF) ^ xorMask) & 0xFFFFFFFF
			if( BitDepthcomponent > 8 )
					sum = (sum + (component[ cIdx ][ x ][ y ] >> 8) ^ xorMask) &
					 0xFFFFFFFF
		}
	}
	checksum[ cIdx ] = sum
}